3GPP TSG-RAN WG3 Meeting #129                                                                  			R3-255707

Bengaluru, India, August 25th - August 29th, 2025

Agenda Item:	8.3

Source:	CMCC, CATT

Title:	Discussion on UAV regulation

Document for:	Discussion

# Introduction

In last meeting, RAN3 discussed the necessary NGAP signaling enhancement to support aerial UE altitude reporting with the following agreements and leftover issues.

From RAN3#128 (UAV): The timestamp of the altitude report refers to Time Stamp IE. Introduce a new mandatory IE in User Location Information IE for Timestamp. Whether to introduce Aerial UE Reporting Reference ID? Failure indication of UAV UE flight information reporting: Option 1: Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message. Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped. Option 2: Add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message.

In this meeting, RAN3 will continue to discuss the unsolved issues on aerial UE altitude reporting for aerial UE’s monitoring based on the latest updates in TS 23.256 from SA2.

# Discussion

Aerial UE Reporting Reference ID

Regard to the aerial UE reporting reference ID, the discussion about this IE did not reach a conclusion in the last meeting. The proponents support to carry this IE within Aerial UE Flight Information Reporting Control IE to identify each location reporting procedure for aerial UE, and the aerial UE reporting reference ID is considered to be placed in a newly introduced Aerial UE Flight Information Reporting Control IE. In the latest endorsed 38.413 CR, this IE used to indicate the aerial UE flight information reporting information within the Location Reporting Request Type IE is shown as follows:

#### 9.3.1.65 Location Reporting Request Type This IE indicates the type of location request to be handled by the NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Event Type; M; ENUMERATED (direct, change of serving cell, UE presence in the area of interest, stop change of serving cell, stop UE presence in the area of interest, cancel location reporting for the UE, … change of serving cell and UE presence in the area of interest, report the Aerial UE flight information, cancel the Aerial UE flight information reporting); -. Report Area; M; ENUMERATED (cell, …); -. Area of Interest List; 0..1; -. >Area of Interest Item; 1..<maxnoofAoI>; -. >>Area of Interest; M; 9.3.1.66; -. >>Location Reporting Reference ID; M; 9.3.1.76; -. Location Reporting Reference ID to be Cancelled; C- ifEventTypeisStopUEPresinAoI; Location Reporting Reference ID 9.3.1.76; -. Additional Location Information; O; ENUMERATED (Include PSCell, ...); YES; ignore. Additional Cancelled Location Reporting Reference ID List; 0..1; YES; reject. >Additional Cancelled Location Reporting Reference ID Item; 1..<maxnoofAoIMinusOne>; -. >>Location Reporting Reference ID to be Cancelled; M; Location Reporting Reference ID 9.3.1.76; -. Aerial UE Flight Information Reporting Control; O; 9.3.1.yy; .; YES; ignore.

Within this IE, the higher altitude threshold, lower altitude threshold, and reporting periodicity are already included to instruct the NG-RAN node to collect flight information for the aerial UE. For whether the Aerial UE Reporting Reference ID is also suitable to be included, we can refer to the SA2 specification.

2. Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point. The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events. NOTE 1: Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting. Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node). 3. Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s).

It is noted that the 5GS may enable configuration of aerial UEs to perform altitude reporting periodically, threshold-based or event-based separately. For the threshold-based, multiple threshold values can be provided which means one aerial UE can be configured to different threshold values by different altitude reporting events. Consequently, the aerial UE reporting reference ID needs to be added in Aerial UE Flight Information Reporting Control IE to identify each altitude information reporting configuration corresponding to separate events.

Observation 1: The aerial UE reporting reference ID needs to be added in Aerial UE Flight Information Reporting Control IE to identify each altitude information reporting configuration corresponding to separate events.

Proposal 1: Propose to add the aerial UE reporting reference ID within the Aerial UE Flight Information Reporting Control ID.

Failure handling of aerial UE’s flight information reporting

Followed by SA2 CR, it is already agreed the enhancement of NGAP signaling to instruct an aerial UE to stop the altitude reporting. During the discussion, companies shared different views on the procedure that the NG-RAN node used to stop providing altitude reporting for aerial UEs. As the outcome, two options are discussed and provided:

Option 1: Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message.

Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped.

Option 2: Add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message.

From our view, both options are theoretically feasible to instruct the aerial UEs to stop the altitude reporting, but the option 2 requires fewer modifications to the protocol. The description of Location Reporting Failure Indication procedure and Location Report procedure in TS 38.413 are excerpted below:

### 8.12.2 Location Reporting Failure Indication #### 8.12.2.1 General The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure, the Handover Resource Allocation procedure or the Initial Context Setup procedure has failed. The procedure uses UE-associated signalling. … ### 8.12.3 Location Report #### 8.12.3.1 General The purpose of the Location Report procedure is to provide the UE's current location, the UE's last known location with time stamp, or the UE's presence in the area of interest to the AMF. The procedure uses UE-associated signalling.

In the current spec, it is noted that the purpose of Location Reporting Failure Indication procedure is used to inform the AMF that the location reporting request contained in the Location Reporting Control procedure has failed, and the purpose of Location Report procedure is to provide the aerial UE’s flight formation as well as legacy UE’s location information. Thus, if the option 1 is adopted, the existing purpose of Location Report procedure needs to be revised to indicate that the ongoing altitude reporting is stopped in addition to provide the aerial UE’s flight formation. If the option 2 is adopted, including the Aerial UE flight information reporting failed IE within LOCATION REPORTING FAILURE INDICATION message is well qualified with the purpose of the Location Reporting Failure Indication procedure. Once aerial UE fails to report the altitude information and/or the NG-RAN fails to receive the altitude information, the NG-RAN node initiates the procedure by sending a LOCATION REPORTING FAILURE INDICATION message to the AMF, and then the AMF UAS NF/NEF or the USS may decide to stop the UAV UE's altitude reporting by informing about the configured altitude information reporting.

Observation 2: Both options are theoretically feasible to instruct the aerial UEs to stop the altitude reporting, the option 2 requires fewer modifications to the protocol.

Observation 3: If the option 1 is adopted, the existing purpose of Location Report procedure needs to be revised to indicate that the ongoing altitude reporting is stopped in addition to provide the aerial UE’s flight formation.

Observation 4: If the option 2 is adopted, including the Aerial UE flight information reporting failed IE within LOCATION REPORTING FAILURE INDICATION message is well qualified with the purpose of the Location Reporting Failure Indication procedure.

Proposal 2: Propose to adopt option 2: add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message.

# Conclusions

In this contribution, we discuss RAN3 will continue to discuss the unsolved issues on aerial UE altitude reporting for aerial UE’s monitoring and provide our preference on whether to introduce aerial UE reporting reference ID and how to achieve failure handling of aerial UE’s flight information reporting. The following observation and proposal are listed below:

Observation 1: The aerial UE reporting reference ID needs to be added in Aerial UE Flight Information Reporting Control IE to identify each altitude information reporting configuration corresponding to separate events.

Observation 2: Both options are theoretically feasible to instruct the aerial UEs to stop the altitude reporting, the option 2 requires fewer modifications to the protocol.

Observation 3: If the option 1 is adopted, the existing purpose of Location Report procedure needs to be revised to indicate that the ongoing altitude reporting is stopped in addition to provide the aerial UE’s flight formation.

Observation 4: If the option 2 is adopted, including the Aerial UE flight information reporting failed IE within LOCATION REPORTING FAILURE INDICATION message is well qualified with the purpose of the Location Reporting Failure Indication procedure.

Proposal 1: Propose to add the aerial UE reporting reference ID within the Aerial UE Flight Information Reporting Control ID.

Proposal 2: Propose to adopt option 2: add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message.

# References

- R3-253020, Reply LS on clarification of UAV regulation from RAN3;

- 3GPP TS 23.256, Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking;

- 3GPP TS 38.413, NG-RAN; NG Application Protocol (NGAP);