3GPP TSG-RAN WG3 Meeting #129	R3-255862

Bengaluru , IN, 25 – 29 August, 2025

Agenda Item:	21.3

Source:	ZTE Corporation, Nokia, Nokia Shanghai Bell, Huawei, CATT, Qualcomm, Meta, Samsung

Title:		(TP to BL CR for TS 37.483) Enhancement to support timely RLC retransmissions

Document for:	Other

1	Introduction

This TP captures the RAN3 agreements on XR:

For DL Timely RLC retransmission, adopt Option 1 “CU based solution” that CP inform UP for the thresholds and CU inform DU for retransmission/poll

2	TP on Timely RLC retransmission

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.3	Bearer Context Management procedures

8.3.1	Bearer Context Setup

8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

If the SCG Activation Status IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when handling DL data transfer as specified in TS 37.340 [19].

If the PDU Set QoS Parameters IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store it and use the information as specified in TS 23.501 [20].

For each GBR QoS flow whose DRB has been successfully established and the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store this information and perform available bitrate reporting as specified in TS 23.501 [20].

For each QoS flow, if the MMSID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, consider that the QoS flow is related to a multi-modal service, as specified in TS 23.501 [20] and TS 38.300 [4].

If the Remaining Time Based RLC Retransmission Threshold IE or the Remaining Time Based RLC Polling Threshold IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store it and use this information for the concerned DRB as specified in TS 38.300 [4].

Interactions with DL Data Notification procedure:

If the MT-SDT Information Request IE is included in the BEARER CONTEXT SETUP REQUEST message and the value is set to 'true', the gNB-CU-UP shall, if supported, store it and report the MT-SDT Information IE in the DL DATA NOTIFICATION message as specified in TS 38.401 [2].

If the SDT Data Size Threshold IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store it and act as specified in TS 38.401 [2].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

For a QoS flow established with PDU Set QoS parameters, if the PDU Set based Handling Indicator IE is included in the PDU Session Data Forwarding Information IE within the BEARER CONTEXT MODIFICATION REQUEST message and the value of the PDU Set based Handling Indicator IE is set to "supported", the gNB-CU-UP shall, if supported, include the PDU Set Information Container in the data to be forwarded.

For each PDU session, if the User Plane Failure Indication IE is included in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, allocate the new NG-U DL endpoint address for the concerned GTP-U tunnel as specified in TS 23.527 [36].

For each GBR QoS flow whose DRB has been successfully established or modified and the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store this information and perform available bitrate reporting as specified in TS 23.501 [20].

For each QoS flow, if the MMSID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, consider that the QoS flow is related to a multi-modal service, as specified in TS 23.501 [20] and TS 38.300 [4].

If the Remaining Time Based RLC Retransmission Threshold IE or the Remaining Time Based RLC Polling Threshold IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store it and use this information for the concerned DRB as specified in TS 38.300 [4].

Interactions with DL Data Notification procedure:

If the MT-SDT Information Request IE is included in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to 'true', the gNB-CU-UP shall, if supported, store it and report the MT-SDT Information IE in the DL DATA NOTIFICATION message as specified in TS 38.401 [2].

If the SDT Data Size Threshold IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store it and act as specified in TS 38.401 [2].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. Corresponds to information provided in the pdcp-SN-SizeUL contained in the PDCP-Config IE as defined in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [33] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - |  |
| <<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| PDCP SN Gap Report | O |  | ENUMERATED (true, …) | Indicates whether the PDCP entity is configured to send a PDCP SN gap report in the downlink, as specified in TS 38.323 [17]. | YES | ignore |
| Remaining Time Based RLC Retransmission Threshold | O |  | INTEGER(1..64, …) | This IE indicates the threshold information for remaining time based RLC retransmission, as specified in TS 38.331 [10]. The unit is millisecond. | YES | ignore |
| Remaining Time Based RLC Polling Threshold | O |  | INTEGER(1..64, …) | This IE indicates the threshold information for remaining time based RLC polling, as specified in TS 38.331 [10]. The unit is millisecond. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-MaximumDataBurstVolume,

id-PDCPSNGapReport,

id-UserPlaneFailureIndication,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

id-MonitoringRequestonAvailableBitrate,

id-MMSID,

id-Remaining-time-based-rLC-retransmission-threshold,

id-Remaining-time-based-rLC-polling-threshold,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

-----------------Next Changes-------------------

#### -- P

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

PDCP-Configuration	::=	SEQUENCE {

pDCP-SN-Size-UL							PDCP-SN-Size,

pDCP-SN-Size-DL							PDCP-SN-Size,

rLC-Mode								RLC-Mode,

rOHC-Parameters							ROHC-Parameters			OPTIONAL,

t-ReorderingTimer						T-ReorderingTimer		OPTIONAL,

discardTimer							DiscardTimer			OPTIONAL,

uLDataSplitThreshold					ULDataSplitThreshold	OPTIONAL,

pDCP-Duplication						PDCP-Duplication		OPTIONAL,

pDCP-Reestablishment					PDCP-Reestablishment	OPTIONAL,

pDCP-DataRecovery						PDCP-DataRecovery		OPTIONAL,

duplication-Activation				Duplication-Activation		OPTIONAL,

outOfOrderDelivery						OutOfOrderDelivery		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Configuration-ExtIEs } }	OPTIONAL,

...

}

PDCP-Configuration-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-PDCP-StatusReportIndication					CRITICALITY ignore	EXTENSION PDCP-StatusReportIndication					PRESENCE	optional}|

{ ID id-AdditionalPDCPduplicationInformation		CRITICALITY ignore	EXTENSION AdditionalPDCPduplicationInformation	PRESENCE optional	}|

{ ID id-EHC-Parameters								CRITICALITY ignore	EXTENSION EHC-Parameters									PRESENCE optional}|

{ ID id-UDC-Parameters								CRITICALITY ignore	EXTENSION UDC-Parameters									PRESENCE optional}|

{ ID id-DiscardTimerExtended								CRITICALITY reject	EXTENSION DiscardTimerExtended									PRESENCE optional}|

{ ID id-PSIbasedDiscardTimer						CRITICALITY ignore	EXTENSION PSIbasedDiscardTimer								PRESENCE optional}|

{ ID id-PDCPSNGapReport						CRITICALITY ignore	EXTENSION PDCPSNGapReport														PRESENCE optional}|

{ ID id-Remaining-time-based-rLC-retransmission-threshold	CRITICALITY ignore	EXTENSION Remaining-time-based-rLC-threshold		PRESENCE optional}|

{ ID id-Remaining-time-based-rLC-polling-threshold	CRITICALITY ignore	EXTENSION Remaining-time-based-rLC-threshold		PRESENCE optional},

...

}

-----------------Next Changes-------------------

#### -- R

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

Remaining-time-based-rLC-threshold	::= INTEGER(1..64, ...)

-----------------Next Changes-------------------

9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-N6JitterInformation											ProtocolIE-ID ::= 202

id-ECNMarkingorCongestionInformationReportingRequest			ProtocolIE-ID ::= 203

id-ECNMarkingorCongestionInformationReportingStatus				ProtocolIE-ID ::= 204

id-PDUSetbasedHandlingIndicator 								ProtocolIE-ID ::= 205

id-IndirectPathIndication										ProtocolIE-ID ::= 206

id-F1UTunnelNotEstablished										ProtocolIE-ID ::= 207

id-F1U-TNL-InfoToAdd-List										ProtocolIE-ID ::= 208

id-F1U-TNL-InfoAdded-List										ProtocolIE-ID ::= 209

id-F1U-TNL-InfoToAddOrModify-List								ProtocolIE-ID ::= 210

id-F1U-TNL-InfoAddedOrModified-List								ProtocolIE-ID ::= 211

id-F1U-TNL-InfoToRelease-List									ProtocolIE-ID ::= 212

id-BroadcastF1U-ContextReferenceE1								ProtocolIE-ID ::= 213

id-PSIbasedDiscardTimer											ProtocolIE-ID ::= 214

id-UserPlaneErrorIndicator										ProtocolIE-ID ::= 215

id-MaximumDataBurstVolume 										ProtocolIE-ID ::= 216

id-BCBearerContextNGU-TNLInfoatNGRAN-Request					ProtocolIE-ID ::= 217

id-PDCPSNGapReport												ProtocolIE-ID ::= 218

id-UserPlaneFailureIndication									ProtocolIE-ID ::= 219

id-PduSetDelayBudgetDownlink									ProtocolIE-ID ::= a1

id-PduSetDelayBudgetUplink										ProtocolIE-ID ::= a2

id-PduSetErrorRateDownlink										ProtocolIE-ID ::= a3

id-PduSetErrorRateUplink										ProtocolIE-ID ::= a4

id-MonitoringRequestonAvailableBitrate							ProtocolIE-ID ::= b1

id-MMSID														ProtocolIE-ID ::= c1

id-Remaining-time-based-rLC-retransmission-threshold			ProtocolIE-ID ::= xx

id-Remaining-time-based-rLC-polling-threshold					ProtocolIE-ID ::= xx

END

-- ASN1STOP

-----------------End of the Changes-------------------