3GPP TSG-RAN WG3 Meeting #129	R3-255868

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	21.3

Source: 	Huawei, CMCC, China Telecom, ZTE, Samsung, Ofinno, Nokia, Nokia Shanghai Bell, CATT, Qualcomm, Meta

Title: 	(TP to BL CR for TS 37.340) Clean-up for QNC

Document for:	Discussion and Decision

# 1. Introduction

This paper provides TPs for TS 37.340 to address the Editor’s note regarding QNC.

# Annex. TP for BL CR for TS 37.340

CHANGE STRATS

### 10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GBR QoS for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node, as specified in TS23.501 [11].

Figure 10.13.2-1: Notification Control Indication procedure

Figure 10.13.2-1 shows an example signalling flow for the Notification Control Indication procedure.

1.	The MN may, for an SN terminated bearer, indicate, that the GBR QoS requested from the MN cannot be fulfilled anymore.
In case the SN terminated bearer is configured as a split bearer, the SN may decide to increase the share provided by the SN or it may decide to notify the MN that resources requested for the SN terminated bearer cannot fulfill the GBR QoS any more.

2.	Continuing the example message flow from step 1, the SN informs the MN that the GBR QoS for an SN terminated bearer cannot be fulfilled any more.

3.	The MN decides to inform the 5GC that NG-RAN cannot fulfill the GBR QoSfor a GBR QoS flow any more.

CHANGE ENDS