3GPP TSG-RAN WG3 Meeting #129	R3-25xxxx

Bengaluru, India, 25 – 29 August 2025

Agenda item:	20.2

Source:	Ericsson (moderator)

Title:	CB: # 17_AIPHY

Document for:	Discussion and Decision

# 1	Introduction

This contribution provides summary of offline discussion on AI/ML for PHY (Positioning) based on the following CB minutes:

CB: # 17_AIPHY

- Further check 5572 and 5337

- TPs to reflect the above agreements

- BL CR cleanup and completion

- Reply LS to SA2 based on 5583

(Ericsson)

# 2	For the Chair’s Notes

- Agree R3-255823 - (TP to NRPPa BL CR): Case 3b

- Agree R3-255813 - (TP to F1AP BL CR): Case 3b

- Agree R3-255873 - (TP to NRPPa BL CR): Case 3a

- Agree R3-255775 – TP to stage 2 TS 38.305

- Agree R3-255824 LS to RAN2

- Agree R3-255825 LS to SA2

# 3		Further check 5572 and 5337 (Case 3b TPs)

Based on offline discussion, it has been agreed to rename the “UL-SRS-TDCP” in R3-255572/5337 TPs into “UL-SRS-TDCP item”, to reflect this is an element of the Nt list constituting the power information.

Proposal 1: Agree revisions R3-255813 and R3-255823 revisions of  5572 and 5337

# 4	Case 3a NRPPa TP

It has been agreed during the online session that:

The "LoS/NLoS indicator" can also be a gNB inferred measurement.

- Change #1: It is proposed to remove the FFS from the TRP Measurement Result IE:

It was also agreed online that In a POSITIONING DATA COLLECTION REPORT message, a LMF can provide the LoS/NLoS label information independently of any timing information label. The current semantics description in IE Positioning Data Information (9.2.Z2 [1]) is removed.

- Change #2: delete the semantics of the LoS/NLoS Information IE in 9.2.Z2:

Another online agreement was that: It should be possible for an LMF to specifically request for AI/ML inferred measurements from TRPs.

- Change #3: add a new bit “inferred timing measurement” in the MCRI

Finally, on the indication of "Positioning Data Unavailable", it was agreed:

Add the following error causes to the Positioning Data Unavailable IE:

- Positioning Data Information Not Supported

- Positioning Data Information Supported But Currently Not Available

- Change #4 revise the codepoints of the Positioning Data unavailable IE (ASN.1 change needed)

Proposal 2: Agree revisions R3-25xxxx NRPPa TP capturing changes #1, #2, #3, #4:

- Change #1: remove the FFS from the TRP Measurement Result IE:

- Change #2: delete the semantics in 9.2.Z2:

- Change #3: add a new bit “inferred timing measurement” in the MCRI

- Change #4 revise the codepoints of the Positioning Dat Unavailable IE (ASN.1 change needed)

| Company | Comment |
| --- | --- |
| Ericsson | Change 1: As said online, can be done in stage 2 with no stage 3 impacts Change #2, #3: OK Change #4: the codepoints names are too long, suggest to be shortened to “not supported”, “not available” |
| CEWiT | Change 1: It is better to introduce one more conditional codepoint as suggested in TDoc R3-255288 as below. This is to reflect that both the measurement and LOS/NLOS information can be inferred and reported but LOS/NLOS can be reported only when measurement reported. >Inferred measurement; O; ENUMERATED(true, …); This IE is only valid for UL-RTOA, gNB Rx-Tx Time Difference.; YES; ignore. >Inferred LoS/NLoS Information; C-ifInferred measurementtrue; ENUMERATED(true, …); LoS/NLoS Information is included only if UL RTOA or gNB Rx-Tx Time Difference is provided; YES; ignore. Change 2: Okay Change 3: Agree with E// cause names are too long. Further are these the only cause possible. Should we add cause “Unknown”? |
| Xiaomi | Replace IE with bitmap |
| QC | Should we add a third codepoint for generic unknown reason? To be added later if needed |

Proposal 1: re-encode the Inferred Measurement IE as a bitmap size (8), same structure as Data Collection needed IE

Proposal 2: add a new bit “inferred timing measurement” in the MCRI

Proposal 3: delete the semantics of the LoS/NLoS Information IE in 9.2.Z2

Proposal 4: shorten the codepoints to “not supported”, “not available”

Huawei to capture P1-P4 in a NRPPa TP for case 3a.

# 5 Stage 2 clean-up

A TP will be provided by CATT capturing the stage 2 edits from other companies in R3-25xxxx. It is proposed to review it offline and provide comments directly in the document.

Proposal 3: Agree revisions R3-255775 merging other TS 38.305 TPs

To CATT:

- QC: rebase the TP on latest TS 38.305, keep change marks from BL CR

- Nokia, Huawei: make the text more generic and avoid mentioning of “training model”. Revert the box in the figure in step 2 in section 7.x

- CEWIT/QC: describe new LMF functions of providing labels to gNB in section 5.4.4

- CATT/Nokia: remove the two last sentences in step 5 from 7.x

# 6 LS reply to SA2 based on 5583

As part of online discussion it is proposed to reply to SA2 in (R3-255023 - S2-2505943) indicating that RAN3 will pursue solution 1 in Rel-19.

Based on R3-255583, below a revised text:

1. Overall Description: RAN3 thanks SA2 for their LS response with feedback to RAN3 solutions on case 3a (NW-sided model, AI/ML-assisted positioning). RAN3 will proceed with solution 1 in Rel-19. RAN3 kindly asks SA2 to add support of related LMF procedures in TS 23.273 as needed. 2. Actions: To RAN2 : RAN3 kindly asks SA2 to take the above into account and introduce support for solution 1 in TS 23.273 as needed.

Proposal 4: Agree revisions R3-25xxxx LS to SA2

| Company | Comment |
| --- | --- |
| CEWiT | Okay with the CR. But as commented online, the CR is not necessary as SA2 has gone ahead and discussing the possible changes in TS 23.273. |

# 7	Other proposals

The following issues have not been discussed online, or slightly touched upon, they are to be discussed on a best effort basis during the offline session:

- Reply LS to RAN2 on logged Measurement Data during HO, capture and send in a LS to RAN2 that RAN3 will not do any work on this in Rel-19.

- Whether the Positioning Data Collection Needed IE should remain a top level IE or included per TRP in a TRP list in the Measurement Response/Report messages

- Addition of time stamp in the Positioning Data Collection Needed IE to indicate the desired time for report of data collection

- Introduce a NOTE in the TP for BLCR to TS 38.305 to indicate that a gNB may use different Nt', Nt and/or k values, other than the signalled parameters for measurement

- New cause values in case gNB cannot provide UL-SRS-TDCT

| Company | Comment |
| --- | --- |
| Ericsson | Issue #1: OK Issue #2: we think it should be per TRP, but fine to keep it as it is. A stage 2 clarification will anyway be needed. Either to be clarified this meeting or during maintenance. Issue #3: OK, gNB hosting the model knows when in time it needs training data and include a reference time. We note this has not been precluded from RAN1 agreements. We are fine to consider this now or during maintenance. Issue #4: OK, either now or during maintenance. Issue #5: NOK, existing cause values are sufficient |
| QC/CATT/Xiaomi | Issue#1 Include the Data Collection needed IE in a list of TRPs |
| Huawei/ZTE | Issue#1 not needed to restrict for a subset of TRPs |
| QC: | Issue#4: not needed in stage 2, maybe in stage 3 |
| Nokia | Issue#4 can be considered in maintenance |
| Huawei | Issue#4 important to be captured somewhere |

# References

| R3-255009 | LS on Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121 (RAN1(Ericsson)) | LS in |
| --- | --- | --- |
| R3-255016 | Logged Data Handling During Handover (RAN2(Nokia)) | LS in |
| R3-255023 | Reply LS on AI/ML Positioning Case 3a (SA2(Nokia)) | LS in |
| R3-255029 | Reply LS on OAM-centric solution for NW-side data collection (SA5(Huawei)) | LS in |
| R3-255160 | Views on the Open Issues for AI/ML Positioning Accuracy Enhancements (Qualcomm Incorporated) | Discussion |
| R3-255161 | TP to BL CR for TS 38.305 (Qualcomm Incorporated) | Other |
| R3-255184 | (TP to BL CR for TS38.305) Support of AI Positioning (CATT) | Other |
| R3-255185 | (TP to BL CR for TS38.455) Support of AI Positioning for Case 3a (CATT) | Other |
| R3-255200 | [TP for BL CR to TS 38.305 & TP for BL CR to TS 38.455] Discussion on AI/ML for positioning Case 3a (NEC) | Other |
| R3-255201 | [TP for BL CR to TS 38.455] Discussion on AI/ML for positioning Case 3b (NEC) | Other |
| R3-255288 | Remaining Issues for AI/ML based positioning case 3a and TP to NRPPa (CEWiT, IITM, Tejas Networks) | Discussion |
| R3-255336 | (TPs for TS 38.305 and TS 38.455) Support of gNB-side model (case 3a) (Xiaomi) | Other |
| R3-255337 | (TP to BL CR TS 38.473) addition of UL SRS time domain channel measurement (Xiaomi, Ericsson, CATT, Nokia, Samsung) | other |
| R3-255338 | (Draft LS out) Logged data handling during handover (Xiaomi) | LS out To: RAN2 CC: |
| R3-255372 | (TPs for AI/ML BLCR to TS 38.300/305/401/455/473) Remaining open issues for AI/ML-based positioning Case 3a and Case 3b (Huawei) | Other |
| R3-255373 | (Draft Reply LS to RAN2) Discussion on R3-255016 (R2-2504950) LS Logged Data Handling During Handover (Huawei) | other |
| R3-255377 | Discussion on support of AI/ML assisted positioning (Case 3a) (China Telecom) | Discussion |
| R3-255378 | (TP to TS38.305) On support of AIML assisted positioning (China Telecom) | other |
| R3-255406 | Discussion on NW side data collection (Lenovo) | discussion |
| R3-255407 | (TP to BLCR 38.455) Provision of associated ID in Case 1 (Lenovo) | Other |
| R3-255456 | (TP to TS 38.300) AI/ML Training in OAM for case 3a (Nokia, Ericsson, Xiaomi, Samsung, CATT, ZTE) | other |
| R3-255457 | (TP to TS 38.305) Addressing remaining aspects in case 3a and case 3b (Nokia) | other |
| R3-255458 | Logged Data Handling after a Handover Event (Nokia) | discussion |
| R3-255459 | [Draft] Reply LS on Logged Data Handling During Handover (Nokia) | LS out To: RAN2 CC: SA5 |
| R3-255503 | (TP to BL CR to 38.401) AI/ML assisted Positioning (ZTE Corporation, Ericsson, Xiaomi, Nokia, Samsung, CATT) | other |
| R3-255504 | (TP to BL CR to 38.455) Support of AI/ML assisted Positioning (case 3a) (ZTE Corporation) | Other |
| R3-255505 | [Draft LS] Reply LS on logged Data Handling During Handover (ZTE Corporation) | other |
| R3-255570 | (TP to NRPPa/TS 38.305 BL CR): Addressing remaining issues of NW-assisted AI/ML positioning with gNB-sided model (Ericsson, Nokia) | other |
| R3-255571 | Way forward for Proactive (NGAP-based solution) for AI/ML NG-RAN assisted positioning (Ericsson) | discussion |
| R3-255572 | (TP to NRPPa BL CR): addition of UL SRS time domain channel measurement (Ericsson, Xiaomi, CATT, CEWIT, Nokia, Samsung) | other |
| R3-255583 | [Draft] Reply LS on AI/ML Positioning Case 3a (Nokia) | LS out To: SA2 CC: |
| R3-255669 | Discussion on AI for positioning case 3a and case 3b (Samsung) | Discussion |
| R3-255670 | Discussion on Logged Data Handling During Handover (Samsung) | other |