3GPP TSG-RAN WG3 Meeting #129	R3-255756

Bengaluru, Karnataka, India, August 25th – 29th 2025

Agenda Item:	8.1

Source:	Ericsson

Title:	MBS Communication Service Type for QMC

Document for:	Agreement

# Introduction

At the RAN3#127 meeting, RAN3 sent a reply LS to SA4 in R3-250858, asking for clarification concerning the @communicationServiceType attribute introduced by SA4 in TS 26.247. At this meeting, RAN3 received in R3-255028 the reply from SA4, including updates in the definition of the @communicationServiceType attribute (ref. CR 0192 attached to the reply LS in S4-251095).

In this contribution, we discuss the impact of the reply LS from SA4 on RAN3 specifications. The corresponding CRs to NGAP and XnAP are provided in R3-255167 and R3-255168, respectively.

# Discussion

According to the CR 0192 attached to the LS received from SA4 in R3-255028, the @communicationServiceType in TS 26.247 is now defined as follows:

| @communicationServiceType | O | When present, this attribute indicates a list of communication service type(s) for which the collection and reporting of QoE metrics is requested, and shall contain one or more of the following values: - The value unicast refers to the common unicast communication type carrying media streaming services. - The value mbsMulticast refers to the MBS Multicast communication service per clause 21.1 of TS 38.300 [71]. - The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300 [71]. - The value all refers to all communication service types. When absent, quality metrics collection is requested for all communication service types. |
| --- | --- | --- |

The questions asked by RAN3 to SA4 in R3-250858 and the SA4 answers are reported below:

- Question 1: What does this “all” value mean? Which of the following different interpretations is in line with SA4’s intention :

  - Option 1: Value “all” means that the UE shall conduct QoE measurements on broadcast multicast, and unicast.

  - Option 2: Value “all” means that the UE shall conduct QoE measurements on broadcast and multicast.

  - Option 3: Value “all” means that the UE shall conduct QoE measurements on broadcast and multicast, but not on unicast

- Answer 1: Option 1 is the right interpretation.  It refers to all communication service types (not limited to MBS communication services).

- Question 2: What is the reason for setting the “Use” value of @communicationServiceType to “OD”?. Some companies in RAN3 think the “OD” property of @communicationServiceType causes problems and should instead be changed to “O”. The reason is that, if the @communicationServiceType is absent from the measurement configuration, the UE will not conduct QoE measurements for sessions delivered via MBS, for any MBS mode. This is because @communicationServiceType is the only indication in the configuration that the measurement is requested for MBS. Is it acceptable to rename the “Use” from “OD” to “O”?

- Answer 2: “OD” is revised into “O” in TS 26.247.  Furthermore, when @communicationServiceType is absent, meaning by using the legacy solution, QoE collection is requested for all communication services, regardless of where the content comes from. By using the @communicationServiceType attribute, depending on the communication type, QoE reports can be collected in certain cases. Additionally, SA4 added unicast refers to the common unicast communication type.

- Question 3: Is it acceptable for SA4 to rename @communicationServiceType to @MBScommunicationServiceType? The intention is to align the naming with the corresponding TS 38.413/423 IE (MBS Communication Service Type) and the TS 28.622 IE mBSCommunicationServiceType, and to avoid potential misunderstandings across WGs.

- Answer 3: From SA4 perspective, @communicationServiceType is a superset of @mbsCommunicationServiceType. As explained in answer 1 and 2, It is recommended to use @communicationServiceType.

We notice that, in the current NGAP and XnAP specifications, the MBS Communication Service Type IE is contained in the UE Application Layer Measurement Configuration Information IE and defined as follows:

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast,...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Based on the answers provided by SA4, we identify the following misalignments between the @communicationServiceType present in the XML QMC configuration file, and the corresponding NGAP and XnAP MBS Communication Service Type IE:

Misalignment 1:

The attribute @communicationServiceType can be set to “all”, while the MBS Communication Service Type IE does not have a corresponding codepoint (its value can only be “broadcast” or “multicast”).

- For example, if the value of the XnAP MBS Communication Service Type IE = “multicast”, a gNB that does not support MBS and receives this IE during handover preparation, may think that the UE will not continue QoE measurements initiated for multicast, and can release the QoE configuration.

Misalignment 2:

In the second case, as described in the updated description of the @communicationServiceType attribute, its meaning, from UE’s point of view, is that “quality metrics collection is requested for all communication service types”, i.e., including “unicast”. However, a target gNB that does not support multicast will not understand that, and a situation similar to the first case can occur, i.e., the gNB releases the configuration.

Based on the above, we think that a correction is needed for the MBS Communication Service Type IE, to allow a gNB to have the same view as the UE in terms of communication service types for which QoE measurement collection should be performed.

We propose to align the codepoints of MBS Communication Service Type IE with the codepoints of @communicationServiceType in TS 26.247, as per below:

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast,..., unicast, all) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 1: Align the codepoints of MBS Communication Service Type IE with the codepoints of @communicationServiceType in TS 26.247.

Proposal 2: Agree the NGAP and XnAP CRs in R3-255167 and R3-255168, respectively.

In addition, we think that two of the newly-agreed changes to TS 26.247 require additional attention.

The agreed CR states the following:

| @communicationServiceType | O | When present, this attribute indicates a list of communication service type(s) for which the collection and reporting of QoE metrics is requested, and shall contain one or more of the following values: - The value unicast refers to the common unicast communication type carrying media streaming services. - The value mbsMulticast refers to the MBS Multicast communication service per clause 21.1 of TS 38.300 [71]. - The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300 [71]. - The value all refers to all communication service types. When absent, quality metrics collection is requested for all communication service types. |
| --- | --- | --- |

In our understanding, the part highlighted in yellow needs correction, because the design of the attribute  is such that the attribute can have only one value at a time, which contradicts the “or more”.

In addition, the part highlighted in cyan states that, in case the attribute is absent, there exists a default interpretation. In our understanding, such statements are pertinent to “OD” type of use, whereas SA4 changed the Use of the attribute from “OD” to “O”.

We propose that SA4 is liaised and asked to resolve the two issues.

Proposal 3: Send an LS asking SA4 to clarify:

- Whether the text in TS 26.247 implies that the @communicationServiceType can have more than one value at the same time.

- Whether the @communicationServiceType can have a default value even though the Use is “O”.

# 3	Conclusions

Based on the discussion in this paper, we propose the following:

Proposal 1: Align the codepoints of MBS Communication Service Type with the codepoints of @communicationServiceType in TS 26.247.

Proposal 2: Agree the NGAP and XnAP CRs in R3-255167 and R3-255168, respectively.

Proposal 3: Send an LS asking SA4 to clarify:

- Whether the text in TS 26.247 implies that the @communicationServiceType can have more than one value at the same time.

- Whether the @communicationServiceType can have a default value even though the Use is “O”.