3GPP TSG-RAN3 Meeting #129	R3-255813

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	20.2

Source:	Xiaomi, Ericsson, CATT, Nokia, Samsung, ZTE, CEWIT, Huawei, Qualcomm Incorporated, NEC

Title:	(TP to BL CR TS 38.473) Addition of UL SRS time domain channel measurement

Document for:	other

1	Introduction

In TS 38.215 V19.0.0 [1], we can see that RAN1 has concluded on defining new measurements:

•	UL SRS time domain channel timing (UL SRS-TDCT) – see section 5.2.9

•	UL SRS time domain channel power (UL SRS-TDCP) – see section 5.2.10

This contribution provides a TP to align with RAN1 specification.

2	Reference

[1]3GPP TS 38.215, Physical layer measurements, Release 19

3	TP to BL CR 38.473 (support of case 3b)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CG	Cell Group

CG-SDT	Configured Grant-Small Data Transmission

CGI	Cell Global Identifier

CHO	Conditional Handover

CN	Core Network

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

eRedCap	Enhanced Reduced Capability

FSA ID	MBS Frequency Selection Area (FSA) ID

GPSI	Generic Public Subscription Identifier

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Service

MP	Multi-path

MT-SDT	Mobile Terminated Small Data Transmission

N3C	Non-3GPP Connection

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PDC	Propagation Delay Compensation

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated NPN

posSIB	Positioning SIB

PSI	PDU Set Importance

PTM	Point to Multipoint

PTP	Point to Point

QMC	QoE Measurement Collection

QoE	Quality of Experience

RANAC	RAN Area Code

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSPP	Ranging/Sidelink Positioning Protocol

RSRP	Reference Signal Received Power

S-CPAC	Subsequent Conditional PSCell Addition or Change

S-NSSAI	Single Network Slice Selection Assistance Information

SDT	Small Data Transmission

SNPN	Stand-alone Non-Public Network

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAG	Timing Advance Group

TAI	Tracking Area Identity

TEG	Timing Error Group

TRP	Transmission-Reception Point

TSS	Timing Synchronisation Status

U2N	UE-to-Network

U2U	UE-to-UE

UL-AoA	Uplink Angle of Arrival

UL-RSCP	UL Reference Signal Carrier Phase

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL SRS-TDCT	UL SRS Time Domain Channel Timing

UL-SRS-TDCP	UL SRS Time Domain Channel Power

V2X	Vehicle-to-Everything

Z-AoA	Zenith Angles of Arrival

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>Search Window Information | O |  | 9.3.1.204 |  | - |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.3.1.238 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, and 30min are not applicable. | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| >Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP, UL-RSCP, UL SRS-TDCT) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5, and so on, see TS 38.133 [38]. | YES | ignore |
| >>Channel Response Information | O |  | 9.3.1.x1 | Applicable to ULSRS-TDCT only. | Yes | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) | This IE is ignored when the Measurement characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.3.1.242 | This IE is ignored when the Positioning Report Characteristics IE is set to “Periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.3.1.254 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4,…) |  | YES | ignore |
| Positioning Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Positioning Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.3.1.334 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.166	Positioning Measurement Result

The purpose of this information element is to provide the measurement result(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning Measured Result Item |  | 1 .. <maxnoofPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  |  |  |  |  |
| >>>UL Angle of Arrival | M |  | 9.3.1.167 |  | - |  |
| >>UL SRS-RSRP |  |  |  |  |  |  |
| >>>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA |  |  |  |  |  |  |
| >>>UL RTOA | M |  | UL RTOA Measurement 9.3.1.168 |  | - |  |
| >>gNB Rx-Tx Time Difference |  |  |  |  |  |  |
| >>>gNB Rx-Tx Time Difference | M |  | 9.3.1.170 |  | - |  |
| >>Zenith Angle of Arrival Information |  |  |  |  | YES | reject |
| >>>Zenith Angle of Arrival Information | M |  | 9.3.1.239 |  | - |  |
| >>Multiple UL AoA |  |  |  |  | YES | reject |
| >>>Multiple UL AoA | M |  | 9.3.1.245 |  | - |  |
| >>UL SRS-RSRPP |  |  |  |  | YES | reject |
| >>>UL SRS-RSRPP | M |  | 9.3.1.246 |  | - |  |
| >>UL RSCP |  |  |  |  | YES | reject |
| >>>UL RSCP | M |  | 9.3.1.335 |  | - |  |
| >>UL SRS-TDCT |  |  |  |  | YES | reject |
| >>>UL SRS-TDCT | M |  | 9.3.1.x2 |  | - |  |
| >Time Stamp | M |  | 9.3.1.171 |  | - |  |
| >Measurement Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Measurement Beam Information | O |  | 9.3.1.173 |  | - |  |
| >ARP ID | O |  | 9.3.1.244 |  | YES | ignore |
| >SRS Resource type | O |  | 9.3.1.247 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.3.1.249 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.3.1.304 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..<maxnoaggregatedSRS-Resources> |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  | - |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER(0..2199,…) |  | - |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>>Carrier Bandwidth | M |  | INTEGER (1..275,…) |  | - |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.254	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by the gNB-CU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: Additional Path Power Fourth Bit: Multiple UL AoA of Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG-ID information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteenth bit: UL SRS-TDCP in UL SRS-TDCT. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.x1	Channel Response Information

This IE contains information about the requested UL SRS-TDCT measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response Window Size | M |  | ENUMERATED (32, 64, 128, …) | Represents the window size over which channel response values are selected. |
| Channel Response Number | M |  | ENUMERATED (8, 16, 24, …) | Represents the number of channel responses selected over the channel response window size. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.x2	UL SRS-TDCT

This information element contains the UL SRS time domain channel timing.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL SRS-TDCT List |  |  |  |  |
| >UL SRS-TDCT Item |  | 1..<maxnoofChannelRes> |  |  |
| >>CHOICE Timing Information | M |  |  |  |
| >>>k0 |  |  |  |  |
| >>>>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] |
| >>>k1 |  |  |  |  |
| >>>>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] |
| >>>k2 |  |  |  |  |
| >>>>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] |
| >>>k3 |  |  |  |  |
| >>>>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] |
| >>>k4 |  |  |  |  |
| >>>>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] |
| >>>k5 |  |  |  |  |
| >>>>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] |
| >> Power Information | O |  | UL SRS-TDCP Item 9.3.1.x3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofChannelRes | Maximum no of channel response. Value is 24 |

9.3.1.x3	UL SRS-TDCP Item

This IE contains an item of the UL SRS time domain channel power for a given channel response.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL SRS-TDCP Item | M |  | INTEGER (0..126) | TS 38.133 [16] |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

//Omitted text unchanged//

id-ChannelResponseInformation,

id-UL-SRS-TDCT,

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

ChannelResponseInformation ::= SEQUENCE {

channelResponseWindowSize		ENUMERATED {ws32, ws64, ws128, ...},

channelResponseNumber			ENUMERATED {n8, n16, n24, ...},

iE-Extension			ProtocolExtensionContainer { { ChannelResponseInformation-ExtIEs} }	OPTIONAL,

...

}

ChannelResponseInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

MeasuredResultsValue-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ZoAInformation	CRITICALITY reject TYPE ZoAInformation	PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA	PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP	PRESENCE mandatory}|

{ ID id-UL-RSCP			CRITICALITY reject TYPE UL-RSCP			PRESENCE mandatory}|

{ ID id-UL-SRS-TDCT		CRITICALITY reject TYPE UL-SRS-TDCT		PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

PosMeasurementType ::= ENUMERATED {

gnb-rx-tx,

ul-srs-rsrp,

ul-aoa,

ul-rtoa,

... ,

multiple-ul-aoa,

ul-srs-rsrpp,

ul-rscp,

UL-SRS-TDCT

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

UL-SRS-TDCT ::= SEQUENCE {

UL-SRS-TDCT-List			UL-SRS-TDCT-List,

iE-Extensions				ProtocolExtensionContainer { { UL-SRS-TDCT-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCT-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCT-List::= SEQUENCE (SIZE(1..maxnoofChannelRes)) OF UL-SRS-TDCT-Item

UL-SRS-TDCT-Item ::=SEQUENCE {

timingInformation	TimingInformation,

powerInformation	UL-SRS-TDCP-Item 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UL-SRS-TDCT-Item-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-TDCT-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-SRS-TDCP-Item ::= INTEGER (0..126)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

//Omitted text unchanged//

id-ChannelResponseInformation						ProtocolIE-ID ::= xx1

id-UL-SRS-TDCT										ProtocolIE-ID ::= xx2

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>