3GPP TSG-RAN WG3 Meeting #129	R3-255965

# Bengauru, India, 25th - 29th August 2025	was R3-255073

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0469 | rev | 4 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Nokia, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-08-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces NG-RAN stage-2 specification for support of A-IoT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces stage-2 specification for - A-IoT specific Application Protocol Identities - A-IoT specific contexts |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-IoT cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 6.1.1, 6.2.1, 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.410 CR0053 TS 38.412 CR0025 TS 38.413 CR1260 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r4 (post#129, R3-255965) implementing TP in R3-255798 and converting the BL CR into a regular CR for submission to TSG RAN r3 (pre #129, R3-255073) re-submission as endorsed post#128 r2 (post #128, R3-253969): implementing TP in R3-253939 r1 (pre #128, R3-253097): re-submission as endorsed post#127bis r0 (post #127-bis, R3-252494): converting agreed TP in R3-252478 into a proper CR |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[5]	3GPP TS 38.414: "NG-RAN; NG data transport".

[6]	3GPP TS 38.424: "NG-RAN; Xn data transport".

[7]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[8]	ITU-T Recommendation G.823 (2000-03): "The control of jitter and wander within digital networks which are based on the 2048 kbit/s hierarchy".

[9]	ITU-T Recommendation G.824 (2000-03): "The control of jitter and wander within digital networks which are based on the 1544 kbit/s hierarchy".

[10]	ITU-T Recommendation G.825 (2001-08): "The control of jitter and wander within digital networks which are based on the synchronous digital hierarchy (SDH)".

[11]	ITU-T Recommendation G.8261/Y.1361 (2008-04): "Timing and Synchronization aspects in Packet networks".

[12]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[13]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[14]	3GPP TS 38.410: "NG-RAN; NG general aspect and principles".

[15]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles"

[16]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[17]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[18]	3GPP TS 37.480: "E1 general aspects and principles".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 32.422: "Trace control and configuration management".

[21]	3GPP TS 37.470: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and NG-RAN; W1 general aspects and principles; Stage-2".

[22]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[23]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[24]	3GPP TS 38.425: "NG-RAN; NR user plane Protocol".

[25]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[26]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[27]	3GPP TS 23.247: " Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[28]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture Description".

[29]	IETF RFC 4555 (2006-06): "RFC IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[30]	3GPP TS 38.321 " NR; Medium Access Control (MAC) protocol specification ".

[31]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[32]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[33]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[34]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”

[35]	3GPP TS 37.460: "Iuant interface: General aspects and principles".

[36]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[z]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

AI	Artificial Intelligence

BAP	Backhaul Adaptation Protocol

BH	Backhaul

BSR	Buffer Status Report

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DU	Distributed Unit

EM	Element Manager

EN-DC	E-UTRA-NR Dual Connectivity

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

FTEID	Fully Qualified TEID

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2		Layer-2

LBT	Listen Before Talk

LTM		L1/L2 Triggered Mobility

MBS	Multicast Broadcast Service

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MOCN	Multi-Operator Core Network

MN	Master Node

MgNB	Master gNB

mIAB	Mobile IAB

mIAB-DU	Mobile IAB Distributed Unit

mIAB-MT	Mobile IAB Mobile Termination

MP	Multi-Path

MRB	MBS Radio Bearer

MRDC	Multi-Radio Dual Connectivity

ML	Machine Learning

MT-SDT	Mobile Terminated Small Data Transmission

N3C	Non-3GPP Connection

NAS	Non-Access Stratum

NCI	NR Cell Identity

NDS	Network Domain Security

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RANAC	RAN Area Code

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS	Remote Interference Management Reference Signal

RLF	Radio Link Failure

RNL	Radio Network Layer

RRC	Radio Resource Control

SA	Standalone

SAP	Service Access Point

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SDT	Small Data Transmission

SFN	System Frame Number

SgNB	Secondary gNB

SM	Session Management

SMF	Session Management Function

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SRAP	Sidelink Relay Adaptation Protocol

TAC	Tracking Area Code

TCE	Trace Collection Entity

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

U2N	UE-to-Network

UL	Uplink

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.1.1	Overall Architecture of NG-RAN

Figure 6.1-1: Overall architecture

The NG-RAN consists of a set of gNBs connected to the 5GC through the NG interface.

NOTE 1:	As specified in TS 38.300 [2], NG-RAN could also consists of a set of ng-eNBs, an ng-eNB may consist of an ng-eNB-CU and one or more ng-eNB-DU(s). An ng-eNB-CU and an ng-eNB-DU is connected via W1 interface. The general principle described in this clause also applies to ng-eNB and W1 interface, if not explicitly specified otherwise.

An gNB can support FDD mode, TDD mode or dual mode operation.

gNBs can be interconnected through the Xn interface.

A gNB may consist of a gNB-CU and one or more gNB-DU(s). A gNB-CU and a gNB-DU is connected via F1 interface.

One gNB-DU is connected to only one gNB-CU.

NOTE 2:	In case of network sharing with multiple cell ID broadcast, each Cell Identity associated with a subset of PLMNs, PNI-NPNs or SNPNs corresponds to a gNB-DU and the gNB-CU it is connected to, i.e. the corresponding gNB-DUs share the same physical layer cell resources.

NOTE 3:	For resiliency, a gNB-DU may be connected to multiple gNB-CUs by appropriate implementation.

NG, Xn and F1 are logical interfaces.

For NG-RAN, the NG and Xn-C interfaces for a gNB consisting of a gNB-CU and gNB-DUs, terminate in the gNB-CU. For EN-DC, the S1-U and X2-C interfaces for a gNB consisting of a gNB-CU and gNB-DUs, terminate in the gNB-CU. The gNB-CU and connected gNB-DUs are only visible to other gNBs and the 5GC as a gNB. A possible deployment scenario is described in Annex A.

The node hosting user plane part of NR PDCP (e.g. gNB-CU, gNB-CU-UP, and for EN-DC, MeNB or SgNB depending on the bearer split) shall perform user inactivity monitoring and further informs its inactivity or (re)activation to the node having C-plane connection towards the core network (e.g. over E1, X2). The node hosting NR RLC (e.g. gNB-DU) may perform user inactivity monitoring and further inform its inactivity or (re)activation to the node hosting control plane, e.g. gNB-CU or gNB-CU-CP.

UL PDCP configuration (i.e. how the UE uses the UL at the assisting node) is indicated via X2-C (for EN-DC), Xn-C (for NG-RAN) and F1-C. Radio Link Outage/Resume for DL and/or UL is indicated via X2-U (for EN-DC), Xn-U (for NG-RAN) and F1-U.

The NG-RAN is layered into a Radio Network Layer (RNL) and a Transport Network Layer (TNL).

The NG-RAN architecture, i.e. the NG-RAN logical nodes and interfaces between them, is defined as part of the RNL.

For each NG-RAN interface (NG, Xn, F1) the related TNL protocol and the functionality are specified. The TNL provides services for user plane transport, signalling transport.

In NG-Flex configuration, each NG-RAN node is connected to all AMFs of AMF Sets within an AMF Region supporting at least one slice also supported by the NG-RAN node except A-IoT only supporting NG-RAN node. The AMF Set and the AMF Region are defined in TS 23.501 [3].

Each A-IoT only supporting NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

If security protection for control plane and user plane data on TNL of NG-RAN interfaces has to be supported, NDS/IP TS 33.501 [13] shall be applied.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.2.1	Principle of handling Application Protocol Identities

For UE-associated signalling, an Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE  over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included.

For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

For signalling related to A-IoT command operation, per A-IoT session (corresponding to an AIOTF identifier and a Correlation ID) and per A-IoT device, a RAN A-IoT Device NGAP ID to be used via the NG-C interface is introduced, as specified below.

The definitions of AP IDs as used on NG interface or Xn interface or F1 interface or E1 interface are shown below:

RAN UE NGAP ID:

A RAN UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within an gNB. When an AMF receives an RAN UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to an AMF this is included in all UE associated NGAP signalling.

The RAN UE NGAP ID shall be unique within the logical NG-RAN node.

AMF UE NGAP ID:

An AMF UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within the AMF. When a NG-RAN node receives an AMF UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to a NG-RAN node this ID is included in all UE associated NGAP signalling.

The AMF UE NGAP ID shall be unique within an AMF Set as specified in TS 23.501 [3].

Old NG-RAN node UE XnAP ID:

An Old NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an old NG-RAN node. When a new NG-RAN node receives an Old NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a new NG-RAN node this ID is included in all UE associated XnAP signalling. The Old NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

New NG-RAN node UE XnAP ID:

A New NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a new NG-RAN node. When an old NG-RAN node receives a New NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an old NG-RAN node this ID is included in all UE associated XnAP signalling. The New NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Source NG-RAN node UE XnAP ID:

A Source NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a source NG-RAN node. When a target NG-RAN node receives a Source NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a target NG-RAN node this ID is included in all UE associated XnAP signalling. The Source NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Target NG-RAN node UE XnAP ID:

A Target NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a target NG-RAN node. When a source NG-RAN node receives a Target NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a source NG-RAN node this ID is included in all UE associated XnAP signalling. The Target NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

M-NG-RAN node UE XnAP ID:

An M-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an M-NG-RAN node for dual connectivity. When an S-NG-RAN node receives an M-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an S-NG-RAN node this ID is included in all UE associated XnAP signalling. The M-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

S-NG-RAN node UE XnAP ID:

A S-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an S-NG-RAN node for dual connectivity. When an M-NG-RAN node receives a S-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an M-NG-RAN node this ID is included in all UE associated XnAP signalling. The S-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

gNB-CU UE F1AP ID:

A gNB-CU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-CU UE F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU UE F1AP ID:

A gNB-DU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-DU UE F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP UE E1AP ID:

A gNB-CU-CP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]). When a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]) receives a gNB-CU-CP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-CP UE E1AP ID shall be unique within the gNB-CU-CP (respectively the ng-eNB-CU-CP, or the eNB-CP as defined in TS 36.401[28]) logical node.

gNB-CU-UP UE E1AP ID:

A gNB-CU-UP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]). When a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]) receives a gNB-CU-UP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-UP UE E1AP ID shall be unique within the gNB-CU-UP (respectively the ng-eNB-CU-UP, or the eNB-UP as defined in TS 36.401[28]) logical node.

ng-eNB-CU UE W1AP ID:

An ng-eNB-CU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-CU. When an ng-eNB-DU receives an ng-eNB-CU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-CU UE W1AP ID shall be unique within the ng-eNB-CU logical node.

ng-eNB-DU UE W1AP ID:

An ng-eNB-DU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-DU. When an ng-eNB-CU receives an ng-eNB-DU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-DU UE W1AP ID shall be unique within the ng-eNB-DU logical node.

gNB-CU MBS F1AP ID:

A gNB-CU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for that MBS Session. The gNB-CU MBS F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU MBS F1AP ID:

A gNB-DU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for this MBS Session. The gNB-DU MBS F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP MBS E1AP ID:

A gNB-CU-CP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-CP. When a gNB-CU-UP receives a gNB-CU-CP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for that MBS Session. The gNB-CU-CP MBS E1AP ID shall be unique within the gNB-CU-CP logical node.

gNB-CU-UP MBS E1AP ID:

A gNB-CU-UP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-UP. When a gNB-CU-CP receives a gNB-CU-UP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for this MBS Session. The gNB-CU-UP MBS E1AP ID shall be unique within the gNB-CU-UP logical node.

A-IoT Correlation ID:

An A-IoT Correlation ID shall be allocated so as to uniquely identify an A-IoT session context. When an NG-RAN node receives an A-IoT Correlation ID, it shall store it for the duration of the A-IoT service operation. The A-IoT Correlation ID shall be unique within an AIOTF as specified in TS 23.369 [z].

RAN A-IoT Device NGAP ID:

A RAN A-IoT Device NGAP ID shall be allocated so as to uniquely identify an NG-RAN node A-IoT device context over the NG interface within a NG-RAN node for a certain A-IoT session. When an AIOTF receives an RAN A-IoT Device NGAP ID, it shall store it for the duration of the A-IoT session.

The RAN A-IoT Device NGAP ID shall be unique per A-IoT session within the logical NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

## 6.X	A-IoT related contexts in an NG-RAN Node

NG-RAN node A-IoT session context:

An NG-RAN node A-IoT session context is a block of information in an NG-RAN node and encompasses control plane and user plane, transport and radio resources to support an A-IoT Session. An NG-RAN node A-IoT session context may be associated with one or multiple NG-RAN node A-IoT device contexts.

NG-RAN node A-IoT device context:

An NG-RAN node A-IoT device context is a block of information in an NG-RAN node and encompasses control plane and user plane, transport and radio resources to support an A-IoT service to one A-IoT device within an NG-RAN node A-IoT session context.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>