3GPP TSG-RAN WG3 Meeting #130	R3-258124

Dallas, US, 17th ~21th November 2025

Title:	Consideration on the presence of Reader Report List

Agenda Item:	9.2.7

Source:	Huawei, CMCC, China Unicom, China Telecom

Document for:	Discussion

# Introduction

In this contribution, we further discuss the leftover issue on the presence of Reader Report List for Ambient IoT Rel-19 WI.

# Discussion

In NGAP asn.1 part, the following can be found:

InventoryReportTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

globalgNB-ID					GlobalGNB-ID,

readerReportList				AIoT-ReaderReportList,--the presence of the IE may be revisited--

inventoryCompleteIndication		ENUMERATED {true, ...}													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

In the offline discussion in in RAN3#129bis, refers to the sod R3-257254, about the moderator’s proposal “Update the presence of Reader Report List IE in the Inventory Report Transfer IE from mandatory to optional.”, there were 7 support, 3 no strong view, and 1 objection. The following inputs were received:

Support:

- Considering of the singnalling efficiency and potential latency, it is better to change the Reader Report List IE to optional.

- OK to make this optional to allow the gNB to flexibly send Inventory Complete IE at a later time and not necessarily together with the last Inventory Report (otherwise either the last inventory report might have to be sent late when reader is sure that inventory is complete or the reader has to send it a bit early even if it’s not fully sure)

- AF, through AIOTF, can ask for aggregated reporting by providing the time interval for aggregation, therefore, it is possible that the reader collects no response to be reported for some of the interim aggregated reporting moments. If the gNB reader can only use the release procedure to indicate such case, it may prematurely terminate the service session.

Not support:

- We have a procedure for nothing to report, which is the gNB initiated Release procedure. (The CN intiated release procedure needs to be triggered as per stage-2 anyhow)

For the comment about using gNB triggered A-IoT Session Release Request procedure, we can discuss this case by case:

Upon determine the completion of the inventory procedure, i.e., no more device response:

  - if there is buffered A-IoT NAS PDU(s) which includes the device ID(s) reported by the device(s)), the gNB sends the Inventory Complete Indication IE together with the Reader Report List IE.

  - if there is no buffered A-IoT NAS PDU which includes the device ID reported by the device, currently the gNB is not able to send the Inventory Complete Indication IE in INVENTORY REPORT message, as the presence of Reader Report List IE is mandatory,

    - in case of inventory with follow-on command, what should the gNB do? (case b-1)

    - in case of inventory only procedure, what should the gNB do? (case b-2)

In RAN3#129 meeting, it was agreed that “In the case of Inventory with command scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session.”, this has already been supported by TS 38.300 and TS 23.369. for NGAP spec, to enable this in case of b-1, there are different solutions:

- Solution 1: change the presence of Reader Report List IE in the Inventory Report Transfer IE to optional, the gNB sends Inventory Report with Inventory Complete Indication and without Reader Report List;

- Solution 2: include Inventory Complete indication IE in A-IoT Session Release Request procedure as optional IE, the gNB triggers A-IoT Session Release Request procedure to provide such information;

Solution 2 is not preferred as in case there is ongoing command transmission, we should not use A-IoT Session Release (Request) procedures which are intended to be used in case of the completion/termination of the session.

In case of case b-2, indeed there is nothing wrong if the gNB triggers A-IoT Session Release Request procedure, but on ther other hand, if solution 1 is adopted for case b-1), the gNB is also be able to use it to inform the completion of the inventory.

Proposal: Change the presence of Reader Report List IE in the Inventory Report Transfer IE to optional.

If the above Proposal is not agreeable, then we may need to update stage 2 description as the gNB will not be able to always provide the Inventory Complete Indication IE by using INVENTORY REPORT message.

The stage 2 changs to TS 38.300 could be:

5./6. Upon receiving the response(s) from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node. The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s). If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried. Upon completion of the inventory procedure the gNB, the gNB sends an Inventory Complete Indication in the Inventory Report message, if applicable. If multiple Inventory Report messages are sent, the Inventory Complete Indication is included in the last Inventory Report message. NOTE 2: If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

# Conclusion

In this contribution, we further discussed the leftover issue on the presence of Reader Report List, and get the following proposal:

Proposal: Change the presence of Reader Report List IE in the Inventory Report Transfer IE to optional.

The corresponding NGAP CR is provided in [1].

# Reference

[1]	R3-258125 Correction on the presence of Reader Report List, R19 NGAP CR