3GPP TSG-RAN WG3 Meeting #130	R3-258126

Dallas, US, 17th ~21th November 2025

Title:	[TPs to BL CRs] Consideration on Ambient IoT Topology 2

Agenda Item:	14.2

Source:	Huawei, China Unicom, China Telecom

Document for:	Agreement

# Introduction

During the 3 RAN3 meetings in 2025 Q4 and 2026 Q1, only the support of Topology 2 should be discussed, and the discussion can be classified into the following topics:

- UE Reader Authorization

- UE Reader Selection

- Reader ID design

- Inter-gNB RRC-Connected UE Reader Mobility

In this contribution, we further elaborate our considerations about these topics, and provides related TPs.

# Discussion

## UE Reader Authorization

The following agreements and open issues were made in RAN3#129bis:

UE Reader Authorization is performed at 5GC

The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

It is also noticed that SA2 agreed the following:

UE reader authorization and revocation part: … - UE Reader subscription information is available to AMF. … - If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader. … - If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader. …

To achieve the RAN3 and SA2 agreements, we should include the UE Reader authorization status (authorized, non-authorized) over NGAP related messages, and revocation is also supported.

Proposal 1-1: Include UE Reader authorization status (authorized, non-authorized) in the NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, and PATH SWITCH REQUEST ACKNOWLEDGE messages.

in Oct meeting, the following RAN2 agreements were made:

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

It can be understood that the gNB will allocate AIoT resources to be used by the UE-reader via RRC dedicated signalling. And the RAN3 objectives stated that “The objective includes specification of UE reader authorization, including F1AP support, ….”, considering that the A-IoT radio resources should be controlled by the gNB-DU in case of split architecture of the macro gNB, upon receiving the UE Reader authorization status from the CN, the gNB-CU should send it to the gNB-DU via UE Context Setup/Modification Request.

Besides the UE Reader Authorization Status, some companies think that the gNB-CU should inform the gNB-DU about whether the UE is selected as the UE-reader or not. From our view, these two aspects can be discussed separately.

Proposal 1-2: The gNB-CU informs the gNB-DU about the UE Reader authorization status (authorized, non-authorized), via F1AP: UE CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST messages.

As inter-gNB RRC-Connected UE Reader Moblity is supported, and RAN2 objective mentioned RLF scenario, it is better to also include the UE Reader Authorized IE also in XnAP Handover procedure (for HO) and Retrieve UE Context procedure (for RRC re-establishment after RLF).

Proposal 1-3: The source/old gNB informs the target/new gNB about the UE Reader authorization status (authorized, non-authorized), via XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

## UE Reader Selection

In Rel-19, the Reader Selection in T1 was concluded as:

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list). If the Requested Service Area Information contains: - neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all readers it serves - the Requested AIoT Area List, the gNB shall select readers within the indicated AIoT Area(s) - the Requested Reader List, the gNB shall take the reader list into account to perform inventory.

In TR 38.769, the following can be found:

In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN. NOTE 3: How to perform reader selection needs further discussions. NOTE 4: In NAS/UP based solution, whether A-IoT CN selects the A-IoT-enabled UE for the selection needs further discussion.

In TR 23.700-30:

UE reader selection part: Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case for all types of UE reader. - For AF providing UE reader ID case: - If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN. Editor’s note: How to select UE reader(s) based on the AF provided area information is FFS.

Based on R19 T1 discussion, the CN may provide a list of reader ids to the gNB, and the gNB shall take the reader list into account. Considering of the similarity of Topology 1 and Topology 2 RRC based solution, it is better to reuse the solution adopted by R19 T1 for R20 T2, i.e., in case there are information provided by the A-IoT CN, the gNB shall take the information into account.

Proposal 2-1: Reuse R19 T1 reader selection solution for R20 T2 reader solution, i.e., upon receiving the requested service area information from the CN, the gNB shall take it into account.

Currently the Requested Service Area Information IE in Inventory Request is designed as follows:

#### 9.3.3.71	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.276 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area ID | M |  | 9.3.1.277 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

In Rel-19, if the Requested Service Area Information contains neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all readers it serves. Last meeting there was comment on the applicability of this in T2, i.e., in case two UEs are very close to each other, the gNB may only need to select one of them to perform inventory. Therefore, it is better to allow the gNB selects UE readers among all the served connected mode UEs which were authorized as UE readers.

Proposal 2-2: In case an empty Requested Service Area Information IE is received, the gNB selects UE readers among all the served connected mode UEs which were authorized as UE readers.

## Reader ID design

As NGAP procedures specified for Topology 1 are baseline for Topology 2, we need to further discuss how to design the Reader ID for UE Reader.

In 23700-30, the following can be found:

UE reader ID allocation: Editor’s note: Whether and how to allocate the UE reader ID is FFS.

In Rel-19 there are two The Reader ID is included in Inventory Request Transfer and the Inventory Report Transfer, the reader (gNB-reader) id consists of the Global gNB ID and the Reader Index, it is unclear whether this can be reused for T2 UE-reader or not.

Proposal 3: Further discuss the Reader ID to represent a UE reader, it is FFS whether Rel-19 defined Reader ID can be reused or not.

## Inter-gNB RRC-Connected UE Reader Mobility

For following two bullets:

  - Inter-gNB resource coordination is not specified.

  - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

As the UE reader is selected by the gNB with or without assistance information from CN, once the UE changes gNB, the A-IoT service provided by the UE for source gNB should be stopped, and if the new/target gNB would like to use the UE as UE reader, the new/target gNB triggers the related actions to the UE after mobility procedure. With this consideration, except the UE reader authorization status exchanging between gNBs, there is no need to introduce other A-IoT related signalling exchanging between gNBs.

Proposal 4-1: Once the UE changes gNB during mobilty, the A-IoT service provided by the UE in source gNB should be stopped.

Proposal 4-2: Except the UE reader authorization status exchanging between gNBs, there is no need to introduce other A-IoT related signalling exchanging between gNBs.

# Conclusion

This document proposes:

UE Reader Authorization

Proposal 1-1: Include UE Reader authorization status (authorized, non-authorized) in the NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, and PATH SWITCH REQUEST ACKNOWLEDGE messages.

Proposal 1-2: The gNB-CU informs the gNB-DU about the UE Reader authorization status (authorized, non-authorized), via F1AP: UE CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST messages.

Proposal 1-3: The source/old gNB informs the target/new gNB about the UE Reader authorization status (authorized, non-authorized), via XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

UE Reader Selection

Proposal 2-1: Reuse R19 T1 reader selection solution for R20 T2 reader solution, i.e., upon receiving the requested service area information from the CN, the gNB shall take it into account.

Proposal 2-2: In case an empty Requested Service Area Information IE is received, the gNB selects UE readers among all the served connected mode UEs which were authorized as UE readers.

Reader ID design

Proposal 3: Further discuss the Reader ID to represent a UE reader, it is FFS whether Rel-19 defined Reader ID can be reused or not.

Inter-gNB RRC-Connected UE Reader Mobility

Proposal 4-1: Once the UE changes gNB during mobilty, the A-IoT service provided by the UE in source gNB should be stopped.

Proposal 4-2: Except the UE reader authorization status exchanging between gNBs, there is no need to introduce other A-IoT related signalling exchanging between gNBs.

The TPs to support T2 RRC based solution in indirect connectivity, and to support UE reader authorization, are provided in the following sections.

# Text Proposal for 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

//skip unchanged part

A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF.

A-IoT device: a device that supports A-IoT radio interface towards A-IoT reader, as defined in 16.23.

A-IoT MSG1: first D2R message transmission in the A-IoT CBRA procedure, as defined in 16.23.

A-IoT MSG2: R2D message in response to A-IoT MSG1 in the A-IoT CBRA procedure, as defined in 16.23.

A-IoT reader: reader providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.23.

//skip unchanged part

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

gNB-reader: an A-IoT reader that is deployed within the gNB, as defined in 16.23.

//skip unchanged part

U2N Remote UE: a UE that communicates with the network via one or more U2N Relay UEs on an indirect path.

U2U Relay UE: a UE that provides functionality to support connectivity between two U2U Remote UEs.

U2U Remote UE: a UE that communicates with other UE(s) via a U2U Relay UE.

UE-reader: a UE that providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.23.

UE-side (AI/ML) model: an AI/ML model whose inference is performed at the UE.

UE-to-Network Relay communication: A mode of communication in which a UE communicates with the network through an indirect path involving only one U2N Relay UE for single-hop L2 U2N Relay communication or multiple L2 U2N Relay UEs for multi-hop L2 U2N Relay communication.

UE-to-Network Relay discovery: A mode of NR sidelink discovery in which a UE discovers other UEs for U2N Relay communication.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.23	Support of Ambient IoT

### 16.23.1	General

A-IoT radio interface provides the communication between A-IoT device(s) and A-IoT reader, as illustrated in Figure 16.23.1-1. In this release, the A-IoT reader is a gNB-reader or a UE-reader. A-IoT radio interface can support both inventory procedure and command procedure as defined in TS 23.369 [68]. An A-IoT device monitors the R2D message as long as it has sufficient energy.

Figure 16.23.1-1: Architecture supporting the A-IoT radio interface

### 16.23.2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or multiple A-IoT readers.

-	A gNB is aware of the mapping relationship between A-IoT Areas and gNB-readers by means of OAM configuration.

-	An A-IoT Area may include gNB-readers belonging to the same or different gNB.

-	One reader only belongs to one gNB. One UE-reader only belongs to its serving gNB, the A-IoT related signalling is carried between UE-reader and it serving gNB via RRC.

-	One gNB-reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [68], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF). A gNB is aware of direct connectivity with the AIOTF via OAM.

NOTE:	A deployment with both direct connectivity and indirect connectivity is not expected.

NOTE x:	Only indirect connectivity is supported in a deployment with UE-reader.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of gNB-readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the gNB-readers supported by the gNB.

The AIOTF may also be aware of the gNB-reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, gNB-readers and A-IoT areas by means of OAM configuration, as needed.

UE-reader authorization is performed at core network, the UE’s serving AMF provides UE-reader authorization information to the gNB during UE context management procedures.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Text Proposal for NGAP BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

//skip unchanged part

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

//skip unchanged part

-	store the received PDU Set QoS parameters, if supported, in the UE context and use it as specified in TS 23.501 [9];

-	store the received A-IoT UE Reader Authorized Authorization information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

//skip unchanged part

If the Ranging and Sidelink Positioning Service Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, update the Ranging and Sidelink Positioning service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the A-IoT UE Reader Authorized IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, update the A-IoT UE Reader authorization information for the UE accordingly. If the A-IoT UE Reader Authorized IE includes and set to “not authorized”, the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

//skip unchanged part

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and the Area Scope of MDT IE.

If the A-IoT UE Reader Authorized IE is contained in the HANDOVER REQUEST message and set to "authorized", the NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

//skip unchanged part

If the AMF UE NGAP ID 2 IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the gNB shall, if supported, store this information in the UE context and use it for subsequent Xn handovers.

If the A-IoT UE Reader Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "authorized", the NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| //skipunchanged part |  |  |  |  |  |  |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the AMF to provide UE Context information changes to the NG-RAN node.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| //skipunchanged part |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| //skipunchanged part |  |  |  |  |  |  |
| Target RAN Node ID | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| //skipunchanged part |  |  |  |  |  |  |
| AMF UE NGAP ID 2 | O |  | AMF UE NGAP ID 9.3.3.1 | This IE indicates the AMF UE NGAP ID assigned by the AMF. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xxx	A-IoT UE Reader Authorized

This IE provides the authorization status of the UE to act as an A-IoT UE Reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | M |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as A-IoT UE Reader or not. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Text Proposal for F1AP BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-IoT	Ambient IoT

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

//skip unchanged part

For each DRB that has been successfully established and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform delay measurements on the successfully established DRBs.

If the A-IoT UE Reader Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it is set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized as A-IoT UE Reader.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

//skip unchanged part

For each DRB that has been successfully established or modified and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE or in the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform or update delay measurements on the successfully established or modified DRBs.

If the A-IoT UE Reader Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its authorization information for the UE accordingly. If the A-IoT UE Reader Authorized IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer acting as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| //skip unchanged part |  |  |  |  |  |  |
| LTM Information SN Addition |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, …) |  | – |  |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| LTM Information SN Modification |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, …) |  | – |  |
| A-IoT UE Reader Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xxx	A-IoT UE Reader Authorized

This IE provides information on the authorization status of the UE to act as A-IoT UE Reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized to act as A-IoT UE Reader. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Text Proposal for XnAP BL CR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5QI	5G QoS Identifier

AI	Artificial Intelligence

A-IoT	Ambient IoT

AMF	Access and Mobility Management Function

A2X	Aircraft-to-Everything

BH	Backhaul

CAG	Closed Access Group

CGI	Cell Global Identifier

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

//skip unchanged part

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

If the A-IoT UE Reader Authorized IE is included in the HANDOVER REQUEST message and set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission. The procedure can also be used to transfer the authorization status information of the mobile IAB-node.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

//skip unchanged part

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, and the Geographical Area IE contains the MDT PLMN List IE, the new NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

If the A-IoT UE Reader Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized to act as A-IoT UE Reader.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.2.3.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| //skip unchanged part |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Services Information | O |  | 9.2.3.208 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| A-IoT UE Reader Authorized | O |  | 9.2.3.xxx |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.xxx	A-IoT UE Reader Authorized

This IE provides information on the authorization status of the UE to act as A-IoT UE Reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized to act as A-IoT UE Reader. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>