3GPP TSG-RAN WG3 Meeting #130	R3-258127

Dallas, US, 17th ~21th November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0137 | Rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on LTE-based 5G Broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, EBU, Qualcomm Incorporated, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R2-2507927, RAN2 agreed to add pmch-TimeInterleaving-N-lastMTCH = 1 as candidate value and clarify value 1 indicates time interleaving is not enabled for the last MTCH service. In M2AP, we also need to update the Value N-Last MTCH IE to include the codepoint “n1”. It is also noted that the semantics description of several IEs needs to be updated to align with latest version of RRC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Value N-Last MTCH IE to include the codepoint “n1”. Update the semantics description of several related IEs to align with RRC spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot provide the value “n1” of the Value N-Last MTCH IE to the eNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.8, 9.2.1.30, 9.2.1.31, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------Start of the change-------------------------

#### 9.2.1.8	PMCH Configuration

This information element provided PMCH configuration related content for MCCH.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated Subframes End | M |  | INTEGER (0..1535) | Encoded as the sf-AllocEnd IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme | M |  | INTEGER (0..28) | Encoded as the dataMCS IE in TS 36.331 [11]. | − | − |
| MCH Scheduling Period | M |  | ENUMERATED (rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024) | Encoded as the mch-SchedulingPeriod IE in TS 36.331 [11]. | − | − |
| Modulation and Coding Scheme 2 | O |  | INTEGER (0..27) | Encoded as the t2 in the dataMCS IE in TS 36.331 [11]. If this IE is present, the value signalled in the Modulation and Coding Scheme IE is ignored. | YES | reject |
| MCH Scheduling Period Extended | O |  | ENUMERATED (rf4, …) | Encoded as the mch-SchedulingPeriod-r12 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 2 | O |  | ENUMERATED (rf1, rf2, …) | Encoded as the mch-SchedulingPeriod-v14x0 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| MCH Scheduling Period Extended 3 | O |  | ENUMERATED (rf7, rf14, rf28, rf53, rf56, rf108, rf112, rf212, rf424, …) | Encoded as the mch-SchedulingPeriod-v1900 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. If this IE is present, the value signalled in the MCH Scheduling Period IE is ignored. | YES | reject |
| Frequency Interleaving Indicator | O |  | 9.2.1.29 |  | YES | reject |
| Time Interleaving Parameters | O |  | 9.2.1.30 |  | YES | reject |

--------------------Start of the change-------------------------

#### 9.2.1.30	Time Interleaving Parameters

This IE defines the time interleaving parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Value M | M |  | ENUMERATED (sf4, sf8, sf16, sf32, …) | Encoded as the pmch-TimeInterleavingM-r19 in the pmch-TimeInterleavingConfig-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value N | M |  | ENUMERATED (n2, n4, n8, n16, …) | Encoded as the pmch-TimeInterleavingN-r19 in the pmch-TimeInterleavingConfig-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Scaling factor Beta | M |  | ENUMERATED (one32th, one5th, one3rd, three8th, five12th, onehalf, five8th, two3rd, five6th, one, …) | Encoded as the scalingFactorBeta-r19 in the PMCH-SoftBufferSizeParameters-r19 IE in TS 36.331 [11]. |
| Reference UE Category | M |  | INTEGER (4..26, ...) | Encoded as the refUE-CategoryDL-r19 in the PMCH-SoftBufferSizeParameters-r19 IE in TS 36.331 [11]. |
| Value M-Last MTCH | O |  | ENUMERATED (sf4, sf8, sf16, sf32, …) | Encoded as the pmch-TimeInterleavingM-LastMTCH-r19 in the pmch-TimeInterleavingConfig-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Value N-Last MTCH | O |  | ENUMERATED (n1, n2, n4, n8, n16, …) | Encoded as the pmch-TimeInterleavingN-LastMTCH-r19 in the pmch-TimeInterleavingConfig-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331 [11]. |
| Cyclic Shift Alpha | O |  | ENUMERATED (alphaOne, alphaOther, …) | Encoded as the pmch-CyclicShiftAlpha-r19 in the pmch-TimeInterleavingConfig-r19 contained in the PMCH-TFI-Config-r19 IE in TS 36.331[11]. |

--------------------Start of the change-------------------------

#### 9.2.1.31	CAS Muting Parameters

This IE indicates the parameters of where the non-MBSFN subframes containing the cell acquisition signals in an MBMS-dedicated cell shall only be transmitted, as defined in TS 36.211 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| K-CAS | M |  | INTEGER (4..63, ...) | The same encoding as the k-CAS-r19 in the cas-MutingConfig-r19 contained in the SystemInformationBlockType1-MBMS-v1900 IE as specified in TS 36.331 [11]. |
| N-CAS | M |  | ENUMERATED (n2, n4, n8, n16, ...) | The same encoding as the n-CAS-r19 in the cas-MutingConfig-r19 contained in the SystemInformationBlockType1-MBMS-v1900 IE as specified in TS 36.331 [11]. |

--------------------Start of the change-------------------------

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

M2AP-Ies {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) m2ap (4) version1 (1) m2ap-Ies (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MCH-Scheduling-PeriodExtended,

id-MCH-Scheduling-PeriodExtended2,

id-Modification-PeriodExtended,

//skip unchanged part

#### -- T

TimeInterleavingParameters ::= SEQUENCE {

valueM					ENUMERATED {sf4, sf8, sf16, sf32, ...},

valueN					ENUMERATED {n2, n4, n8, n16, ...},

scalingfactorBeta		ENUMERATED {one32th, one5th, one3rd, three8th, five12th, onehalf, five8th, two3rd, five6th, one, ...},

referenceUECategory		INTEGER (4..26,...),

valueM-LastMTCH			ENUMERATED {sf4, sf8, sf16, sf32, ...}								OPTIONAL,

valueN-LastMTCH			ENUMERATED {n1, n2, n4, n8, n16, ...}									OPTIONAL,

cyclicShiftAlpha		ENUMERATED {alphaOne, alphaOther, ...}								OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TimeInterleavingParameters-ExtIEs} } OPTIONAL,

...

}

TimeInterleavingParameters-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

--------------------End of the change-------------------------