3GPP TSG-RAN WG3 Meeting #130	R3-258130

Dallas, TX, USA, 17 – 21 November 2025

Agenda item:	13.2

Source: 	Qualcomm Incorporated

Title: 	Network Architecture and Protocol Aspects for NR Sensing Support

Document for:		Discussion and Decision

# 1.	Introduction

At RAN#108, a new (RAN1-led) Study Item has been approved to study "Integrated Sensing And Communication (ISAC) for NR", which has been slightly revised at RAN#109 [1]. The objectives for this study are shown below.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For the RAN3 objectives, the study is expected to be completed at RAN#112 (June 2026; which gives five RAN3 meetings until RAN3#132). A parallel study is ongoing in SA2 [2], which is expected to be completed at SA#110 (December 2025). The RAN1 aspects are expected to be completed at RAN1#124 (February 2026 (RAN#111)).

At RAN3#129bis, the logical architecture to support sensing was endorsed for TR 38.765 [3] with the following agreements:

Capture logical architecture for ISAC RAN3 focus on sensing protocol in coordination with SA2 Continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

SA2 agreed on some "interim conclusions" at SA2#171 relevant to the RAN3 work.

WT-1: Architecture and function enhancements to support sensing [4]:

Principle 1: One new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2: No dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. NOTE 5: Other KI’s interim conclusions will be aligned with KI#1 conclusions.

WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities [5]:

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area. NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation [6]:

The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor’s Note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

From the above "interim conclusions" we can observe the following.

Observation 1:	There is currently no "interim conclusion" within SA2 regarding the selection of a preferred architecture for SF connectivity (i.e., direct connectivity between SF and gNB or connectivity via an AMF).

Observation 2:	The Sensing Function (SF) may contain "Sensing Control Functionality (SCF)" and "Sensing Processing Functionality (SPF)". Since this functionality split is internal to the SF it may have no RAN3 impacts.

Observation 3:	The Sensing Entities (SE) (i.e., the gNBs) for the sensing operation are selected by the Sensing Function (SF) based on the sensing service request (e.g., target area) and functionalities supported by the Sensing Entities (gNBs).

Observation 4:	Sensing data/measurements may be provided from the gNB to the SF "on demand" or "periodically".

Observation 5:	The Sensing Result, derived from the collected sensing data/measurements, may be computed by the Sensing Function (SF).

RAN1 continued the study on Integrated Sensing and Communication (ISAC) for NR at RAN1#122bis [7]. Agreements were made on the evaluation assumptions and performance evaluation [7].

Observation 6:	At present, RAN1 has not provided any details regarding the nature, format, or content of the sensing data/measurements that may be reported by the gNBs.

In this contribution, we continue the discussion from [8] on the network architecture and protocol aspects to support gNB sensing taking the current SA2 and RAN1 status above into account. Accordingly, a TP to the pCR for TR 38.765 [3] is proposed in the Annex of this contribution.

# 2.	System Architecture and Protocol Aspects

The two high-level architecture options under consideration in SA2 to provide support for sensing are illustrated in Figure 1.

- There is a "direct connection/interface" between the SF and the gNB.

- There is an "indirect connection/interface" between the SF and the gNB, where the communication between SF and gNB is via an AMF.

Figure 1: Illustration of Architecture Options.

Independent of the architecture options, a RAN3 application protocol (referred to as "Sensing Protocol" in the following) would be required to support the various sensing operations. This protocol should be end-to-end between the SF and the gNB, independent on whether it is routed via an AMF or not. If the transport is via an AMF (Option (2) above), the protocol stack could be the NG interface stack with SBI between SF and AMF, as illustrated in Figure 2.

For direct connectivity between gNB and SF (Option (1)), the transport layers for the "Sensing Protocol" would need to be defined, however, the actual "Sensing Protocol" could be independent of the transport. If the "direct interface" is "NG-like", a possible protocol stack is illustrated in Figure 3. An "NGAP-like" protocol (referred to as NxAP) could be defined for interface management and transport of the "Sensing Protocol". Essentially, the NxAP could be a subset of NGAP with the following services, functions, and procedures.

-	Non UE-associated services: Nx interface instance between the gNB and SF utilising a non UE-associated signalling connection.

-	"Sensing Protocol" transport function (analogous to NGAP Uplink/Downlink non-UE Associated NRPPa Transport).

-	Nx Interface Management procedures (e.g., Nx Setup, Nx Reset, etc.).

In principle, one could use NGAP for the above services, functions, and procedures, similar to A-IoT, where an A-IoT CN Node may initiate procedures over NG interface.  For example, the NGAP Uplink/Downlink non-UE Associated NRPPa Transport could be extended to carry "Sensing Protocol" PDUs. This would be straightforward if NRPPa includes also the sensing procedures (see also [9]).  However, this would mean a lot of overhead for a SF since only a very small subset of NGAP would be applicable. In addition, it may not be appropriate to define the "Nx Interface" as "NG Interface".

Observation 7:	An end-to-end "Sensing Protocol" to support the various sensing operations could be defined independent on the architecture options under consideration in SA2.

Figure 2: Possible Protocol Layering for SF to gNB Signalling for Architecture Option 2.

Figure 3: Possible Protocol Layering for SF to gNB Signalling for Architecture Option 1.

For positioning [13], the separation of the "Positioning Protocol" (e.g., LPP [10] and NRPPa [11]) from the transport protocol has proven beneficial. This design allows the positioning protocol to operate over various transport layers, such as NAS or IP protocols. Such modularity enables reuse of the same positioning logic regardless of the underlying transport, simplifying both development and (specification) maintenance.

For instance, LPP [10] can be applied across multiple access technologies, including LTE, NR, WiFi, etc. Similarly, a 5G "Sensing Protocol" need not be limited to 5G or a specific transport like NG-C. Even if the sensing protocol is confined to 5G, decoupling it from the transport layer allows the sensing functions to evolve independently of the transport mechanisms.

As an example, when new sensing procedures are introduced in future releases, only the sensing protocol components in the gNB and SF implementations would need to be updated and tested. The transport layer and interface management – including for example NxAP or NGAP – can remain unchanged. This separation supports faster and simpler deployment of enhancements in sensing techniques across future 3GPP Releases, since not a complete new NxAP or NGAP etc. must be implemented and tested. This is in particular the case if "NGAP" should be reused for the Nx-interface, since any sensing procedures/PDUs should not impact the AMF (i.e., it would mean unnecessary "Sensing Functionality" (ASN.1) overhead for an AMF, since NG is still the interface between NG-RAN and an AMF).

Observation 8: 	Separation of the "Sensing Protocol" from the transport layer enables modularity, cross-technology applicability, and independent evolution of sensing functions, facilitating faster and simpler deployment of enhancements without requiring changes to the underlying interface management and transport mechanisms.

Proposal 1:	RAN3 consider an end-to-end (SF–gNB) "Sensing Protocol", independent on the lower interface management and transport layers.

# 3.	Sensing Data/Results

The Stage 1 service requirements for Integrated Sensing and Communication TS 22.137 [12] defines various performance metrics for the sensing capabilities in 5G networks:

| Metric | Description |
| --- | --- |
| Positioning Accuracy | Horizontal and vertical accuracy in meters. |
| Velocity Accuracy | Horizontal and vertical velocity estimation accuracy in m/s. |
| Confidence Level | The probability that the sensing estimate (e.g., position or velocity) falls within the specified limits, e.g., 95%, 99%. |
| Sensing Resolution | Includes range resolution and velocity resolution. |
| Missed detection/false alarm probability | Percentage of targets not detected when they should be/the percentage of detections that are incorrect (i.e., no actual target present). |
| Latency ("Response Time") | Time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result. |
| Refresh Rate ("reporting interval") | Inverse of the time elapsed between two successive sensing results. |

Performance requirements for various use cases using the metrics above are defined in [12]. Apart from the missed detection/false alarm probability, the sensing performance indicators are the same as for positioning (incl. sidelink positioning) and location services [13].

Observation 9:	The key performance metrics defining the service requirements for Integrated Sensing and Communication are similar to positioning/location services (e.g., positioning/velocity accuracy, etc.).

One aspect which may affect the selection of the architecture and "Sensing Protocol" transport is the type of "data" (data volume) which need to be exchanged between SF and gNB (i.e., the "Sensing Data/Measurement Report"). This is being studied in RAN1 (see RAN1 objectives of the SID shown in section 1 above), and there appears the following general options:

(1)	"Sensing Measurements", i.e., (raw) physical layer measurements representing range, angle, doppler (RAD), etc. measurements. These measurements may be similar or the same as for positioning (incl. sidelink positioning) such as RTT, AoA, range and direction, etc.

(2)		Position/velocity of detected object(s), incl. a "point cloud" (i.e., a collection of points in a three-dimensional coordinate system used to represent the shape and surface of the detected object).

Analogous to positioning [13], the above general options could be mapped to "gNB-assisted" and "gNB-based", referring to the node that is responsible for determining the position/velocity of the object(s) (SF or gNB).

The specific measurement details reported by a gNB are subject to ongoing developments and conclusions within RAN1.

Observation 10:	Two general "sensing measurement types" are under consideration in RAN1:

(1)	"Sensing Measurements", i.e., (raw) physical layer measurements representing range, angle, Doppler (RAD), etc. measurements.

(2)	Position/velocity of detected object(s), incl. a "point cloud" (i.e., a collection of points in a three-dimensional coordinate system used to represent the shape and surface of the detected object).

However, according to Observation 6 in section 1 above, no details regarding the nature, format, or content of the sensing data/measurements are currently available from RAN1.

For both "sensing measurement types" above, it is assumed that a final processing step is needed (either at SF or gNB). This processing step would interpret the "sensing data" and extract the meaningful sensing information, such as identifying and classifying the target object (e.g., distinguishing a specific UAV from other detected objects). This information is referred to as "Sensing Result" in TS 22.137 [12] and defined as: "Processed 3GPP sensing data requested by a service consumer". It is assumed that the details of the "Sensing Service Request/Result" will be defined by SA2. According to Observation 5 in section 1 above, SA2 assumes this final processing step is performed by the SF.

Observation 11:	Regardless of the general "sensing measurement type", it is assumed that a final processing step is needed, which would interpret the results and extract the meaningful sensing information, such as identifying and classifying the target object (e.g., distinguishing a specific UAV, etc.). This final processing step could be at the gNB or SF.

The "sensing measurement type" may impose distinct requirements on several system aspects. In particular, the two types may differ in terms of the volume and size of data exchanged between the gNB and SF. Additionally, they may impact the service request information required at the gNB, such as object classification parameters, Quality of Service (QoS) indicators, etc.

Proposal 2:	RAN3 should await further input from RAN1 regarding the nature, format, or content of the sensing data/measurements reported by a gNB. This information may impact e.g., the selection of appropriate transport for the "Sensing Protocol".

# 4.	RAN Architecture

The gNB architecture to support sensing could be the same as for positioning [13] by defining Transmission-Reception Points (TRPs) hosted by the gNB-DUs as illustrated in Figure 4. The TRPs may support Transmission Point (TP) functionality and/or Reception Point (RP) functionality. For bi-static sensing for example, the TP and RP functionality may not be collocated.

Figure 4: gNB Architecture.

The Nx interface would terminate at the gNB-CU and the F1 interface supports the sensing operation between gNB-CU and gNB-DU/TRP, similar to positioning [13]. Note however, that the F1 interface may not need to replicate all the procedures to be defined over the Nx interface like in F1AP for positioning [11], [14], but may just forward OCTET STRINGs with embedded "Sensing Protocol" PDUs to the appropriate gNB-DU/TRP, which is discussed in greater detail in [9]. A corresponding protocol layering is illustrated in Figure 5, where lower layers at gNB-CU and SF side depends on the architecture options discussed in section 2.

Figure 5: Possible Protocol Layering for SF to gNB-DU Signalling (see also [9]). The empty boxes depend on the architecture option discussed in section 2.

Observation 12:	The gNB architecture to support sensing could be the same as for positioning support by defining Transmission-Reception Points (TRPs) in the gNB-DUs to support sensing signal transmission and/or reception.

The RP component of the TRP within a gNB-DU is responsible for signal acquisition and the extraction of sensing data. Regardless of the sensing measurement type – whether raw physical layer measurements (e.g., range, angle, Doppler, etc.) or higher-level object information (e.g., position, velocity, point cloud) – the generation of sensing data can be performed locally at the gNB-DU. The gNB-DU processes the reflected signals to derive essential parameters such as range, angle, and Doppler shift, and translates these into location/velocity of the object(s). This enables real-time object detection directly at the gNB-DU. The gNB-CU serves primarily as a routing entity, forwarding the Sensing Protocol PDUs to the appropriate gNB-DU, as further discussed in [9], possibily reducing the processing latency.

Proposal 3:	The gNB includes "Transmission-Reception Point (TRP)" functionality to support sensing signal transmission (TP) and/or reception (RP).

Proposal 4:	In case of a split-gNB, the gNB-DU determines the "sensing data" (raw sensing measurements or position/velocity of the object(s) (e.g. "point-cloud")).

Proposal 5:	The gNB-CU can be transparent to the "sensing signalling" and would only need to route the "sensing signalling" PDU to the desired gNB-DU/TRP.

# 5.	Summary

In this contribution, we discussed the network architecture to support gNB sensing and the general protocol aspects which need to be studied to enable the gNB sensing capability in 5G. The following Observations and Proposals were made.

Status in Other WGs

Observation 1:	There is currently no "interim conclusion" within SA2 regarding the selection of a preferred architecture for SF connectivity (i.e., direct connectivity between SF and gNB or connectivity via an AMF).

Observation 2:	The Sensing Function (SF) may contain "Sensing Control Functionality (SCF)" and "Sensing Processing Functionality (SPF)". Since this functionality split is internal to the SF it may have no RAN3 impacts.

Observation 3:	The Sensing Entities (SE) (i.e., the gNBs) for the sensing operation are selected by the Sensing Function (SF) based on the sensing service request (e.g., target area) and functionalities supported by the Sensing Entities (gNBs).

Observation 4:	Sensing data/measurements may be provided from the gNB to the SF "on demand" or "periodically".

Observation 5:	The Sensing Result, derived from the collected sensing data/measurements, may be computed by the Sensing Function (SF).

Observation 6:	At present, RAN1 has not provided any details regarding the nature, format, or content of the sensing data/measurements that may be reported by the gNBs.

System Architecture and General Protocol Aspects

Observation 7:	An end-to-end "Sensing Protocol" to support the various sensing operations could be defined independent on the architecture options under consideration in SA2.

Observation 8: 	Separation of the "Sensing Protocol" from the transport layer enables modularity, cross-technology applicability, and independent evolution of sensing functions, facilitating faster and simpler deployment of enhancements without requiring changes to the underlying interface management and transport mechanisms.

Proposal 1:	RAN3 consider an end-to-end (SF–gNB) "Sensing Protocol", independent on the lower interface management and transport layers.

Sensing Data and Results

Observation 9:	The key performance metrics defining the service requirements for Integrated Sensing and Communication are similar to positioning/location services (e.g., positioning/velocity accuracy, etc.).

Observation 10:	Two general "sensing measurement types" are under consideration in RAN1:

(1)	"Sensing Measurements", i.e., (raw) physical layer measurements representing range, angle, Doppler (RAD), etc. measurements.

(2)	Position/velocity of detected object(s), incl. a "point cloud" (i.e., a collection of points in a three-dimensional coordinate system used to represent the shape and surface of the detected object).

Observation 11:	Regardless of the general "sensing measurement type", it is assumed that a final processing step is needed, which would interpret the results and extract the meaningful sensing information, such as identifying and classifying the target object (e.g., distinguishing a specific UAV, etc.). This final processing step could be at the gNB or SF.

Proposal 2:	RAN3 should await further input from RAN1 regarding the nature, format, or content of the sensing data/measurements reported by a gNB. This information may impact e.g., the selection of appropriate transport for the "Sensing Protocol".

RAN Architecture

Observation 12:	The gNB architecture to support sensing could be the same as for positioning support by defining Transmission-Reception Points (TRPs) in the gNB-DUs to support sensing signal transmission and/or reception.

Proposal 3:	The gNB includes "Transmission-Reception Point (TRP)" functionality to support sensing signal transmission (TP) and/or reception (RP).

Proposal 4:	In case of a split-gNB, the gNB-DU determines the "sensing data" (raw sensing measurements or position/velocity of the object(s) (e.g. "point-cloud")).

Proposal 5:	The gNB-CU can be transparent to the "sensing signalling" and would only need to route the "sensing signalling" PDU to the desired gNB-DU/TRP.

TP for pCR to TR 38.765

Proposal 6:	Endorse the TP for pCR to TR 38.765 provided in the Annex of this contribution.

# References

[1]	RP-252819, "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[2]	SP-250401, "New SID on Study on Stage 2 for Integrated Sensing and Communication".

[3]	R3-257332, "(pCR to TR 38.765) TR skeleton for Study on Integrated Sensing And Communication (ISAC) for NR", China Telecom, Xiaomi.

[4]	S2-2509659, "Interim agreement/Conclusion for KI#1 System Architecture to Support Sensing", Xiaomi, et al.

[5]	S2-2509825, "Interim agreements for FS_Sensing_ARC KI#3 ", Qualcomm Incorporated, et al.

[6]	S2-2509826, "Interim agreements and further considerations for KI#4", Ericsson, et al.

[7]	Chair Notes RAN1#122bis, RAN1.

[8]	R3-256826, "Network Architecture and Protocol Aspects for NR Sensing Support", Qualcomm Incorporated.

[9]	 R3-258131, "Signalling and Procedures for NR Sensing Support", Qualcomm Incorporated.

[10]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[11]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[12]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[13]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[14]	3GPP TS 38.473: "F1 application protocol (F1AP)".

# Annex: TP to (BL) pCR TR 38.765

# 7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

## 7.1	Logical Architecture for ISAC

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

## 7.2	NG-RAN Architecture for ISAC

A gNB may include "Transmission-Reception Point (TRP)" functionality to support sensing signal transmission (TP) and/or reception (RP), as illustrated in Figure 7.2.

In case of split gNB architecture, a gNB-DU may include TRP functionality where the TRP functionality may support functions for a TP, RP or both TP and RP.

Figure 7.2: gNB Architecture for ISAC.

## 7.3	NG-RAN terminated protocols

## 7.3.1	Sensing Protocol

The Sensing Protocol carries information between the gNB and the SF. It is used to support the following sensing functions:

-	Exchange of sensing measurement/data between gNB and SF.

Editor's Note:	The name of the "Sensing Protocol" is FFS.

Editor's Note:	Other functions of the "Sensing Protocol" are FFS.

## 7.3.2	Nx Application Protocol

NxAP provides the non UE-associated signalling service between the gNB and the SF that is required to fulfil the following functions:

-	Nx Interface Management function (e.g., Nx Setup, Nx Reset, etc.).

-	"Sensing Protocol" transport function.

Editor's Note:	Whether NGAP can be used on the Nx interface instead is FFS.

### 7.3.3	F1 Application Protocol

F1AP provides the non UE-associated signalling service between the gNB-DU and the gNB-CU that is required to fulfil the following sensing functions:

-	"Sensing Protocol" transport function.

## 7.4	Signalling between an SF and gNB

## 7.4.1	Protocol Layering

## Editor's Note: The Protocol Layering used to support transfer of Signalling Protocol messages between an SF and gNB will be described when the ISAC architecture is completed in SA2.

### 7.4.2	Sensing Protocol PDU Transfer

Figure 7.4.2-1 shows the Sensing Protocol PDU transfer between an SF and gNB.

Figure 7.4.2-1: Sensing Protocol PDU Transfer between an SF and gNB.

1.	The SF sends the Sensing Protocol PDU to the identified gNB in a NxAP Downlink Non-UE Associated Sensing Protocol Transport message.

2.	[Optional] In case of a split gNB architecture, the gNB-CU forwards the Sensing Protocol PDU to the identified gNB-DU/TRP.

3.	[Optional] In case of a split gNB architecture, the gNB-DU/TRP sends a Sensing Protocol PDU to the gNB-CU.

4.	The gNB sends a Sensing Protocol PDU to the SF in an NxAP Uplink Non UE Associated Sensing Protocol Transport message. In case of Step 3 occurred, the gNB-CU includes the Sensing Protocol PDU received from the gNB-DU in the NxAP Uplink Non UE Associated Sensing Protocol Transport message.

Editor's Note:	In case of Nx transport is via an AMF, steps 1 and 4 would occur between an AMF and gNB. In this case, the Sensing Protocol PDU would be provided from the SF to the AMF in a service operation (see also Note in section 7.4.1).