3GPP TSG-RAN WG3 Meeting #130	R3-258131

Dallas, TX, USA, 17 – 21 November 2025

Agenda item:	13.3

Source: 	Qualcomm Incorporated

Title: 	Signalling and Procedures for NR Sensing Support

Document for:		Discussion and Decision

# 1.	Introduction

At RAN#108, a new (RAN1-led) Study Item has been approved to study "Integrated Sensing And Communication (ISAC) for NR", which has been slightly revised at RAN#109 [1]. The objectives for this study are shown below.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For the RAN3 objectives, the study is expected to be completed at RAN#112 (June 2026; which gives five RAN3 meetings until RAN3#132). A parallel study is ongoing in SA2 [2], which is expected to be completed at SA#110 (December 2025). The RAN1 aspects are expected to be completed at RAN1#124 (February 2026 (RAN#111)).

At RAN3#129bis, the signalling and procedures to support sensing was endorsed for TR 38.765 [3] with:

Continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.

SA2 agreed on some "interim conclusions" at SA2#171 relevant to the RAN3 work.

WT-1: Architecture and function enhancements to support sensing [4]:

Principle 1: One new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2: No dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. NOTE 5: Other KI’s interim conclusions will be aligned with KI#1 conclusions.

WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities [5]:

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area. NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation [6]:

The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor’s Note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

From the above "interim conclusions" we can observe the following.

Observation 1:	There is currently no "interim conclusion" within SA2 regarding the selection of a preferred architecture for SF connectivity (i.e., direct connectivity between SF and gNB or connectivity via an AMF).

Observation 2:	The Sensing Function (SF) may contain "Sensing Control Functionality (SCF)" and "Sensing Processing Functionality (SPF)". Since this functionality split is internal to the SF it may have no RAN3 impacts.

Observation 3:	The Sensing Entities (SE) (i.e., the gNBs) for the sensing operation are selected by the Sensing Function (SF) based on the sensing service request (e.g., target area) and functionalities supported by the Sensing Entities (gNBs).

Observation 4:	Sensing data/measurements may be provided from the gNB to the SF "on demand" or "periodically".

Observation 5:	The Sensing Result, derived from the collected sensing data/measurements, may be computed by the Sensing Function (SF).

RAN1 continued the study on Integrated Sensing and Communication (ISAC) for NR at RAN1#122bis [7]. Agreements were made on the evaluation assumptions and performance evaluation [7].

Observation 6:	At present, RAN1 has not provided any details regarding the nature, format, or content of the sensing data/measurements that may be reported by the gNBs.

Some general aspects on the architecture and protocols to support sensing are discussed in our companion contribution in [8].

In this contribution, we continue the discussion from [9] on the signalling and procedures to support gNB sensing taking the current SA2 and RAN1 status above into account. Accordingly, a TP to the pCR for TR 38.765 [3] is proposed in the Annex of this contribution.

# 2.	Procedures and signalling between SF and gNB

New procedures between the Sensing Function (SF, see [8]) and gNB are required to support (at least) one or more of the following functions.

-	Request Sensing TRP information by an SF from the gNB, such as TP and/or RP functionality, geographical coordinates of the TPs and RPs, and possibly other configuration information (e.g., beam antenna information, etc., dependent on RAN1).

-	Request DL "Sensing Signal Information" by an SF from a TP which may be required to perform the sensing measurements at an RP (e.g., Sensing Signal Transmission Characteristics, dependent on RAN1), e.g., if TPs and RPs are not co-located (bi-static).

-	Request Sensing Signal Measurements/Results by an SF from an RP, possibly including the Sensing Signal Transmission Characteristics (e.g., frequency, bandwidth, etc.).

By comparing the above with positioning [10], [11], one can observe that the above would resemble the following NRPPa procedures [11]:

-	TRP Information Exchange

-	Positioning Information Exchange

-	Measurement Information Transfer

Accordingly, a general procedure for the sensing operation could be as shown in Figure 1.

Figure 1: Sensing Procedure between SF and gNB.

(1)	The Sensing Function (SF) receives a Sensing Request from some entity in the core network. The details of this Sensing Request may be defined by SA2. However, it is assumed that this Sensing Request includes (at least) some information on the sensing area (e.g., a geographical area where the sensing should be performed to detect the object, together with sensing quality of service requirements (QoS) such as desired location/velocity accuracy).

(2)	The SF may perform a TRP Information Exchange procedure to obtain required information from gNBs/TRPs, such as geographical coordinates of TRPs, sensing functionality (e.g., TP and/or RP functionality, etc.). The SF may send a TRP INFORMATION REQUEST message to the gNB to request the desired information, which the gNB may then provide in a TRP INFORMATION RESPONSE message. If the gNB is not able to provide any information, it may return an TRP INFORMATION FAILURE message indicating the cause of the failure.

(3)	The SF may select TRPs in the sensing area which should perform the sensing operation.

NOTE 1:	According to the SID, only single-TRP sensing is considered with no coordination between gNBs. However, the sensing operation may still need to be performed by multiple gNBs/TRPs in the sensing area, since the location of the desired object may not be known a priori, and the desired object (UAV in our case) is generally moving (see also Observation 3).

NOTE 2:	According to the SID, applicability to bi-static sensing may be considered. In this case, the SF may select the TP(s) and RP(s) pairs performing the sensing operation.

(4)	The SF may request a Sensing Signal Configuration from the sensing TP. The request may include a desired sensing signal characteristic, such as desired bandwidth of the sensing signal. The SF may send a SENSING INFORMATION REQUEST message to the gNB/TP to request a sensing signal configuration, which the gNB/TP provides in a SENSING INFORMATION RESPONSE message to the SF. If the gNB is not able to configure a DL sensing signal (radio resources to generate sensing measurements/data) at the requested TP, it may return a SENSING INFORMATION FAILURE message indicating the cause of the failure.

(5)	The SF sends a SENSING DATA REQUEST message to the selected RP to request sensing measurement information. The message includes any information required for the RP to perform the measurements, e.g., frequency, bandwidth, etc. of the sensing signal (as obtained at Step 4).

(6)	If the report characteristics in step 5 is set to "on demand", the RP obtains the requested sensing measurement information and returns them in a SENSING DATA RESPONSE message to the SF. The SENSING DATA RESPONSE message includes the obtained sensing measurement information.

If the report characteristics in step 5 is set to "periodic", the RP replies with a SENSING DATA RESPONSE message without including any sensing measurements in the message. The RP then periodically initiates the SENSING DATA REPORT procedure in step 7, at the requested reporting periodicity.

If the gNB/RP is not able to accept the SENSING DATA REQUEST message in step 5, the gNB/RP returns a failure message indicating the cause of the failure.

(7)	The RP periodically provides the sensing measurement information to the SF if that was requested at step 5.

(8)	At any time after step 6, the SF may send a SENSING DATA UPDATE message to the RP providing updated information required for the RP to perform the sensing measurements. Upon receiving the message, the RP overwrites the previously received measurement configuration information.

(9)	If the previously requested sensing measurement information can no longer be reported, the RP notifies the SF by sending a SENSING DATA FAILURE message.

(10)	When the SF wants to abort an ongoing sensing measurement it sends a SENSING DATA ABORT message to the gNB.

Observation 7:	A SF needs to have control and visibility of individual TRPs in (at least some) sensing procedures.

Observation 8:	A sensing session could be performed with the following elementary procedures:
-	TRP Information Exchange
-	Sensing Signal Information Exchange
-	Sensing Data Information Transfer.

Observation 9:	The elementary procedures according to Observation 8 would be analogous to the corresponding NRPPa procedures:
-	TRP Information Exchange
-	Positioning Information Exchange
-	Measurement Information Transfer.

Proposal 1:	A "Sensing Protocol" should support at least the following elementary procedures:
-	TRP Information Exchange (to exchange TRP related information),
-	Sensing Signal Information Exchange (to exchange sensing signal characteristics),
-	Sensing Data Information Transfer (to exchange the sensing measurements/results).

Given Observation 9, RAN3 should study whether NRPPa could be extended to support the sensing operations (instead of defining a new "Sensing Protocol").

Proposal 2:	RAN3 consider the possibility of extending NRPPa to support sensing.

# 3.	Signalling between gNB-CU and gNB-DU

For the "Sensing Protocol", the messages and functionality should not depend on the specific architecture deployed (e.g. monolithic- or split gNB). The NRPPa elementary procedures according to Observation 9 [11] are also mirrored in F1AP [12]. This duplication of messages seems unnecessary and increases the specification maintenance and implementation complexity. Based on the procedure in section 2 above, the "Sensing Protocol" messages specifically address TRP(s) (TP and/or RP). Corresponding response messages will also be structured per TRP (e.g., sensing results of a specific RP). In case of a split-gNB, the gNB-CU will essentially route these requests to the gNB-DU hosting the TRP(s). Therefore, the structure of the F1AP message would be very similar or the same to that in the "Sensing Protocol".

Observation 10:	A number of "Sensing Protocol" messages may include (a list of) TRP(s) with associated requests or responses, and the resulting F1 messages will have the same structure.

In order to avoid duplicated sensing message definition in the "Sensing Protocol" and in F1AP (which increases specification maintenance effort and could increase implementation), it would be desirable to collect all sensing related messages and procedures in a single protocol ("Sensing Protocol"). This would be straightforward if "Sensing Protocol" messages can be relayed through a gNB-CU without change between an SF and a gNB-DU/TRP.

For example, Figure 2 below shows a typical NRPPa message structure [11] which includes some header information and TRP-level information. The messages for the "Sensing Protocol" (whether new protocol or extension of NRPPa) are expected to have the same structure.

Figure 2: Generic message structure for TRP information.

In case of aggregated architecture, the gNB processes the information in "Sensing Messages" and controls TRPs based on implementation. In the case of the split architecture, the gNB-CU would only need to process some header information and forward the TRP-level Information to specific TRPs in F1AP messages.

The TRP-level information would not need to be comprehended by the gNB-CU. The TRP-level information could be included in OCTET STRINGs whose internal encoding would be defined in the "Sensing Protocol" only (e.g., NRPPa if re-used for sensing). This would enable simple copy/paste without interpretation, when transferring TRP-level information from a "Sensing Protocol" message to an F1AP message or the reverse. However, interpretation could still take place in the gNB-CU, if required (just as RRC messages conveyed in XnAP can be optionally interpreted in some cases).

Observation 11:	A generic F1-AP UL/DL Transport Message could be defined, which carries an OCTET STRING container. The message definition required for gNB-CU and gNB-DU/TRP signalling can be defined in the "Sensing Protocol".

This approach would not require mirroring every single "Sensing Protocol" change in F1AP, which has proven to be a rather cumbersome process in positioning and may also lead to errors (either in the specs or implementation).

Note, if existing NRPPa procedures — such as TRP Information Exchange — are extended to support sensing (as outlined in Observations 8 and 9), the current approach of mirroring NRPPa changes to F1AP will still be necessary. However, any newly introduced sensing-specific messages — whether integrated into NRPPa or defined within a dedicated "Sensing Protocol" — can adopt the simpler design principles described above (e.g., Sensing Data Information Transfer, which may have more stringent "real-time" requirements).

Proposal 3:	RAN3 consider a generic F1-AP UL/DL Transport Message for sensing messages, which carry an OCTET STRING container for the TRP-level message information whose internal encoding would be defined in the "Sensing Protocol" only.

# 4.	Summary

In this contribution, we discussed the general procedures to support sensing. The following Observations and Proposals were made.

Status in other WGs

Observation 1:	There is currently no "interim conclusion" within SA2 regarding the selection of a preferred architecture for SF connectivity (i.e., direct connectivity between SF and gNB or connectivity via an AMF).

Observation 2:	The Sensing Function (SF) may contain "Sensing Control Functionality (SCF)" and "Sensing Processing Functionality (SPF)". Since this functionality split is internal to the SF it may have no RAN3 impacts.

Observation 3:	The Sensing Entities (SE) (i.e., the gNBs) for the sensing operation are selected by the Sensing Function (SF) based on the sensing service request (e.g., target area) and functionalities supported by the Sensing Entities (gNBs).

Observation 4:	Sensing data/measurements may be provided from the gNB to the SF "on demand" or "periodically".

Observation 5:	The Sensing Result, derived from the collected sensing data/measurements, may be computed by the Sensing Function (SF).

Observation 6:	At present, RAN1 has not provided any details regarding the nature, format, or content of the sensing data/measurements that may be reported by the gNBs.

Elementary Sensing Procedures

Observation 7:	A SF needs to have control and visibility of individual TRPs in (at least some) sensing procedures.

Observation 8:	A sensing session could be performed with the following elementary procedures:
-	TRP Information Exchange
-	Sensing Signal Information Exchange
-	Sensing Data Information Transfer.

Observation 9:	The elementary procedures according to Observation 8 would be analogous to the corresponding NRPPa procedures:
-	TRP Information Exchange
-	Positioning Information Exchange
-	Measurement Information Transfer.

Proposal 1:	A "Sensing Protocol" should support at least the following elementary procedures:
-	TRP Information Exchange (to exchange TRP related information),
-	Sensing Signal Information Exchange (to exchange sensing signal characteristics),
-	Sensing Data Information Transfer (to exchange the sensing measurements/results).

Proposal 2:	RAN3 consider the possibility of extending NRPPa to support sensing.

Signalling between gNB-CU and gNB-DU

Observation 10:	A number of "Sensing Protocol" messages may include (a list of) TRP(s) with associated requests or responses, and the resulting F1 messages will have the same structure.

Observation 11:	A generic F1-AP UL/DL Transport Message could be defined, which carries an OCTET STRING container. The message definition required for gNB-CU and gNB-DU/TRP signalling can be defined in the "Sensing Protocol".

Proposal 3:	RAN3 consider a generic F1-AP UL/DL Transport Message for sensing messages, which carry an OCTET STRING container for the TRP-level message information whose internal encoding would be defined in the "Sensing Protocol" only.

TP for pCR to TR 38.765

Proposal 4:	Endorse the TP for pCR to TR 38.765 provided in the Annex of this contribution.

# References

[1]	RP-252819, "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[2]	SP-250401, "New SID on Study on Stage 2 for Integrated Sensing and Communication".

[3]	R3-257332, "(pCR to TR 38.765) TR skeleton for Study on Integrated Sensing And Communication (ISAC) for NR", China Telecom, Xiaomi.

[4]	S2-2509659, "Interim agreement/Conclusion for KI#1 System Architecture to Support Sensing", Xiaomi, et al.

[5]	S2-2509825, "Interim agreements for FS_Sensing_ARC KI#3 ", Qualcomm Incorporated, et al.

[6]	S2-2509826, "Interim agreements and further considerations for KI#4", Ericsson, et al.

[7]	Chair Notes RAN1#122bis, RAN1.

[8]	R3-258130, "Network Architecture and Protocol Aspects for NR Sensing Support", Qualcomm Incorporated.

[9]	R3-256827, "Signalling and Procedures for NR Sensing Support", Qualcomm Incorporated.

[10]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[12]	3GPP TS 38.473: "F1 application protocol (F1AP)".

# Annex: TP to (BL) pCR TR 38.765

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x	Exemplary Sensing Procedure

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing operation

1.	The SF may perform a TRP Information Exchange procedure to obtain required information from gNBs/TRPs, such as geographical coordinates of TRPs, sensing functionality (e.g., TP and/or RP functionality, etc.).

2.	The SF may select TRPs in the sensing area which should perform the sensing operation.

3.	The SF may request a Sensing Signal Configuration from the sensing gNB/TP. The request may include a desired sensing signal characteristic, such as desired bandwidth of the sensing signal.

Editor's Note: The need and details for this procedure depends on further RAN1 progress.

4.	The SF sends a Sensing Request to the selected gNB/RP to request sensing measurement information. The message includes any information required for the RP to perform the measurements, e.g., frequency, bandwidth, etc. of the sensing signal (as obtained at Step 3).

5.	If the report characteristics in step 4 is set to "on demand", the gNB/RP obtains the requested sensing measurement information and sends them in a Sensing Response to the SF.

If the report characteristics in step 4 is set to "periodic", the gNB/RP replies with a Sensing Response without including any sensing measurements. The gNB/RP then periodically initiates the Sensing Report in step 6, at the requested reporting periodicity.

6.	If requested in Step 1, the gNB/RP periodically sends sensing report to the SF with the sensing measurements.

7.	At any time after step 5, the SF may send a Sensing Update message to the gNB/RP providing updated information required for the RP to perform the sensing measurements.

Editor's Note: The need and details for this procedure depends on further RAN1 progress.

8.	If the previously requested sensing measurement information can no longer be reported, the gNB/RP notifies the SF by sending a Sensing failure message.

9.	When the SF wants to abort an ongoing sensing measurement it sends a Sensing Abort message to the gNB/TRP.