3GPP TSG RAN WG3 #130		  R3-258184

Dallas, USA, 17 - 21 November, 2025

Title:	Discussion on RAN-CN procedures and signaling for ISAC

Agenda item:	13.3

Source:	Google

Document for:	Discussion and Decision

# Introduction

The objectives for ISAC in the agreed SID [1] are shown below:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In RAN3#129bis meeting, a basic message flow for sensing reporting was agreed and other functionalities for RAN-CN interface are FFS:

To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.

In this contribution, we discuss more necessary RAN-CN interface functions and potential parameters for the agreed sensing reporting procedure.

# Discussion

In RAN3#129bis meeting, the following functionalities for RAN-CN interface were discussed:

• Start Sensing

• Modify Sensing

• SF-initiated Stop Sensing

• gNB-initiated Stop Sensing

• Sensing Data Report

For the functions of Start Sensing and Sensing Data Report, both were agreed and captured in [2]. The remaining should also be considered for RAN-CN procedures and signaling to support the complete LCM for sensing.

Additionally, the gNB sensing capability report function should also be included for appropriate sensing entity selection.

Proposal 1:  The following functions are considered for RAN-CN procedures and signaling:

- gNB Sensing Capability Report

- Start Sensing

- Modify Sensing

- Stop Sensing

- Sensing Data Report

## gNB Sensing Capability Report

Currently, the definitive protocol stacks between the gNB and SF remains undecided, e.g., reusing existing NGAP interface versus defining a new, direct interface. Regardless of the eventual protocol stack, a mandatory prerequisite for initiating any sensing start procedure is that the SF is aware of the sensing capability of a target gNB. With this in mind, the SF can select appropriated gNBs for sensing operation(s).

Therefore, the gNB may firstly register its sensing capability to the SF. The sensing capability information may include:

- supported sensing mode: e.g., gNB-based mono-static only in R20

- supported sensing service area: cell information where the gNB can perform sensing. Additionally, the location and the geographical of the cells in the gNB are related with the network deployment and can be transmitted to the SF by network implementation

- supported sensing object type: detection of a target object, or tracking of a moving target object

- supported sensing resource(s): details on the resources for sensing, e.g., specific spectrum, uplink or downlink frequency information

Proposal 2: For gNB Sensing Capability Report function, the gNB reports to the SF its supported sensing mode, sensing service area, sensing object type and sensing resource information.

## Start Sensing

As described in [2], the SF initiates a Start Sensing function by sending a Sending Request message.

Figure 8.x-1: Message flow for sensing reporting

To enable the sensing operation, the SF needs to provide the necessary sensing measurement requirement parameters. One critical parameter is the sensing area information. This information can be specified as a list of cells/TACs, or a defined physical area. Utilizing this information, the gNB can then configure the appropriate beam(s) or TRPs for the sensing task.

Furthermore, the SF needs to provide sensing measurement and reporting related parameters. These parameters may include:

- sensing service type (e.g., environment sensing, detection or tracking of a target object),

- sensing reporting configuration (e.g., periodic reporting, event-triggered reporting).

Additionally, if the SF has prior context information about the sensing target object, it can provide sensing assistance information. The sensing assistance information may also provide to assist the sensing of target object(s).

Proposal 3: For Start Sensing function, the Sensing Request message includes sensing area information, sensing measurement and reporting related parameters and sensing assistance information of target object(s) if available.

Upon receiving the sensing parameters in the Sensing Request message, the gNB evaluates the request to determine whether to accept or reject the requested sensing procedure. The gNB can respond with a Sensing Response message. This response can either be a Sensing Request Response (for successful case) or a Sensing Failure (for failure case) message. In the event of a failure, the gNB may include a failure reason to prevent the SF from repeatedly sending the same invalid sensing request.

Proposal 4: For Start Sensing function, the Sensing Response procedure contains a Sensing Request Response message, or a Sensing Failure message with a failure cause.

## Modify Sensing

When the SF need to update the parameters of an ongoing sensing operation at the gNB, the SF can initiate a sensing modification procedure.

Theoretically, the sensing modification procedure should only be initiated by the SF to modify the previous sensing parameters. The sensing modification procedure may consist of a Sensing Modification Request message and a corresponding Sensing Modification Response message.

The parameters included in the Sensing Modification Request message should be similar to those used in the initial Sensing Request message, allowing the SF to update any of these fields. The Sensing Modification Response message may contain a Sensing Modification Response (for successful case) or Sensing Modification Failure message, with similar design as the start sensing procedure.

Proposal 5: For Modify Sensing function, SF-initiated Modify Sensing is considered, comprising Sensing Modification Request, and Sensing Modification Response or Failure messages.

## Stop Sensing

The Stop Sensing function is to stop one or more existing sensing operation(s) at the gNB. The SF can initiate a sensing stop procedure, for example, by sending a Sensing Release message. This message is intended to release one or more sensing measurement tasks established by the original Sensing Request. The SF-initiated sensing stop procedure should be classified as a class-2 procedure (i.e., without response message from the gNB).

Another aspect to be consider is whether the gNB can initiate a sensing stop procedure to stop existing sensing operation(s) at the gNB. In some cases, the gNB may no longer have available sensing resource(s) to continue the sensing operation. In other cases, the gNB may detect that the target object is no longer within its sensing coverage, e.g., no reflected sensing signal is detected, or the sensing measurement quality significantly degrades.

In these scenarios, it is expected that the gNB is allowed to initiate a Sensing Stop procedure. For example, the gNB may send a Sensing Stop Required message to the SF to stop the existing sensing operation(s). The gNB may include a sensing stop cause to inform the SF of the stop reason. The SF may then confirm the sensing stop request form the gNB.

The specific conditions that trigger a gNB-initiated Stop Sensing procedure could optionally be configured by the SF, e.g., during the initial Start Sensing procedure.

Proposal 6: The Stop Sensing function can include:

- SF-initiated Stop Sensing: class-2 procedure  (e.g., Sensing Stop/Release message)

- gNB-initiated Stop Sensing: classe-1 procedure (e.g., Sensing Stop Required and Sensing Stop Confirm messages)

## Sensing Data Report

When the gNB accepts a Sensing Request or Sensing Modification Request from the SF, the gNB initiates sensing operation(s) based on the received sensing parameters. The gNB will generate raw sensing measurement result(s) for these ongoing sensing operations.  As agreed in last RAN3#129bis meeting, the gNB sends a Sensing Report to the SF.

In RAN1, sensing measurements from level 1~6 are being studied. With the increasing of the level, the gNB need to process the sensing measurements more, and consequently, the data volume decreases. For example, the Level 1 sensing measurements refer to raw sensing measurement results. The volume of level 1 sensing measurements are substantial and one example of the overhead may be ~ 2.7Gbits. For the level 6 sensing measurements, the final sensing results of a detected object with a target ID can be reported and the overhead may be ~2.4kbits (for 15 objects). It is not decided which level(s) of sensing measurements can be reported as the content in the Sensing Report. Furthermore, it is worth noting that different contents corresponds to different volume. The volume of Sensing Report should be taken into consideration when designing solutions for RAN-CN data transfer.

Additionally, in SA2 [3], key issue #4 addresses sensing data transfer and associated information from gNB to SF. The primary architectural approaches are summarized below:

- Direct interface/data tunnel bypassing AMF: there may be a direct transport channel between gNB and SF. This can typically be implemented in the user plane (e.g., solutions #1, 21, 22, 24, 25). The solution(s) will be significantly efficient for bulk sensing data reporting.

- Control Plane forwarding (AMF-inclusive): this solution utilizes exiting CN signaling path, possibly suitable for smaller, non-critical sensing data reporting (e.g., solutions #5, 7).

From RAN3’s perspective, the choice of reporting solution need to account for sensing data size, latency requirement, ISAC architecture or other aspects.  As discussed in the above sections, there are relevant discussions in RAN1 and SA2. RAN3 can wait for the inputs of RAN1 and SA2.

Taking into consideration of the above analysis, it is suggested that:

Proposal 7: For Sensing Data Report function, RAN3 wait for the inputs from RAN1 and SA2 for the content and the choice of CP/UP delivery of Sensing Report.

# Conclusion

In this contribution, we discussed the functions for RAN-CN procedures and signaling for ISAC. The related proposals are summarized below:

Proposal 1:  The following functions are considered for RAN-CN procedures and signaling:

- gNB Sensing Capability Report

- Start Sensing

- Modify Sensing

- Stop Sensing

- Sensing Data Report

Proposal 2: For gNB Sensing Capability Report function, the gNB reports to the SF its supported sensing mode, sensing service area, sensing object type and sensing resource information.

Proposal 3: For Start Sensing function, the Sensing Request message includes sensing area information, sensing measurement and reporting related parameters and sensing assistance information of target object(s) if available.

Proposal 4: For Start Sensing function, the Sensing Response procedure contains a Sensing Request Response message, or a Sensing Failure message with a failure cause.

Proposal 5: For Modify Sensing function, SF-initiated Modify Sensing is considered, comprising Sensing Modification Request, and Sensing Modification Response or Failure messages.

Proposal 6: The Stop Sensing function can include:

- SF-initiated Stop Sensing: class-2 procedure  (e.g., Sensing Stop/Release message)

- gNB-initiated Stop Sensing: classe-1 procedure (e.g., Sensing Stop Required and Sensing Stop Confirm messages)

Proposal 7: For Sensing Data Report function, RAN3 wait for the inputs from RAN1 and SA2 for the content and the choice of CP/UP delivery of Sensing Report.

# Reference

- RP-252819, "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

- R3-257326, Discussion on Sensing procedures and singalling

- TR 23.700-14