3GPP TSG-RAN3 Meeting #130	R3-258192

Dallas, US, Nov 17 - 21, 2025

Agenda Item:	12.2.3

Source: 		Qualcomm Incorporated, CATT, CMCC, NEC, NTT Docomo, Vodafone

Title: 	Discussion on Inter-CU LTM

Document for:	Discussion/Decision

# Introduction

TSG RAN#109 approved SID on NG-RAN AI/ML use cases in RP-252867 [1]. The SID contains the following objectives for AI/ML use case study.

RP-252867 The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

In the last meeting RAN3#129bis, companies disputed that Inter-CU LTM is not in the scope of the Agenda 12.2.3 Handover Enhancements. In this paper, we justify the need to study Inter-CU LTM in Rel20.

# Discussion

In the RAN plenary RAN#109, it was agreed to study Inter-CU LTM and that’s the reason it was added to the scope of the WI.  Infact during the RAN#109 online comeback session, some companies attempted to remove Inter-CU LTM objective from study but it was agreed not to remove.

However, some companies in RAN3#129bis argued that Inter-CU LTM is not in the scope of the SI. This is not the understanding of majority companies in RAN3. Majority companies think that Inter-CU LTM is in the scope of study for NG-RAN AI/ML Phase 3 in Handover Enhancements. Based on Tdocs submitted in RAN3#129 bis 6 companies (QC, SS, NEC, CMCC, CATT, E///) supports to study Inter-CU LTM. In the online session along with the 6 infra vendors, operators - Vodafone and Docomo also supported studying Inter-CU LTM.

- AI/ML based optimization of Inter-CU LTM was agreed to be studied in Handover Enhancements in RAN#109 and hence added to the SI scope of NG-RAN AI/ML Phase 3. Majority companies and Operators support studying Inter-CU LTM in Rel20 in RAN3#129bis.

Specifying AI/ML based optimization only for Intra-CU LTM will not help commercial deployments of the feature. AI/ML based Inter-CU LTM optimization also needs to be specified for any useful commercial deployments in operator networks, otherwise the feature is not complete.

- Useful commercial deployment of AI/ML optimization of LTM feature will require both AI/ML based Inter-CU LTM and Intra-CU LTM.

Additionally, the procedure for Intra-CU LTM and Inter-CU LTM is almost the same. Intra-CU LTM has signaling procedures over F1 interface whereas Inter-CU LTM has the same signaling procedures over Xn. Normative specification of both Intra-CU LTM and inter-CU LTM is already completed in Release 18 and 19 respectively.

- Intra-CU LTM and Inter-CU LTM have similar procedures for LTM Handover preparation and execution over F1 and Xn interfaces respectively.

The AI/ML optimizations studied for Intra-CU LTM over F1 interface can easily be applied to Inter-CU LTM over Xn Interface, as the LTM call flow procedures are identical between both the flavours. This will reduce additional effort to study a new feature for AI/ML based optimization and also will help in deeper analysis of overall LTM optimization.

- Most of the AI/ML based optimization discussed and studied for Intra-CU LTM over F1 interface will be applicable for Inter-CU LTM over Xn interface and only delta changes needed to study AI/ML based Inter-CU LTM.

- The purpose for Inter-CU LTM in AI/ML optimization is different with Inter-CU LTM in SON/MDT, while AI is more focused on UE and NW performance improvement, SON/MDT concern more about HO success.

Hence we propose RAN3 to study AI/ML based Inter-CU LTM in Handover enhancements and enhancements studied for AI/ML based Intra-CU LTM can be extended to Inter-CU LTM as well.

- RAN3 to study AI/ML based optimization for Inter-CU LTM in Handover Enhancements for NG-RAN AI/ML Phase 3.

The topics agreed to be studied for Intra-CU LTM can be propagated to Inter-CU LTM as well for study. Additional Xn impacts for the topics agreed in Intra-CU LTM can be studied in Inter-CU LTM.

Similar to Release 19 AI/ML use cases, the Release 20 Inter CU LTM should also consider both split and non-split architecture for study. Agreement of AI/ML framework in AI/ML based Intra-CU LTM can be adopted in AI/ML based Inter-CU LTM.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	AI/ML based optimization of Inter-CU LTM was agreed to be studied in Handover Enhancements in RAN#109 and hence added to the SI scope of NG-RAN AI/ML Phase 3. Majority companies and Operators support studying Inter-CU LTM in Rel20 in RAN3#129bis.

Observation 2.	Useful commercial deployment of AI/ML optimization of LTM feature will require both AI/ML based Inter-CU LTM and Intra-CU LTM.

Observation 3.	Intra-CU LTM and Inter-CU LTM have similar procedures for LTM Handover preparation and execution over F1 and Xn interfaces respectively.

Observation 4.	Most of the AI/ML based optimization discussed and studied for Intra-CU LTM over F1 interface will be applicable for Inter-CU LTM over Xn interface and only delta changes needed to study AI/ML based Inter-CU LTM.

Observation 5.	The purpose for Inter-CU LTM in AI/ML optimization is different with Inter-CU LTM in SON/MDT, while AI is more focused on UE and NW performance improvement, SON/MDT concern more about HO success.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to study AI/ML based optimization for Inter-CU LTM in Handover Enhancements for NG-RAN AI/ML Phase 3.

4 References

[1] RP-252867, Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3