3GPP TSG-RAN WG3 Meeting #130	R3-258198

Dallas, US, 17 – 21 November, 2025

Agenda item:	11.2.1

Source:	Nokia

Title:	MRO Enhancements for Inter-CU LTM

Document for:	Approval

# 1	Introduction

This is the contribution related to MRO enhancements for Inter CU-LTM in Rel.20.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 # 129bis

The agreements achieved in RAN3 #129bis [1]:

Use cases:

- At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.: BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA; LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

- Near failure case: SHR successful HO

- Successful case: ping/pong

Solution options:

- Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

- For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line.

- For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

- For the connection failure cases:

  - After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

### 2.1.2 Connection failure monitoring for inter-CU LTM

As part of the NR_SON_MDT_Ph4-Core WI in Rel. 19 MRO for Rel. 18 LTM was provided. The focus was related to enhancement of the RLF report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 2 “RACH less LTM cell switch failure due to wrong beam” and case 3 “LTM failure due to outdated TA”. With the introduction of the inter-CU LTM the source DU and target DU may be in the different gNBs. Thus, the proposed signaling between the target and source DU for case 2 and 3 will not work for inter-CU LTM.

During RAN3 # 129bis the following was agreed:

Use cases:

- At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, failure due to wrong beam and outdated TA; LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

Addressing the inter-CU LTM cell switch failure scenario (case 2), which stems from improper beam selection, necessitates an Xn message enhancement. This is crucial for enabling the target DU to communicate re-established or recovery beam information to the source DU, particularly when the UE successfully performs RACH-based access during re-establishment or recovery to the same target cell. Such an enhancement would facilitate the adoption of the Release 19 solution previously established for intra-CU LTM.

Proposal 1: RAN3 agrees the target CU sends the re-established or recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Addressing inter-CU LTM cell switch failures attributable to an outdated TA necessitates an Xn message protocol enhancement. This modification is crucial for the target DU to convey the TA value utilized during the RACH procedure to the source DU, particularly following a successful RACH-based access by the UE during re-establishment or recovery to the same target cell, which can indicate an outdated TA condition. The objective is to apply the established Release 19 solution from intra-CU LTM contexts.

Proposal 2: RAN3 agrees the target CU sends the TA information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

With a NW triggered UL synchronization, the UE performs early TA acquisition. However, for inter CU LTM new message TA Information is transferred from candidate gNB to source gNB which introduces additional delay compared to Rel. 18 LTM. The most critical aspect of the LTM is that the UE has got the correct TA for the moment of the MAC-CE triggered cell switch, i.e. the right timing of the early TA Acquisition (TAA) in relation to the cell switch command (CSC). In case the TAA is not completed before the CSC, the UE will be forced to do normal RACH towards the candidate cell, leading to delays in RACH succeeding and losing the advantage of the LTM mobility procedure. Similarly, the chances of RACH failing also increase. So, it is critical to have the correct value of the TA at the right time at the UE for the overall LTM mobility procedure to succeed.

Observation 1: An extra delay caused by transferring the TA value from candidate to source gNB may have an impact on triggering the Early TA acquisition procedure for inter-CU LTM.

Proposal 3: RAN3 agrees to analyse optimization of triggering the Early TA acquisition procedure for inter-CU LTM.

During RAN3 # 129bis the following was agreed:

Solution options:

- For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line.

For Rel. 18 LTM it was agreed in Rel. 19 to re-use the MRO connection failures defined for standard HO and CHO via adding appropriate text into TS 38.300, chapter 15.5.2.2.2 which seems to be valid also for inter- CU LTM.

Proposal 4: RAN3 agrees to re-use the MRO connection failures defined for Rel. 18 LTM also for inter-CU LTM with no enhancement needed in TS 38.300.

During Rel. 19 mobility discussions, security vulnerabilities were identified concerning the clear-text transmission of the Next Hop Chaining Count (NCC) within the Cell Switch Command (CSC) for inter-CU LTM. Two primary issues were highlighted:

- Issue 1: Corrupted NCC from a Malicious gNB

  - A malicious gNB sends a CSC containing a corrupted NCC to the UE.

  - The UE generates security keys based on this corrupted NCC.

  - Upon sending the RRCReconfigurationComplete message to the target gNB, a security key mismatch occurs, leading to an integrity check failure.

  - The target gNB ceases further communication, resulting in T304 expiry and a Radio Link Failure (RLF) on the UE side.

- Issue 2: Corrupted RRC Configuration Part from a Malicious gNB

  - A malicious gNB sends a CSC containing a corrupted RRC configuration part (e.g., an invalid 'RRC configuration ID') which prevents the successful generation of the RRCReconfigurationComplete message.

  - T304 expires which leads to an RLF on the UE side.

In both scenarios, while the UE generates an RLF report, it currently lacks adequate information to properly inform the network about the underlying security nature of the failure.

SA3 clearly indicated that there are no plans to discuss the security failures related to MAC transmission of NCC in Rel. 19. RAN2 then concluded that it shall be considered as part of SON/MDT features for inter-CU LTM.

Proposal 5: RAN3 agrees to analyse a UE-based solution for MRO to address inter-CU LTM connection failures caused by security issues.

### 2.1.4 Near failure monitoring for inter-CU LTM

As part of the NR_SON_MDT_Ph4-Core WI in Rel. 19 MRO for Rel. 18 LTM was provided. The focus was related to enhancement of the SHR report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 1 “BFR shortly after successful LTM cell switch to the wrong beam”. With the introduction of the inter-CU LTM the source DU and target DU may be in different gNBs. Thus, the proposed signaling between the target and source DU will not work for case 1 for inter-CU LTM.

During the RAN3 # 129bis meeting the following was agreed:

Use cases:

- At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.: BFR shortly after successful LTM cell switch;

For MRO in intra-CU LTM (Case 1), the target DU conducts initial analysis and transmits recovery beam information to the source DU via the CU using F1AP. In contrast, for the inter-CU LTM scenario, transferring recovery beam information to the source DU necessitates an XnAP message enhancement. This enhancement would facilitate the signalling of recovery beam information from the target CU to the source CU, and an existing message, such as the ACCESS AND MOBILITY INDICATION message, could be reused for this purpose.

Proposal 6: RAN3 agrees the target CU sends the recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

At the end of the Rel. 19 NR_SON_MDT_Ph4-Core WI some of the companies proposed to focus on monitoring of the almost outdated TA value. Even majority of the companies acknowledged the issue there was no consensus on definition of the almost outdated TA value.

Proposal 7: RAN3 agrees to define the almost outdated TA use case for inter-CU LTM and analyse possible solution options.

# 3	Conclusions

We have the following observations and proposals:

Proposal 1: RAN3 agrees the target CU sends the re-established or recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Proposal 2: RAN3 agrees the target CU sends the TA information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Observation 1: An extra delay caused by transferring the TA value from candidate to source gNB may have an impact on triggering the Early TA acquisition procedure for inter-CU LTM.

Proposal 3: RAN3 agrees to analyse optimization of triggering the Early TA acquisition procedure for inter-CU LTM.

Proposal 4: RAN3 agrees to re-use the MRO connection failures defined for Rel. 18 LTM also for inter-CU LTM with no enhancement needed in TS 38.300.

Proposal 5: RAN3 agrees to analyse the need for a UE-based solution for MRO to address inter-CU LTM connection failures caused by security issues.

Proposal 6: RAN3 agrees the target CU sends the recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Proposal 7: RAN3 agrees to define the almost outdated TA use case and possible solution for inter-CU LTM.

# References

- RAN3 #129bis Meeting report