3GPP TSG-RAN WG3 Meeting #130	R3-258229

USA Dallas 17th-21st November 2025

Source: 	Vodafone, Deutsche Telekom

Title: 		SBI and Point2Point Definitions

Document for: decision

Agenda Item: 10.3.2

Introduction

During RAN Plenary 108, the 6G RAN Study was agreed in RP-251881 containing the milestone for RAN 3 in June 2026:

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

The aim of this paper is to clarify the definitions of the RAN-CN Point-to-Point Interface and the Service-Based Interface (SBI).

Discussion

Even though SBA (Service Based Architecture) is used in the core network since Release 15, there does not seem to be a clear/sufficient 3GPP definition of SBI (service-based Interface).

E.g. in 29.500, you can find the following:

4.2 Service Based Interfaces

A service-based interface represents how the set of services is provided or exposed by a given NF. This is the interface where the NF service operations are invoked.

Additionally, the terminology P2P is not self-explanatory because AMF/UPF can have multiple IP addresses and even be geographically distributed.

As RAN3 is tasked to compare Point2Point vs SBI on NG, definitions of SBI and Point2Point are needed.

It must be pointed out that the definitions are focused on control plane. In this contribution, the control plane is limited to legacy 5G services only. Emerging services such as AI/ML and ISAC-related data are excluded from these definitions, and the design of their associated data transfer may follow different architectural principles."User Plane protocol stack is always working as point-to-point interface.

The following is proposed:

NG SBI (Service-Based Interface) is an interface that enables communication between the RAN logical node and the Core Network control plane logical node through standardized Application Programming Interfaces (APIs) by using Network Repository Function (NRF) for example, for registration, lifecycle management, and endpoint discovery, enabling secure and multi-vendor interoperable service exchange allowing flexible deployment scenarios including virtualized and cloud-native environments.

Note 1: Within this framework, RAN logical node is modeled as services (e.g., connection management, radio resource management, slice management, security, etc), where each of these services could have their own registration, lifecycle management, etc

Note 2: The protocol stack for API-based communication is defined by 3GPP.

The NG Point-to-Point Interface is a logical, standardized interface between a RAN logical node and a Core Network control plane logical node using protocol stack as defined by 3GPP. It enables the exchange of control signaling between RAN and CN in multi-vendor interoperable and secure way. Although the interface is logically point-to-point, it does not require a direct physical connection, allowing flexible deployment scenarios including virtualized and cloud-native environments.

Proposal:

It is proposed to discuss and agree on the definitions above and capture them in corresponding section of TR.

# Annex

## pCR to TR 38.760

### 6.1.3	RAN-CN Interface Options

For NG Control plane:

NG SBI (Service-Based Interface) is an interface that enables communication between the RAN logical node and the Core Network control plane logical node through standardized Application Programming Interfaces (APIs) by using Network Repository Function (NRF) for example, for registration, lifecycle management, and endpoint discovery, enabling secure and multi-vendor interoperable service exchange allowing flexible deployment scenarios including virtualized and cloud-native environments.

Note 1: Within this framework, RAN logical node is modeled as services (e.g., connection management, radio resource management, slice management, security, etc), where each of these services could have their own registration, lifecycle management, etc

Note 2: The protocol stack for API-based communication is defined by 3GPP.

The NG Point-to-Point Interface is a logical, standardized interface between a RAN logical node and a Core Network control plane logical node using protocol stack as defined by 3GPP. It enables the exchange of control signaling between RAN and CN in multi-vendor interoperable and secure way. Although the interface is logically point-to-point, it does not require a direct physical connection, allowing flexible deployment scenarios including virtualized and cloud-native environments.

Note 1: These definitions are specifically tailored to NR legacy services, excluding considerations for emerging applications like AI/ML and ISAC)

For NG User plane:

NG User Plane protocol stack is point-to-point interface between RAN and User Plane logical function in CN.