3GPP TSG-RAN WG3 Meeting #130						R3-258241

Dallas, USA, 17 – 21 Nov. 2025

Agenda item:		9.2.7

Title:		Discussion on A-IoT device identifier privacy protection impact on RAN3

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The SA3 has some progress on the A-IoT device identifier privacy, this contribution will discuss the impacts on RAN3.

# 2. Discussion

In clause 5.4.2 of TS 33.369, there is the following statement on the AIoT device identifier protection for inventory with filtering information that:

### 5.4.2 The AIoT device identifier protection for inventory with filtering information For the protection of AIoT device permanent ID during the inventory procedure described in clause 5.2.2, the following change shall apply: - In step 4, the AIoT device determines it needs to reply to the NG-RAN based on the received filtering information. NOTE 1: The attacker may obtain a AIoT device ID by performing a bitwise enumeration in multiple paging messages. To mitigate the attack, the AIoT device need to be configured with filtering information to match by limiting which bits of AIoT device identifier is allowed for filtering information (guidance would be to limit to the leftmost n bits of the permanent device identifier, e.g., only allow filtering information for the leftmost 64 bits and not respond otherwise). - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, filtering information is used as a device identification information if the AIOTF received it in step 0. NOTE 2: The AIOTF identifies the AIoT device by checking the received RESAIoT. Therefore, device identification information is not needed in the D2R and Inventory Report message.

In the inventory procedure with group devices, if the device ID privacy protection is active, the A-IoT device should not report its permanent ID to the RAN Reader, and the RAN Reader should also not include the device's permanent IDs in the inventory report.

Observation 1: For a group of device inventory, if the device ID privacy protection is active, the A-IoT device should not report its permanent ID to the RAN Reader, and the RAN Reader should also not include the device's permanent IDs in the inventory report.

In addition, in clause 5.4.3 of TS 33.369, there is the following statement on the A-IoT Device identifier protection with Temp ID update during Individual inventory that

### 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1. - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile.

In the inventory procedure with a single device, if the device ID privacy protection is active, a temporary device ID will be sent from AIoT CN to the AIoT device. The AIoT device will generate a temporary ID to determine whether it needs to reply to the paging message. If it needs to reply, the AIoT device will send a D2R message to the RAN reader, but not include the device's permanent ID. And the inventory report will not include the device's permanent ID.

Observation 2: In the inventory procedure with a single device, if the device ID privacy protection is active, the A-IoT device will send a D2R message to the RAN reader, but not include the device's permanent ID. And the Reader will not include the device's permanent IDs in the Inventory Report message.

The “indication of type of T-ID handling” instructs the device ID reporting and has no impact on the Reader’s behavior, and it should be included in the NAS PDU in the Inventory Request message, which is invisible to the Reader. While a device ID protection indication over NGAP is necessary and enough so that the RAN reader will not include the AIoT device data in the inventory report if the device ID protection indication is included.

Observation 3: The “indication of type of T-ID handling” has no impact on the Reader’s behavior, and it should be included in the NAS PDU in the Inventory Request message, which is invisible to the Reader.

Overall, from the RAN3 point of view, the indication about whether the privacy protection is active or not should be included in the INVENTORY REQUEST message from the A-IoT CN to the RAN Reader. And the AIoT NAS PUD (i.e., the permanent A-IoT device ID) in the inventory report is not mandatory.

Regarding this issue, we provide CR R3-258242 for TS 38.413 accordingly.

Proposal 1: Introduce an Identifier Protection in the Inventory Request message. If the Identifier Protection is included in the Inventory Request message, the Reader will not include the device's permanent IDs in the Inventory Report message.

Proposal 2: Discuss and agree on the corresponding CR in R3-258242.

# 3. Conclusion

Observation 1: For a group of device inventory, if the device ID privacy protection is active, the A-IoT device should not report its permanent ID to the RAN Reader, and the RAN Reader should also not include the device's permanent IDs in the inventory report.

Observation 2: In the inventory procedure with a single device, if the device ID privacy protection is active, the A-IoT device will send a D2R message to the RAN reader, but not include the device's permanent ID. And the Reader will not include the device's permanent IDs in the Inventory Report message.

Observation 3: The “indication of type of T-ID handling” has no impact on the Reader’s behavior, and it should be included in the NAS PDU in the Inventory Request message, which is invisible to the Reader.

Proposal 1: Introduce an Identifier Protection in the Inventory Request message. If the Identifier Protection is included in the Inventory Request message, the Reader will not include the device's permanent IDs in the Inventory Report message.

Proposal 2: Discuss and agree on the corresponding CR in R3-258242.

# References