3GPP TSG-RAN WG3 Meeting #130	R3-258248

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		12.2.2

Source:		NEC

Title:			(pCR for TR38.745) AI/ML based intra-CU LTM

Document for:		Discussion and decision

# Introduction

In RAN3#129bis meeting, the following agreements are achieved for AI/ML based intra-CU LTM [1]:

Prioritize normal intra-CU LTM scenarios.

For CU-DU split architecture, the following two scenarios are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

AI/ML assisted candidate cell selection for LTM Handover Preparation

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Besides, a pCR to TR38.745 [2] is agreed to capture Use case description and Locations for AI/ML Model Training and AI/ML Model Inference.

In this contribution, we provide our further considerations on AI/ML based intra-CU LTM and TP to TR38.745.

# Discussion

  - LTM prediction

L1/L2 Triggered Mobility (LTM) was introduced in Rel-18 to reduce overhead and interruption time through enabling mobility using lower-layer signaling. The basic procedures of intra-CU LTM are shown as figure 1 below.

Figure 1: Intra-CU LTM

In order to further improve reliability, reduce latency and optimize resource allocation of legacy intra-CU LTM mechanism, AI/ML based intra-CU LTM can be adopted to address potential mobility issues through predictive and data-driven approaches.

Below is an example procedure for AI/ML based intra-CU LTM when AI/ML Model Inference located in gNB-CU:

Figure 2: AI/ML based intra-CU LTM with AI/ML Model Inference located in gNB-CU

Step 0. AI/ML Model Training function locates, collects the input data for training. For intra-CU LTM, the input data for training includes L1 and L3 measurements, UE location information, UE mobility history information, resource status (including cell level and beam level). After (re-)training, the AI/ML model is deployed in the gNB-CU or gNB-DU.

Step 1. gNB-CU configures UE with radio measurements information.

Step 2. UE collects the indicated measurement, e.g., L3 measurements of serving cell and neighbouring cells and sends measurement report message, including the required measurement, to gNB-CU.

Step 3. Model Inference. Required measurements are leveraged into Model Inference to output the LTM prediction, including e.g., predicated LTM candidate cells.

Step 4. gNB-CU sends the LTM prediction to the serving gNB-DU for LTM configuration, and gNB-CU configures the UE with LTM configuration of the predicted candidate cells.

Step 5. UE performs early synchronization with the predicted LTM candidate cells in both UL and DL. UE collects the required L1 measurements of serving cell and neighbouring cells, and sends them to the serving gNB-DU. The serving gNB-DU can send the received L1 measurements to gNB-CU for LTM prediction.

Step 6. The serving gNB-DU makes intra-CU LTM decision and triggers intra-CU LTM cell switch towards the UE.

Step 7: UE executes intra-CU LTM cell switch towards the LTM target cell.

In step 3, gNB-CU can generate predicted LTM candidate cells as output as we already agreed, besides, predicted LTM configuration for each candidate cells can also be generated.

In step 5, the serving gNB-DU can send the received L1 measurements to gNB-CU for LTM prediction.

Proposal 1: The serving gNB-DU sends the received L1 measurements to gNB-CU for LTM prediction.

In legacy LTM, UE can perform early synchronization including early timing advance (TA) acquisition with candidate target cell(s) before receiving the LTM cell switch command. This procedure involves multiple signalling exchanging among several nodes (source gNB-DU, gNB-CU, candidate gNB-DU). Besides, from UE side, a gap is needed to perform the early synchronization. In order to reduce signaling cost and simplify UE behavior for early TA acquisition, gNB-CU can use AI/ML to generate predicted TA value for each LTM candidate cell(s).

Proposal 2: gNB-CU can generate predicted TA value for each LTM candidate cell(s).

2.2	Location for AI/ML Model

For LTM use case, L1 measurements are collected by gNB-DU and it is gNB-DU to make LTM decision and trigger LTM cell switch towards the UE. If AI/ML Model Inference located in gNB-DU, instead of transferring large amount of L1 measurements to gNB-CU, gNB-DU can perform model inference to directly generate one predicted LTM target cell, based on the required input data, e.g. L1 measurements. Therefore, for this use case, AI/ML Model Inference located in gNB-DU should also be considered.

Proposal 3: Study AI/ML based intra-CU LTM with AI/ML Model Inference located in gNB-DU.

2.3	Input, output and feedback

As analysis above, gNB may need the following information as input data for AI/ML based intra-CU LTM:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

-	Early synchronization information, including TCI state, RACH configuration.

From the gNB-DU:

-	UE L1 measurements

From the local gNB-CU:

-	Current/predicted resource status, including cell level and beam level

-	UE trajectory prediction

-	Current/predicted UE traffic

The output generated by AI/ML model inference is the LTM prediction, including predicated LTM candidate cells, predicted LTM configuration for each candidate cells, predicted TA value, predicated LTM target cell.

Legacy UE performance feedback, beam level UE performance feedback for those UEs that performed intra-CU LTM cell switch and SON reports (e.g., RLF, CEF, RA) can also be used as feedback data.

2.4	Potential standard impacts

According to the analysis above, following standard impacts are foreseen for subsequent Rel-20 normative work:

F1 interface:

-	Enhanced existing procedure or defined new procedure to collect information between gNB-CU and gNB-DU:

-	LTM prediction

-	UE L1 measurements

TP to TR38.745 which captures the above information is in Annex.

Proposal 4: Agree the TP to TR38.745 for AI/ML based intra-CU LTM in Annex.

# Proposals

In this contribution, we provide our initial considerations on AI/ML based intra-CU LTM, and the following proposals were drawn from the analysis.

Proposal 1: The serving gNB-DU sends the received L1 measurements to gNB-CU for LTM prediction.

Proposal 2: gNB-CU can generate predicted TA value for each LTM candidate cell(s).

Proposal 3: Study AI/ML based intra-CU LTM with AI/ML Model Inference located in gNB-DU.

Proposal 4: Agree the TP to TR38.745 for AI/ML based intra-CU LTM in Annex.

# Reference

[1] RAN3_129bis_agenda_20251017_EOM2

[2] R3-257301, (TP to TR 38.745) AIML based Intra-CU LTM

# Annex	TP for TR38.745

<Start of change>

## 4.2	AI/ML based Intra-CU LTM

### 4.2.1	Use case description

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [x].

Intra-CU LTM is specified in TS38.401[x].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

### 4.2.2	Solutions and Standard impacts

#### 4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Besides, AI/ML Model Inference located in gNB-DU is also be considered for AI/ML based intra-CU LTM.

#### 4.2.2.2		Input data of AI/ML based Intra-CU LTM

Editor’s Note: To be updated

A gNB may need the following information as input data for AI/ML based intra-CU LTM:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

-	Early synchronization information, including TCI state, RACH configuration.

From the gNB-DU:

-	UE L1 measurements

From the local gNB-CU:

-	Current/predicted resource status, including cell level and beam level

-	UE trajectory prediction

#### 4.2.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell and beam for LTM HO Preparation

- Predicated LTM candidate cells

- Predicted LTM configuration for each candidate cells

- Predicted TA value

- Predicated LTM target cell.

#### 4.2.2.4	Feedback of AI/ML based intra-CU LTM

To optimize the performance of AI/ML based intra-CU LTM model, following feedback can be considered to be collected:

- Legacy UE performance feedback

- Beam level UE performance feedback

- SON reports (e.g., RLF, CEF, RA)

#### 4.2.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work compared with what was specified during Rel-18 and Rel-19:

Potential F1 interface impact:

- Enhanced existing procedure or new procedure to exchange information between gNB-CU and gNB-DU:

  - LTM prediction

  - UE L1 measurements

<End of change>