3GPP TSG-RAN WG3 Meeting #130                                                        R3-258257

Dallas, Texas, USA, 17-21 November 2025

Title:	Discussion on RAN-CN procedures and signaling for ISAC

Agenda item:	13.3

Source:	KPN N.V., TNO

Document for:	Agreement

- Background

In RAN #108, the SID on the Study on Integrated Sensing and Communication (ISAC) for NR was agreed [1].

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing request to the gNB.

2.	The gNB sends sensing response to the SF.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

In RAN3 #129-bis, the following call-flow was agreed [2].

In this contribution, we would like to present our views on RAN-CN procedures and signaling for ISAC.

- Discussion

## 2.1 General

A sensing service request is associated with a desired QoS, such as probability of detection or false alarm rate. The SF sends this request to gNB to perform sensing. However, due to the location or capability of the gNB, it may not be able to provide sensing results with sufficient quality.

Figure 1: A sensing target being sensed by multiple gNBs

As shown in Figure 1, a single gNB is unable to provide sensing results with sufficient sensing quality, thus, multiple gNBs perform sensing to improve the sensing performance.

Observation 1: A target may not be sensed with sufficient quality if it is sensed by a single gNB. This may especially occur when the target is located at the cell edge.

Proposal 1: An SF should be able to select multiple gNBs to perform sensing for the same sensing service request.

Current study item considers UAV as a sensing target. Consider Figure 2. Initially gNBa is performing the sensing task to detect/track the UAV. However, as it moves out of its coverage, there would be loss in sensing quality. In this case, it is crucial to maintain service continuity by performing sensing handover, that is, finding and requesting a suitable gNB, in this case gNBb, to perform sensing.

Figure 2: As UAV moves from coverage area of gNBa to that of gNBb, a sensing handover occurs

Observation 2: Sensing performance may degrade when the target moves away from the sensing entities or when the sensing entities move away from the target.

Proposal 2: A gNB may suggest to SF adding a neighboring gNB as a sensing entity to the sensing request considering the sensing quality and target trajectory.

If the gNB is no longer able to satisfy the sensing quality requirements, it may release the sensing task, for example, when the UAV moves out of the coverage of the gNB. In addition, even if a gNB seems suitable from sensing perspective, it might be heavily loaded with communication tasks at a particular moment and thus may not be able to perform the sensing task.

Observation 3: A gNB selected by SF may not be capable of performing sensing at a given moment, for example, because it is heavily loaded with communication tasks.

Proposal 3: A gNB may reject or release the sensing task if it is not able to provide good sensing performance or if it is heavily loaded with communication services.

## 2.2 Reporting results to the SF

### [FL5][H] Proposal 7.1-1-rev3

- The following measurements are identified in the study of NR ISAC.

  - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent

  - Level 2: (partial raw data)

    - Amplitude and phase profile of delay, and/or Doppler, and/or angle

      - E.g., Amplitude and phase profile of delay-angle

  - Level 3 per path (subset of Level 2):

    - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp

      - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles

        - A path can correspond to one or multiple detected points

        - A path can correspond to one or multiple detected objects

  - Level 4 per point:

    - Option 1: Range, velocity, angle, and power per detected point for a given time stamp

      - Definition: A point is defined by one range/delay, one Doppler and one angle

    - Option 2: Position, velocity, and power per detected point for a given time stamp

      - Definition: A point is defined by one location and one velocity

  - Level 5: e.g., {position, velocity, no target ID}

    - Definition: Position and velocity for a detected object/target for a given time stamp

      - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target

      - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported

  - Level 6: e.g., {position, velocity, target ID}

    - Definition: Position and velocity for a detected object/target for a given time stamp

      - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target

      - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported

  - SINR, RSRP from perspective of sensing

  - Time stamp

RAN1 is currently discussing the level of measurements that can be reported. Following a summary from RAN#122bis is provided [3].

Level 1 data comprises of raw data, that is, subcarrier-domain channel responses may result in high amount of data. Sharing this data to the SF may cause unnecessary overhead.

Observation 4: A sensing task might generate large amount of data to be shared between gNBs and SF, and this needs to be controlled.

In order to minimize the amount of data to be sent to SF, gNB should send locally processed data to the SF. However, if SF requests detailed data such as Level 1 data in order to combine the sensing results from multiple gNBs then that may be provided by the gNB.

Proposal 4: A gNB should provide intermediate/final sensing results to the SF and can provide further data to the SF, when requested to improve the sensing performance.

- Conclusion

Observation 1: A target may not be sensed with sufficient sensing quality if it is sensed by single gNB. This may especially happen when the target is located at the cell edge.

Observation 2: Sensing performance may degrade when the target moves away from the sensing entities or when the sensing entities move away from the target.

Observation 3: A gNB selected by SF may not be capable of performing sensing at a given moment, for example, because it is heavily loaded with communication tasks.

Observation 4: A sensing task might generate large amount of data to be shared between gNBs and SF, and this needs to be controlled.

Proposal 1: An SF should be able to choose multiple gNBs to perform sensing for the same sensing service request.

Proposal 2: A gNB may suggest to SF adding a neighboring gNB as a sensing entity to the sensing request considering the sensing quality and target trajectory.

Proposal 3: A gNB may reject or release the sensing task if it is not able to provide good sensing performance or if it is heavily loaded with communication services.

Proposal 4: A gNB should provide intermediate/final sensing results to the SF and can provide further data to the SF, when requested to improve the sensing performance.

- References

RP-252819	Study on Integrated Sensing and Communication (ISAC) for NR.

R3-257326 (TP to BL pCR to 38.765) for basic sensing call flow

R1-2507427 Summary #5 on evaluations for NR ISAC