3GPP TSG-RAN WG3 Meeting #130	R3-258262

Dallas, US, 17 – 21 November 2025

Agenda Item:		14.2

Source:		Ofinno

Title:	Discussion on the Issues of Topology 2 for A-IoT

Document for:		Discussion

# Introduction

In last RAN3 #129bis meeting, the following agreements were achieved for on A-IoT in NR Phase 2:

General Architecture:

For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied.

NGAP procedures specified for Topology 1 are baseline for Topology 2.

NGAP signaling procedures (Inventory and Command):

The AIOT procedures (Topology 2, RRC-based solution) in TR38.769 can be as baseline.

UE Reader Authorization:

UE Reader Authorization is performed at 5GC

The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

This contribution discusses further the potential open issues of Topology 2 for A-IoT.

# Discussion

2.1 UE reader authorization

In last meeting, the following agreement was achieved:

- UE Reader Authorization is performed at 5GC

- The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

Based on the agreement above, it’s straightforward that a new IE, shown as follows, should be defined to indicate to A-IoT enabled gNB if an UE is an authorized A-IoT UE Reader. The A-IoT enabled gNB gNB may perform UE reader selection based on the new IE and other assistant information.

#### 9.3.1.xxx	A-IoT UE Reader Authorized

This IE provides information about the authorization status of the A-IoT UE reader.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT UE Reader Authorized | M |  | ENUMERATED (authorized, not authorized, ...) | Indicates the A-IoT UE reader authorization status. |

To support this function in the A-IoT enabled gNB, the following NGAP messages should be enhanced to include the new A-IoT UE Reader Authorized IE.

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

Proposal 1: To define a new A-IoT UE Reader Authorized IE and include it in the following NGAP message:

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

In last meeting, the following issues are open to discuss:

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

On the handover case, whether UE Reader authorization status (authorized, non-authorized) should be sent from the source/old gNB to the target/new gNB.

During the online meeting, there was a comment, i.e., resource authorized in old gNB is not applicable to new gNB, which is true. As we have agreed, the UE Reader authorization status (authorized, non-authorized) is performed at 5GC. Here, the intention is to pass the UE Reader authorization status (authorized, non-authorized) to the target/new gNB so that the target/new gNB decides the resources based on that. The decision is totally done by the target/new gNB.

On the other hand, if the target/new gNB does not receive from the source/old gNB on the UE Reader authorization status, the only way is to receive it from AMF via the path switch procedure. It is late since the target/new gNB may already decide the resources or even reject the UE reader in the handover acknowledge message. Therefore, it is beneficial to allow that UE Reader authorization status (authorized, non-authorized) is sent from the source/old gNB to the target/new gNB.

Based on the analysis above, the following proposals are suggested.

Proposal 2: To allow UE Reader authorization status (authorized, non-authorized) to be sent from the source/old gNB to the target/new gNB.

Proposal 3: To include the new A-IoT UE Reader Authorized IE in the following handover related messages:

NGAP:

- HANDOVER REQUEST

XnAP:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

In case of split RAN architecture, i.e., from the gNB-CU to the gNB-DU, the gNB-DU should also know if the A-IoT enbled UE reader is authorized so that it determine and perform radio resource allocation based on the indication. If it’s not authorized, the gNB-DU may reject it during the UE context setup procedure. So it’s beneficial to allow that UE Reader authorization status is sent from the gNB-CU to the gNB-DU.

Based on the analysis above, the following proposals are suggested.

Proposal 4: To allow UE Reader authorization status (authorized, non-authorized) to be sent from the gNB-CU to the gNB-DU.

Proposal 5: To include the new A-IoT UE Reader Authorized IE in the following F1AP messages:

F1AP:

- UE CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

2.2 On how to send inventory report to A-IoTF

In Rel-19, a time interval was defined for A-IoT enabled gNB to refer to and send the inventory report to A-IoTF, as shown below, defined in TS 23.369.

- Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation.

A fixed time interval may be fine for Topology 1. A-IoT enabled gNB may send the inventory report via NG interface, which is wired interface.

However, it’s not enough for Topology 2. In Topology 2, a UE reader is involved for sending the inventory report. A short time interval may force the UE reader to send the inventory report very frequently, which may cause radio resource waste from the UE reader to A-IoT enabled gNB. Within a short time interval, the UE reader may not identify enough number of A-IoT devices. On the other hand, a long time interval may cause infrequent inventory report from UE reader to A-IoTF, which may delay the network process.

Furthermore, in order to send the inventory report by the UE reader, the UE reader needs to receive a UL grant, which may triggers a random access procedure and/or a scheduling report. Thus, the time to send the inventory report may not be aligned with the time interval defined in the core network.

Therefore, an enhancement on how to send inventory report to A-IoTF should be investigated in Rel-20. For example, sending the inventory report to A-IoTF may be based on the inventory status that the UE reader completes the inventory procedure (e.g., the number of A-IoT devices identified by UE reader, a number of times the UE reader sends the paging message/access triggers, etc).

Proposal 6: To enhance on how to send inventory report to A-IoTF, e.g., based on the inventory status that the UE reader completes the inventory procedure.

# Proposals

Proposal 1: To define a new A-IoT UE Reader Authorized IE and include it in the following NGAP message:

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

Proposal 2: To allow UE Reader authorization status (authorized, non-authorized) to be sent from the source/old gNB to the target/new gNB.

Proposal 3: To include the new A-IoT UE Reader Authorized IE in the following handover related messages:

NGAP:

- HANDOVER REQUEST

XnAP:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

Proposal 4: To allow UE Reader authorization status (authorized, non-authorized) to be sent from the gNB-CU to the gNB-DU.

Proposal 5: To include the new A-IoT UE Reader Authorized IE in the following F1AP messages:

F1AP:

- UE CONTEXT SETUP REQUEST

- UE CONTEXT MODIFICATION REQUEST

Proposal 6: To enhance on how to send inventory report to A-IoTF, e.g., based on the inventory status that the UE reader completes the inventory procedure.

# Reference

[1] RP-252894, Solutions for Ambient IoT (Internet of Things) in NR Phase 2 (WID)

[2] R3-256501, RAN3#129-bis Meeting Agenda