3GPP TSG-RAN3 Meeting #130	R3-258282

Dallas, US, 17 - 21 November, 2025

Agenda item:	10.2.2

Source:	Xiaomi

Title:	RAN3 Considerations on 6GR Data transfer

Document for:	Discussion and Decision

1	Introduction

One of the objectives of the study is to progress Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc).

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] Overall RAN architecture aspects [RAN3, RAN2]

Considering the new data transfer requirement on AI and sensing, In 6GR, as captured in [1], data transfer becomes one of the most important study components of 3GPP RAN.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

RAN2 has started the study on the data transfer and made some initial guidance for the further study. And we think the data transfer between UE and CN (via RAN), RAN and CN, RAN and RAN, RAN and OAM are related to RAN3 study as well.

- For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

In this contribution, we’d like to discuss the general considerations for RAN3 on the data transfer aspects.

2	Discussion

2.1 	RAN3 scope for data collection and transfer

Backgound information analysis

In the existing 5G system, several data collection-related functions have been introduced. The following summary outlines each function from the gNB data handling perspective, including its initiator, procedure, and consumer.

Table 1 Existing 5G Functions related to data collection and transfer

| Function | Initiator | Producer | Content | Consumer | gNB functionalities from data handling perspective |
| --- | --- | --- | --- | --- | --- |
| Trace | OAM(CN) | CN/gNB | Signalling messages, handover events, connection failures | OAM(TCE) | Trace/MDT/QoE share similar functionalities: Data collection configuration (for UE-side data) Data forwarding (for UE side data) Data generation (for gNB-side data) Data reporting (for gNB-side data) |
| MDT | OAM(CN) | gNB/UE | KPIs (latency, packet loss), Radio measurements, cell ID, timing advance, RF conditions | OAM(TCE) |  |
| QoE | OAM(CN) | UE | UE application measurements | OAM(MCE)/gNB |  |
| LCS | LMF | gNB[Note1] | Positioning measurements | LMF | Data generation Data processing Data reporting |
| SON[Note2] | gNB | Peer gNB | RAN resource status | gNB | Data collection initiation Data generation Data reporting |
| AI RAN | gNB | Peer gNB | Predicted RAN status and performance info | gNB | Data collection initiation Data generation Data reporting |
| AI Pos | gNB | LMF | Ground truth label for AI positioning | gNB | Data collection initiation |
| Note 1, When the UE acts as a data producer, configuration and reporting are handled via LPP, which remains transparent to the gNB, so UE-based/assisted positioning is not in RAN3 discussion scope. Note 2, as there are many technical directions for SON, for data collection, we focus on the resource status exchange part. |  |  |  |  |  |

The following table summarizes the new use cases and the corresponding requirements in 5G Release 20 and 6G, from the perspective of RAN node data handling.

Table 2 New use cases in 5GA and 6G related to data collection and transfer

| Use cases | Initiator | Producer | Content | Consumer | gNB functionalities |
| --- | --- | --- | --- | --- | --- |
| AI Two-sided model | OTT server | gNB | AI model parameters/data sets for AI training | OTT server | Data generation Data processing Data reporting |
| AI NW-side model (Logged data collection) | gNB /OAM | UE | Logged measurements from UE side | gNB/OAM | Data collection initiation Data collection configuration Data forwarding (in case handover) |
| 5G Sensing | SF | gNB | Sensing measurement results | SF | Data generation Data processing Data reporting |
| 6G Sensing[Note3] | SF/RAN node | Peer RAN node/UE | Sensing measurement results | SF | Data collection initiation Data generation Data processing Data reporting |
| Note 3, for 6G sensing, RAN node may collect sensing measurements from UE or other RAN node and forwarded to data consumer (e.g., SF or other RAN node) |  |  |  |  |  |

In 5GA, emerging use cases such as the two-sided model (e.g., AI/ML-based CSI compression) have been discussed. These introduce a need to transfer AI model parameters and datasets from the gNB to OTT servers via OAM/CN. Another AI-related air interface use case involves logged data collection for network-side model training. RAN3's involvement focus on enabling data forwarding after UE handover, which is currently unsupported.

For 5GA sensing, the focus is on gNB-based monostatic mode, where sensing data must be reported from the gNB to the Sensing Function. Furthermore, 6G sensing supports all sensing modes and involves a broader range of sensing entities. This introduces a requirement to support sensing data transmission between these entities.

Summary of data related functionality in RAN

Based on the above analysis, a RAN node may support the following functionalities related to data collection and transfer. The purpose of discussing these data-related functionalities is to help identify potential areas for future RAN3 considerations.

| Data Functionality | Description | Use cases | Remarks |
| --- | --- | --- | --- |
| Data collection configuration | RAN node, as indeterminate node, provides configurations to UE and receive the data from UE. | MDT, MDT, QoE, AI Air | Different configurations may be used for different use cases, and as UE is involved, RAN3 can wait for other WGs’ progresses on this aspect. |
| Data forwarding | RAN node, as indeterminate node, forward the data from UE/other RAN node to the data consumer | AI Air, 6G sensing | Data forwarding is for UE side data, the data is received from UE can be forwarded to CN/RAN/OAM, in general, RAN3 could capture this RAN functionality, but the details are subject to RAN2 discussion. |
| Data generation | RAN node, as data producer, performs measurements | Trace, MDT, Sensing, AI RAN, AI Air | Data generation is considered an implementation-level function and does not have direct impacts on specification work. |
| Data processing | RAN node, as a processor, performs data processing and calculation | AI, Sensing | Normally, data generation and data processing are located in the same entity, which is RAN implementation, but as new requirements in 6G, new considerations needs to be studied. For sensing, the data processing may be located in RAN node and/or CN function, this can be discussed separately from AI/ML use case, and should wait for SA2 and RAN2 progress. For AI, the data processing could be model training and inference. While RAN node is equipped with data processing capabilities, AI model training typically requires significant computational resources. Due to cost and power consumption concerns, operators may avoid deploying high-performance computing at every base station. As a result, data processing may be handled by dedicated nodes or more capable base stations, meaning the data processing node and the data-generating node may not be the same. In this case, RAN3 could consider whether there is a dedicated computation node (e.g., AI control function as we mentioned in [3]) |
| Data reporting | RAN node reports the data generated/processed by RAN node to the data consumer. | Trace, MDT, AI, Sensing | RAN3 could initiate discussions on data reporting by considering the data types, roles of data producers/consumers, and the associated requirements. Considering trace and MDT are pending to SA5, as a starting point, RAN3 may focus on the use cases related to AI/ML for RAN and sensing (TRP involved sensing). |
| Data collection initiation | RAN node, as data consumer, request data from other gNBs/UE/CN | AI Air, AI RAN. 6G sensing[Note4] | Data collection initiation between RAN and CN may be related to AI air use case, which need to wait for RAN1/2 progress, RAN3 can focus on AI RAN use cases first. Data collection initiation may have relation to data reporting when it comes to two node interactions, e.g., between two RAN nodes, RAN-CN. it can be discussed together with data reporting. |
| [Note4] for 6G sensing, sensing-assisted communication can be considered. |  |  |  |

Observation 1: Potential areas for RAN3 discussion related to data functionalities include:

Data collection configuration – subject to the progress from RAN2, SA5, and SA2

Data forwarding – subject to the progresses from RAN2

Data processing – consideration of dedicated computation node within the AI control function

Data collection initiation and Data reporting – focusing on data types, endpoints, and requirements

Proposal 1: RAN3 initiates the studies on the following data-related functionalities within the 6G RAN:

Data processing – focusing on architectural and deployment aspects

Data collection initiation and Data reporting – focusing on data types, endpoints, and requirements

2.2 	New AI control (data processing) function for AI/ML

As discussed in [4], a new AI control function may be required to enable intelligent and adaptive network operations in 6G. Given the complexity and diversity of AI-native services, this function may be deployed in either a centralized or distributed manner, offering the flexibility to meet varying performance and scalability requirements across different network scenarios. Whether implemented inside or outside of a RAN node, the AI control function encompasses major data processing tasks—such as AI model training and inference—which are integral to its operation.

| - AI Control Function is inside RAN node | - AI Control Function is outside of RAN node |
| --- | --- |
| UE RAN Node 6GC Lower layer AI Control function | UE AI control FUNCTION 6GC RAN Node RAN Node |
| - • Integrated AI execution (LCM+ data processing) | - • Centralized control with distributed processing |

In the 5G system, such data processing can be performed directly within the gNB where the model is used. Alternatively, high-computation tasks like model training may be offloaded to the OAM, while inference and other lower-latency tasks remain within the gNB for local execution. However, offloading data processing to the OAM can introduce additional latency and increase energy consumption due to longer transmission paths and centralized resource usage. Deploying these tasks closer to the base station or at a dedicated edge entity enables faster response times and more efficient energy usage—especially crucial for latency-sensitive AI applications.

To balance cost and performance, operators may opt to deploy high-computation capabilities only at selected base stations or dedicated edge nodes rather than across the entire network. This selective deployment helps manage costs, energy consumption, and operational complexity while still supporting advanced AI use cases.

The introduction of this new network-side entity should be completely transparent to the UE. The UE should operate without any awareness of whether an AI function is running inside its serving gNB, on a dedicated node, This principle ensures that future enhancements in the RAN (AI function) do not break interoperability with legacy UEs deployed in the network, and conversely, that new UEs can operate in networks that have not yet deployed such AI features.

Based on the above, the following observations and proposals are suggested:

Observation 2: The 6G RAN architecture should support an AI control function—deployable in either a centralized or distributed manner - to facilitate intelligent air interface management and efficient handling of AI-related data processing.

Observation 3: When deployed outside of a RAN node, the AI control entity may be implemented as a new logical node or through other implementation-specific means.

Observation 4: Offloading AI model training to the OAM may lead to increased latency and energy consumption, making localized or edge-based processing more suitable for time-sensitive AI/ML applications.

Observation 5: To optimize both cost and performance, operators may deploy high-computation capabilities only at selected base stations or dedicated nodes rather than ubiquitously.

Proposal 2: It is proposed that RAN3 study whether to introduce a dedicated logical node (e.g., an AI control or data processing node) to support high-computation requirements for AI/ML workloads.

Proposal 3: If such a dedicated node or function is introduced, its deployment should be transparent to UE.

2.3 	Data collection initiation and data reporting

As analyzed in Section 2.1, RAN3 can focus on data collection initiation and data reporting between RAN nodes, between RAN and CN, where the RAN node serves as a data generator or/and processor and reports RAN-side data to the data consumer, or served as data consumer, to request data from CN or other RAN node. In cases where OAM and/or UE is involved, the details depends on SA5 and RAN2 progress, and RAN3 can defer its discussion accordingly.

We summarized the following cases for data collection initiation and data reporting in more detailed ways.

| Case 1 | NF as Data Consumer – Direct Request to RAN Node The NF directly requests the RAN node to collect and report data to itself. |
| --- | --- |
| Example Use cases | AI: An AI function in the core network requests RAN-side data for AI operations such as model training or performance monitoring. Sensing: The SF instructs the RAN node to perform sensing and report the results back to SF. |

| Case 1bis | NF as Data Consumer – Indirect Request via Another NF NF 1 requests data from RAN node on behave of NF 2, a dedicated transmission connection may be established between RAN node and NF2 for data reporting. |
| --- | --- |
| Example Use cases | AI: AI function in CN requests RAN-side data via the other NF. The RAN node then forwards the data to the AI Function (AIF) through a direct connection established via other NF. Sensing: SF requests sensing via other NF, and the RAN node reports the sensing data to SF through a direct connection established via other NF. |

| Case 2 | RAN Node as Data Consumer – Direct Request to NF RAN node requests NF to report the data to itself. |
| --- | --- |
| Example Use cases | AI: RAN node request AI function in CN to provide CN side data for AI operation (e.g., AI model training, performance monitoring), similar to 5G AI positioning data collection |

| Case 2bis | RAN Node as Data Consumer – Indirect Request via Another NF The RAN node sends a data collection request to NF1, which forwards it to NF2 (the actual data producer). A dedicated transmission path may be established between the RAN node and NF2 for data reporting. |
| --- | --- |
| Example Use cases | AI: AI function in CN request RAN node to provide RAN side data for AI operation via other NF, and RAN node provide the data via a direct connection between RAN node and AIF that may be established via other NF. |

| Case 3 | RAN Node as Data Consumer – Direct Request to Another RAN Node RAN node 1 requests data collection from RAN node 2. |
| --- | --- |
| Example Use cases | AI: RAN node 1 requests RAN node 2 to collect the data for AI operations (e.g., AI model training, inference, or performance monitoring). Sensing: RAN node 1 request RAN node 2 to provide sensing data for e.g., TRP-TRP bi-static sensing. |

Cases 1/2 and 1bis/2bis reflect different RAN architectural models, highlighting that architecture design directly influences the data reporting mechanisms. These cases can serve as a foundation for RAN3 discussions on data transfer.

In general, RAN3's focus should include the types of data involved, their associated services and use cases, the roles of data producers and consumers, as well as data volume and transmission requirements. The outcome of these studies will have implications for 6G architecture, protocol, and signaling design - particularly in support of new services.

Another key aspect to clarify is the meaning of ‘protocol’. In our understanding, from data collection point of view, ‘protocol’ here means the transport layer protocol used for data transfer. Functionality, such as processing of AI/ML data/management, sensing data/management should be discussed separately in AI/ML and sensing agenda. Based on this assumption, functionality used for AI/ML and/or Sensing can leverage the same transport layer protocol for data transfer if termination points are the same.

Observation 6, the RAN/CN architectures impacts whether data is transferred directly or indirectly (e.g., via intermediate NFs), influencing latency, signaling load, and protocol complexity.

Observation 7, AI-driven tasks (like model training or inference) and sensing introduce diverse needs for data volume, transfer frequency, and latency, directly affecting how data producers and consumers are linked and which transfer paths (direct or indirect) are most suitable.

Observation 8, The type of data, the roles of nodes (as data producer, consumer, or relay), and performance requirements (e.g., throughput, latency) will influence protocol and interface design, especially when supporting new services like native AI and integrated sensing.

Observation 9: Transport layer protocol of data collection framework is used for carrying data defined by AI/ML or sensing specific functionality protocol(s). Functionality protocol(s) for AI/ML and sensing should be discussed in AI/ML and sensing specific agenda, respectively.

Observation 10: Transport layer protocol of data collection is used for carrying data defined by AI/ML or sensing specific functionality. Functionality for AI/ML and sensing should be discussed in AI/ML RAN and sensing specific agenda, respectively.

Proposal 4, RAN3 start the study on data collection initiation and data reporting in the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, etc);

- For each type of data, study:

-- Applicable use case(s)

-- End point pairs (i.e., producer and consumer), including RAN and RAN, RAN and CN;

-- Data size in a single reporting;

-- Frequency of data reporting;

-- transmission requirements.

In the Annex, we also provide the a pCR to capture the data related functions to the TR 38.760-3.

Proposal 5, RAN3 agrees the pCR on data management to TR 38.760-3.

3	Conclusion

In this contribution, we discussed the RAN3 scope for data collection and transfer to identify the discussion areas, and then analysis the potential impacts on architecture and protocol design from the potential discussion area, and have the following observations and proposals.

RAN3 scope for data collection and transfer

Observation 1: Potential areas for RAN3 discussion related to data functionalities include:

Data collection configuration – subject to the progress from RAN2, SA5, and SA2

Data forwarding – subject to the progresses from RAN2

Data processing – consideration of dedicated computation node within the AI control function

Data collection initiation and Data reporting – focusing on data types, endpoints, and requirements

Proposal 1: RAN3 initiates the studies on the following data-related functionalities within the 6G RAN:

Data processing – focusing on architectural and deployment aspects

Data collection initiation and Data reporting – focusing on data types, endpoints, and requirements

- New AI control (data processing) function for AI/ML

- Observation 2: The 6G RAN architecture should support an AI control function—deployable in either a centralized or distributed manner - to facilitate intelligent air interface management and efficient handling of AI-related data processing.

- Observation 3: When deployed outside of a RAN node, the AI control entity may be implemented as a new logical node or through other implementation-specific means.

- Observation 4: Offloading AI model training to the OAM may lead to increased latency and energy consumption, making localized or edge-based processing more suitable for time-sensitive AI/ML applications.

- Observation 5: To optimize both cost and performance, operators may deploy high-computation capabilities only at selected base stations or dedicated nodes rather than ubiquitously.

- Proposal 2: It is proposed that RAN3 study whether to introduce a dedicated logical node (e.g., an AI control or data processing node) to support high-computation requirements for AI/ML workloads.

- Proposal 3: If such a dedicated node or function is introduced, its deployment should be transparent to UE.

- Data collection initiation and data reporting

Observation 6, the RAN/CN architectures impacts whether data is transferred directly or indirectly (e.g., via intermediate NFs), influencing latency, signaling load, and protocol complexity.

Observation 7, AI-driven tasks (like model training or inference) and sensing introduce diverse needs for data volume, transfer frequency, and latency, directly affecting how data producers and consumers are linked and which transfer paths (direct or indirect) are most suitable.

Observation 8, The type of data, the roles of nodes (as data producer, consumer, or relay), and performance requirements (e.g., throughput, latency) will influence protocol and interface design, especially when supporting new services like native AI and integrated sensing.

Observation 9: Transport layer protocol of data collection framework is used for carrying data defined by AI/ML or sensing specific functionality protocol(s). Functionality protocol(s) for AI/ML and sensing should be discussed in AI/ML and sensing specific agenda, respectively.

Observation 10: Transport layer protocol of data collection is used for carrying data defined by AI/ML or sensing specific functionality. Functionality for AI/ML and sensing should be discussed in AI/ML RAN and sensing specific agenda, respectively.

Proposal 4, RAN3 start the study on data collection initiation and data reporting in the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, etc);

- For each type of data, study:

-- Applicable use case(s)

-- End point pairs (i.e., producer and consumer), including RAN and RAN, RAN and CN;

-- Data size in a single reporting;

-- Frequency of data reporting;

-- transmission requirements.

Proposal 5, RAN3 agrees the pCR on data management to TR 38.760-3.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC

[3] 3GPP TS 38.300, NR; NR and NG-RAN Overall Description, stage2.

[4] R3-258281, RAN logical architecture and RAN functions, Xiaomi

5	Annex (pCR to 6G TR 38.760-3)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

6G RAN node hosts the following functions:

-  Radio Resource Management, details subject to RAN1/2 progress;

-  Connection Setup and Release;

-  Session Management, subject to SA2 progress;

-  QoS flow management and mapping to DRBs, subject to SA2 progress;

-  Support of Network Slicing;

-  NF selection;

-  User Plane Data Routing to UPF;

-  Control Plane Information Routing to NF;

-  Distribution function for NAS messages;

-  Scheduling and Transmission of Paging Messages, subject to RAN2 progress;

-  Scheduling and Transmission of System Broadcast Information, subject to RAN2 progress;

-  Radio Access Network Sharing;

- 	AI/ML, FFS on details;

-	Sensing, FFS on details;

- 	Data management, including data collection initiation, data reporting and data forwarding;

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>