3GPP TSG-RAN3 Meeting #130	R3-258283

Dallas, US, 17 - 21 November, 2025

Agenda item:	10.3.1

Source:	Xiaomi

Title:	General principles and functions on RAN-CN interface

Document for:	Discussion and Decision

1	Introduction

In RAN3#129bis meeting, general principles and RAN-CN interface functions has been discussed and approved in [1] draft_R3-257330 Skeleton TR38.760-3.

The discussion were mainly based on the 5G principles and requirements. However, according to RAN #109 meeting [2], the following services have been agreed as 6GR requirements of new and existing services, including services such as AI, sensing, etc, which will generate tremendous amount of data. Supporting data transmissions in RAN-CN interface for new services becomes critical in 6GR.

It is agreed to capture following into TR 38.914 section 5.4: - Mobile broadband - Immersive Communication - Massive Communication (IoT) - Sensing - AI - Voice - Regulatory services

In this contribution, we’d like to discuss the general principles on RAN-CN interface and RAN-CN functions, specifically focus the requirements on from the new services (i.e., sensing and AI).

2	Discussion

2.1	Data transfer requirements for new services

In 5G, RAN architecture supports user plane and control plane separation as they experience different load and have different characteristics, as shown in Figure 1.

Figure 1 5G system architecture and interfaces

To support emerging 6G services such as AI and sensing, RAN-CN interface should consider the data transmission requirements.

We summarized the following data transmission requirements especially for RAN3 to consider, for the data transmission from UE to network, we can wait for RAN2 progress.

| Use case | Data path | Data volumes and delay/reliability requirements | Data types (UE associated or user plane data) | Fit for existing CP or UP |
| --- | --- | --- | --- | --- |
| Training data/inference data for RAN-side/CN-side model | RAN<->CN | Similar to existing signalling but with large amount of data Low or high latency depending on the purpose High reliability | UE associated (non user plane data) or non UE associated (depending on use cases) | No 5G CP cannot afford large volume of data 5G UP is not designed for non-UE associated data and not reliable |
| Model parameters/data set for two-sided model | RAN->CN->AF (OTT server) | ~ tens of KBs to hundreds of MBs Relaxed latency High reliability | Non UE associated | No 5G CP cannot afford large volume of data 5G UP is not designed for non-UE associated and not reliable |
| Sensing data (e.g., row data, spectrum measurements) from TRP | RAN->CN | ~ tens of KBs to hundreds of MBs, depends on the data type High latency High reliability | Non UE associated | Yes or No depends on the data type 5G CP cannot afford large volume of data 5G UP is not designed for non-UE associated and not reliable |

Unlike traditional user data, AI-related traffic - such as model parameters, training datasets, requires high-throughput, relax-latency transmission but does not fit within the scope of existing user or control plane functions. The 5G control plane is not designed for large volumes of data transport, and non-UE-associated data or non user plane data is not well-suited for transmission via the current user plane.

Observation 1, 5G control plane and user plane between RAN and CN does not fit for reliable large-volume data transmission for AI and sensing (e.g., AI model parameters/dataset, some types of the sensing data).

If large volume of data transmission is needed, a new dedicated plane between RAN and CN is needed for the reliable and efficient big data transmission, which needs to be studied in RAN3, in coordination with SA2. It should be noted that these data types differ from user plane data: while user plane data is terminated between the UE and the data network and remains transparent to the 3GPP network, sensing or AI-related data is generated within the 3GPP network and must be visible to it.

Figure 2 6G system architecture and interfaces

Observation 2, Sensing and AI data are generated by the RAN node and/or the UE, unlike legacy user plane data that remains transparent to the RAN.

Proposal 1, the 6G RAN-CN interface supports efficient and reliable transfer of high-volume non-user plane data (e.g., AI for Net, Sensing).

2.2	RAN-CN Interface Functions

- 2.2.1		Control plane functions

RAN-CN functions based on 5G functions

According to the 5G NG control plane interface function as specified in TS 38.410, we summarized in the following table to see whether it can be applicable for 6G RAN-CN user plane interface.

| General Functions | RAN-CN control plane functions | Basic Function for 6G | Remarks |
| --- | --- | --- | --- |
| Session Management | Handles user data transmission connectivity lifecycle 6.1 PDU Session Management Procedures | PDU Session Management (already agreed) | PDU session management is already captured, while details pending to SA2, and MBS related are FFS |
| Context Management | Manages UE state and resources 6.2 UE Context Management Procedures 6.19 UE Context Suspend (ng-eNB ) 6.22 UE Context Resume (ng-eNB) 6.20 Connection Establishment Indication (CIoT-specific) | UE Context Management (already agreed) | UE context management is already captured, while others are not for basic functions |
| Mobility Management | Orchestrates handovers and location updates, UE reachability operations 6.4 UE Mobility Management Procedures 6.5 Paging Procedure | UE Mobility Management Paging | Both UE mobility management and paging are essential function for any cellular system |
| Signaling Transport | Carries control plane messages 6.3 NAS Transport Procedures (Uplink/Downlink NAS message delivery) 6.12 NRPPa Procedures (Positioning protocol transport) | NAS Transport (already agreed) | There must be NAS transport function, however, whether RAN node also need perform NAS routing is FFS and depends on the architecture design. |
| Network Management | Maintains infrastructure operations 6.6 AMF Management Procedures (AMF status, TNLA binding release) 6.7 NG Interface Management (Setup/Reset/Configuration updates) 6.14 Configuration Transfer (RAN configuration exchange) 6.13 Overload Control (Start/Stop traffic throttling)） | FFS, depends on the protocol options | The applicability of these existing 5G functions in 6G will largely depend on the architectural options chosen for the RAN–CN interface in 6G. P2P interface model could allow these functions to be retained with minimal changes,. On the other hand, SBI approach may not directly map to all traditional RAN–CN management functions. |
| UE-Specific Services | UE-specific functions 6.10 UE Radio Capability Management 6.11 UE Tracing (Trace activation/deactivation) 6.17 Retrieve UE Information 6.9 Location Reporting | FFS, subject to RAN2/SA | UE-specific functions from 5G are closely tied to UE behavior and radio interface control. Their definitions and mechanisms in 6G will depend heavily on ongoing discussions in RAN2 and SA2. |
| Optimization & Efficiency | Specialized resource handling 6.15 Secondary RAT Data Usage Report 6.16 RIM Information Transfer (Remote interference management) 6.18 RAN CP Relocation 6.21 AMF CP Relocation | FFS | These 5G functions are closely related to DC and SON functionalities. Their applicability and potential reuse in 6G may require discussion and alignment at the RAN plenary level. |
| Broadcast/Multicast | Group communication services 6.8 Warning Message Transmission (PWS management) 6.23 NR MBS Session Management Procedures 6.24 Multicast Group Paging | Warning Message Transmission | MBS related function are FFS |

Proposal 2, the following existing 5G RAN-CN control plane function can be a start point in 6G.

- PDU Session Management (already agreed)

- UE Context Management (already agreed)

- NAS Transport (already agreed)

- UE Mobility Management

- Paging

- Warning Message Transmission

RAN-CN functions for AI/ML

In 5GA AI/ML for NG-RAN, data collection functions which is limited to Xn interface has been specified to support data collection and reporting for AI/ML NG-RAN use cases. SA2 has also specified the NWDAF in Core Network to support corresponding AI/ML use cases. While some data and features could be mutually beneficial, there is currently no efficient inter-working between RAN and NWDAF, primarily due to complexity and spec impacts, as shown in Figure 3.

Figure 3 No-interworking between RAN and CN in 5G for AI

In 5GA AI/ML positioning, for gNB side model, RAN3 discussed positioning data collection for gNB to obtain the positioning data from LMF, there are two options, i.e., opportunistic data collection via NRPPa (where limitations on transaction initiation), and proactive data collection (where the solution has more SA2 impacts), considering the spec impacts, a less efficient data collection approach was selected.

Figure 4 No-interworking between RAN and CN in 5G for AI positioning

In 6G, with the support of the Native AI, data collection function on RAN-CN interface will be necessary to support more efficient and tight inter-working between RAN and CN.

Observation 3, In 5G, AI is implemented as an add-on service, yet there is currently no efficient data collection mechanism between the RAN and CN to support AI-related use cases.

Proposal 3, RAN3 agree to extending the data collection functionality in 5G Xn interface to 6G RAN-CN interface control plane to support AI/ML for RAN use cases.

RAN-CN functions for Sensing

According to 5G ISAC discussion, the sensing function is deployed in 5G core network, while in 6G, the sensing function that manage the sensing service could be deployed either in core network or RAN side for different purposes.

In case the SF in CN, the sensing management function between RAN-CN is needed. However, detailed aspects (e.g., measurement configuration/reporting, UE selection, data processing) can be discussed later until further progress is made in RAN2 and SA2.

If SF deployed in RAN side, RAN may also need interaction with CN on e.g., sensing service request handling, authorization, etc., so sensing management function is also needed but may for different purposes, details can be further discussed.

Proposal 4, RAN3 agrees to support sensing management function in 6G RAN-CN control plane interface.

- 2.2.2		User plane functions

According to the 5G NG user plane interface function as specified in TS 38.415, we summarized in the following table to see whether it can be applicable for 6G RAN-CN user plane interface.

| General Functions | RAN-CN user plane functions | Basic Function for 6G | Remarks |
| --- | --- | --- | --- |
| PDU Session Information | Provision of control information elements (e.g. QFI, RQI) associated with a PDU session. | Provision of control information associated with a PDU session | Basic user plane function, but details are subject to SA2. |
| PDU Set Information | Provision of PDU Set Information elements associated with a QoS flow. Provision of Indication of End of Data Burst for a QoS flow | FFS | PDU set in 5G is designed for XR services, this function may need confirmed by SA2. |

Proposal 5, Provision of control information associated with a PDU session can be agreed as 6G RAN-CN user plane interface function, details are FFS and subject to SA2.

- 2.2.3		Data plane functions

- In our contribution on RAN3 considerations on data transfer in [3], we discussed the potential area on data related functionalities, in general, we think the data forwarding and data reporting are related to the RAN-CN data plane, the following are the details analysis.

| Data Functionality | Description | Use cases | Remarks |
| --- | --- | --- | --- |
| Data forwarding | RAN node, as indeterminate node, forward the data from UE/other RAN node to the data consumer | AI Air, 6G sensing | Data forwarding is for UE side data, the data is received from UE can be forwarded to CN/RAN/OAM, if the data volumes are large, and when the UP solution is not preferred, then a data plane may be needed, however, the details are subject to RAN2 discussion. |
| Data reporting | RAN node reports the data generated/processed by RAN node to the NFs. | AI Air, AI RAN, Sensing | In case of AI/ML for RAN and sensing (TRP involved sensing), the data is non-UE associated data, if data volume is large, both CP and UP are not suitable, thus, a data plane support reliable data delivery can be considered. |

- Observation 4, both data forwarding and data reporting are related to data plane, however, the details of the data forwarding needs to wait for RAN2 progress.

- Proposal 6, data reporting function can be agreed as 6G RAN-CN data plane interface function.

- In the Annex, we provide a pCR to 6G TR 38.760-3 TR to capture what we discussed in the this contributions.

- Proposal 7, RAN3 agrees the pCR to 6G TR 38.760-3 in the Annex.

3	Conclusion

In this contribution, we discussed the general requirements and functions on RAN-CN interface and have the following observations and proposals.

Data transfer requirements for new services

Observation 1, 5G control plane and user plane between RAN and CN does not fit for reliable large-volume data transmission for AI and sensing (e.g., AI model parameters/dataset, some types of the sensing data).

Observation 2, Sensing and AI data are generated by the RAN node and/or the UE, unlike legacy user plane data that remains transparent to the RAN.

Proposal 1, the 6G RAN-CN interface supports efficient and reliable transfer of high-volume non-user plane data (e.g., AI for Net, Sensing).

RAN-CN Interface Functions

Control plane functions:

Proposal 2, the following existing 5G RAN-CN control plane function can be a start point in 6G.

- PDU Session Management (already agreed)

- UE Context Management (already agreed)

- NAS Transport (already agreed)

- UE Mobility Management

- Paging

- Warning Message Transmission

Observation 3, In 5G, AI is implemented as an add-on service, yet there is currently no efficient data collection mechanism between the RAN and CN to support AI-related use cases.

Proposal 3, RAN3 agree to extending the data collection functionality in 5G Xn interface to 6G RAN-CN interface control plane to support AI/ML for RAN use cases.

Proposal 4, RAN3 agrees to support sensing management function in 6G RAN-CN control plane interface.

User plane functions

Proposal 5, Provision of control information associated with a PDU session can be agreed as 6G RAN-CN user plane interface function, details are FFS and subject to SA2.

Data plane functions

- Observation 4, both data forwarding and data reporting are related to data plane, however, the details of the data forwarding needs to wait for RAN2 progress.

- Proposal 6, data reporting function can be agreed as 6G RAN-CN data plane interface function.

- Proposal 7, RAN3 agrees the pCR to 6G TR 38.760-3 in the Annex.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC

[3] R3-258282, RAN3 Considerations on 6GR Data transfer, Xiaomi

5	Annex (pCR to 6G TR 38.760-3)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6.1		RAN-CN Interface

6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN;

- 	the 6G RAN-CN interface supports efficient and reliable transfer of high-volume non-user plane data.

6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress;

- 	UE Mobility Management: The functionality to prepare, execute or cancel handovers;

-  Paging: The functionality to send paging requests to the 6G RAN nodes involved in the paging area;

- 	AI/ML Support: The functionalitiy to initiate AI related data collection;

- 	Sensing Management: The functionalitiy to handle the sensing related management, FFS on the details.

RAN-CN user plane interface supports following functions:

-	Provision of control information associated with a PDU session: details are FFS;

RAN-CN data plane interface supports following functions:

-	Data Reporting, details are FFS;

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>