3GPP TSG-RAN3 Meeting #130	R3-258284

Dallas, US, 17 - 21 November, 2025

Agenda item:	10.3.2

Source:	Xiaomi

Title:	RAN-CN interface and protocol options

Document for:	Discussion

1	Introduction

A new study item was agreed at RAN#108 and revived in RAN#109 that aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements [1].

One of the objectives of the study is to study the RAN-CN interface and protocol stack.

(6)Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a)Overall RAN architecture aspects [RAN3, RAN2] b)RAN-CN functional split, interface, protocol stack and procedures [RAN3] c)RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we’d like to discuss the RAN-CN interface and protocol options based on the requirements discussed in our paper in [2], and provide the evaluation considerations between SBI and P2P.

- Discussion

As discussed in [2], given the new requirements for 6G, there is a necessity to transmit large volumes of non-user plane data. From a discussion point of view, we assume there will be a new plane for data transmission (e.g., data plane), however, it is also possible that this data plane could be realized via the new control plane or user plane.

Observation 1, From discussion point of view, we consider the possibility of a new plane with requirements that differ from those of the legacy control and user planes, while acknowledging that such functionality could also be realized through enhancements to the new control plane and user plane.

2.1	User Plane Interface and protocol

In our view, the User Plane will continue to carry traditional user traffic (e.g., video streaming, web browsing) associated with a specific UE's PDU Session.

Key requirements:

- Ultra-High Throughput and Low Latency: To support extreme data rates and tactile internet applications.

- QoS Handling: Critical for deterministic communications and XR applications.

- Network Efficiency: Minimizing header overhead and processing latency is crucial.

We think the protocol design of NG-U can still satisfy the above requirements for user plane data transmission, so the GTP-U-based protocol can be reused.

Similar to 5G NG-U, a Nx user plane interface (Nx-U) can be defined between the 6G RAN node and the 6G UPF as shown on Figure below. The transport network layer is built on IP transport and GTP-U is re-used on top of UDP/IP to carry the user plane PDUs between the 6G RAN node and the 6G UPF.

In 5G, NG-U has been enhanced with features such as finer-grained QoS flow support and PDU set handling. As a mature protocol, its security can be adequately addressed through IPsec and application-layer security mechanisms. Introducing or replacing GTP-U with QUIC for enhanced security appears unnecessary, as it could introduce additional overhead and complexity, potentially impairing transmission efficiency and processing performance. Therefore, for user data transmission, adopting QUIC would be redundant and potentially detrimental to the optimized 5G user plane architecture.

Observation 2, GTP-U in 5G already well supports fine-grained QoS and NG-U security can be guaranteed by IPsec and application layer mechanism. Introducing QUIC is unnecessary and could reduce efficiency without significant benefits.

Proposal 1, 6G RAN-CN user plane provides the non-guaranteed delivery of user plane PDUs between the 6G RAN node and the 6G UPF. The User plane PDU/GTP-U/UDP/IP protocol stack is re-used.

2.2	Control Plane Interface and protocol

- The Control Plane's primary role remains the management of connectivity, mobility, and sessions. In addition, it may also need to manage the new Data Plane (if any).

- Key Requirements:

- - Signaling efficiency and latency: While handling core functions, it must be ultra-efficient to support massive numbers of devices and critical latency for control loops.

- - Reliability and security: support guaranteed delivery and integrity of signaling messages.

- - Management of sessions: The CP must be able to establish, modify, and release contexts not just for UP, but also for the new Data Plane connections.

- Candidate Protocol Stacks:

- - SBI, A service-oriented architecture where network functions offer and consume services through standardised APIs, commonly used in 5G core networks.

- - P2P, refers to a direct, dedicated communication link between two network elements—such as between a gNB and AMF - using protocols like SCTP for reliable signalling transmission. It establishes a fixed connection for control or user plane data exchange, similar to the role of NG-C in 5G networks

- In this section, the P2P and SBI interface and protocols for control plane are discussed separately.

2.2.1	P2P Architecture and protocol stacks

- Unlike in 5G, 6G P2P communication needs to consider new service requirements such as sensing and AI. This raises the question of whether base stations should establish direct connections with other NFs - beyond the AMF - that support AI or sensing, or whether indirect connections (similar to those with the SMF or LMF in 5G) would be more suitable. This decision may impact the design of the corresponding interfaces and protocols.

- Option 1 (Direct Connectivity): Establishes direct links between the RAN node and relevant NFs requiring coordination. Its protocol resembles NG-C.

- Option 2 (Indirect Connectivity): Relies on 5G's indirect model via AMF/UPF for all RAN-NF coordination, and this is less preferred but remains open for study. Protocol design offers two sub-options for sensing/AI coordination:

- - Option 2-1: Uses a new protocol over NxAP, similar to the NRPPa model for positioning.

- - Option 2-2: Embed service-related information within NxAP messages, similar to SM info transport in NGAP.

- For NFs in core network shown in the table below, the final definitions depend on SA2 outcomes. However, it is foreseen that some NFs (e.g., NF1, NF2) need tight RAN coordination, particularly for sensing/AI use cases, whereas NF3 and NF4 will operate without RAN coordination.

| - Option 1 direct connectivity with new NFs (sensing, AI) | - Option 2 indirect connectivity with new NFs (sensing, AI) |
| --- | --- |
| - Protocol stacks between 6G RAN node and AMF/SF/AIF |  |
| Option 1 6G Nx control plane interface (Nx-C) | - Option 2-1, new protocol layer above NxAP. - Option 2-2, service container included in NxAP. |
| - For both options, the transport layer protocol could be SCTP or QUIC |  |

- The direct or indirect architecture needs to be discussed in coordination with SA2. At current stage, RAN3 can treat CN as as a unified entity, how to support the connection between RAN and NFs without AMF involvement can be discussed later.

- Observation 3, The selection between direct and indirect connections, though requiring SA2 coordination, could impact interface and protocol design, as well as standardization efforts. While further details may depend on progress from other groups, this factor remains relevant when comparing P2P and SBI approaches.

- Regarding transport protocols, two major candidate transport protocols are being discussed:

- SCTP (Stream Control Transmission Protocol): A reliable, message-oriented transport layer protocol developed for telephony signaling. It supports multi-streaming and multi-homing, making it robust for mission-critical and delay-sensitive applications—traits valued in traditional architectures.

- QUIC (Quick UDP Internet Connections): A modern transport protocol developed by Google, running over UDP. It reduces connection establishment latency and supports built-in encryption. Its multiplexing capabilities and adaptability make it suitable for high-efficiency, low-latency services, especially relevant for AI-driven and sensing-assisted applications in 6G.

| - SCTP-based P2P | - QUIC-based P2P |
| --- | --- |
| - Advantages: - Reliable, message-oriented transmission with strong sequencing and multi-stream support. - Built-in features like multi-homing (redundant network paths) and partial reliability. - Designed for signaling and telecommunication applications with strict delivery requirements. - Limitations: - Connection setup involves a 4-way handshake, introducing higher latency. - Less efficient in high-loss or high-throughput scenarios due to head-of-line blocking within an association. - Limited support in end-user applications and modern web environments. | - Advantages: - Significantly reduced latency with 0-RTT or 1-RTT handshake resumption. - Native encryption (TLS 1.3) and improved security by default. - Prevents head-of-line blocking across streams through multiplexing, enhancing performance. - Adapts well to fluctuating network conditions with advanced congestion control. - Limitations: - Relies on UDP, which may be restricted or impaired by certain firewalls or middleboxes. - Larger protocol header overhead compared to SCTP or TCP. - Primarily designed for client-server models. |

- Proposal 2: RAN3 studies the following protocol stacks for RAN-CN control plane for P2P arch:

- - NxAP/SCTP/TCP/IP

- - NxAP/QUIC/UDP/IP

2.2.2	Service-based Architecture and protocol stacks

- Service-based interface enables RAN to function(s) as both a service producer (exposing RAN service to CN) and consumer (utilizing CN services), there may be two implementation options:

- Option 1 (Full SBA): Implement service-based interfaces for all RAN functions.

- Option 2 (Partial SBA): Apply service-based interfaces only to new services (e.g., AI/sensing), while maintaining traditional point-to-point interfaces for legacy access functions per 5G principles.

| Option1, full SBA | Option 2, partial SBA |
| --- | --- |
| The SBI protocol can refer to the protocol stack defined in TS 29.500, if new protocol stack for 6G is defined in SA2, SBI for RAN-CN need to align with CT4. |  |

Proposal 3, RAN3 study the following options for service-based RAN architecture.

- full RAN service-based architecture

- partial RAN service-based architecture

Proposal 4, the protocol stack for SBI should be aligned with CT4.

2.2	Data Plane Interface and protocol

- Data plane may be the most critical innovation for 6G. This plane is dedicated to large-volume and reliable non-user plane data transfer between RAN node and core network NF(s) for e.g., AI and sensing. The concept of data plane is used for discussion purposes, it may be merged to control plane or user plane, depending on whether the protocol design can support the requirements on the data plane.

- Key Considerations:

- - High Throughput and Efficiency: Optimized for moving large datasets (MBs to GBs) efficiently, potentially necessarily with ultra-low latency depending on the use cases.

- - Reliability: Must guarantee data delivery, unlike the best-effort nature of the UP. Retransmissions and acknowledgments are required.

- - Non-UE associated and Non-user plane data: The protocol must natively support connections between network functions (e.g., gNB <-> Sensing Function) without being tied to a user's PDU session.

- To enable reliable and high-volume data transmission, QUIC is a strong candidate. Given GTP-U’s well-defined QoS handling, offers good backward compatibility, which further enhances its suitability, using GTP-U could also meet the requirements for robust and large-scale data delivery, however, UDP/GTP-U protocol does not support reliable delivery, which may be one challenges to use GTP-U. Even so, we propose the following options as potential solution for data plane.

- Candidate Protocol Stacks:

| - Candidate options | - QUIC-based data plane | - GTP-U-based data plane |
| --- | --- | --- |
| - Protocol stack |  |  |
| - Remarks | - Strong, simple, and efficient option if the data plane operates mostly over IP-based infrastructure without requiring deep network integration. | - Preferable if the data plane must interoperate with cellular core networks, require in-network tunneling, identity-based routing, or traffic steering capabilities. |
| - Relation to the control plane and user plane | - If the QUIC protocol is adopted for both the control plane and the data plane, they can be unified into a single integrated plane. For instance, a QUIC-based control plane can also handle high-volume data transmission. | - If GTP-U is adopted as the user plane protocol, the user plane and data plane could potentially be unified into a single integrated plane. - However, unlike user plane PDUs—which are not interpreted by the RAN node—the data plane PDUs must be explicitly defined by 3GPP specifications. |

- Observation 4, QUIC-based data plane is well-suited for IP-centric environments where simplicity, efficiency, and minimal network integration are prioritized. While the GTP-U-based data plane is preferable for scenarios requiring deep integration with core networks.

- Observation 5, Adopting QUIC for both control and data planes allows for a unified plane support both singllaing and high-throughput data transmission. Using GTP-U may also unify the user and data planes.

- Proposal 6, RAN3 study the following options for the data plane

- - Data PDU/QUIC/UDP/IP

- - Data PDU/GTP-U/UDP/IP

- In the Annex, we provide a pCR to 6G TR 38.760-3 on the RAN-CN interface and protocol options.

- Proposal 7, RAN3 agrees the pCR to 6G TR 38.760-3 in the Annex

3	Conclusion

In this contribution, we discussed RAN-CN interface and protocol options, and have the following observations and proposals.

Observation 1, From discussion point of view, we consider the possibility of a new plane with requirements that differ from those of the legacy control and user planes, while acknowledging that such functionality could also be realized through enhancements to the new control plane and user plane.

- User Plane Interface and protocol

Observation 2, GTP-U in 5G already well supports fine-grained QoS and NG-U security can be guaranteed by IPsec and application layer mechanism. Introducing QUIC is unnecessary and could reduce efficiency without significant benefits.

Proposal 1, 6G RAN-CN user plane provides the non-guaranteed delivery of user plane PDUs between the 6G RAN node and the 6G UPF. The User Plane PDU/GTP-U/UDP/IP protocol stack is re-used.

- Control Plane Interface and protocol

- Observation 3, The selection between direct and indirect connections, though requiring SA2 coordination, could impact interface and protocol design, as well as standardization efforts. While further details may depend on progress from other groups, this factor remains relevant when comparing P2P and SBI approaches.

- Proposal 2: RAN3 studies the following protocol stacks for RAN-CN control plane for P2P arch:

- - NxAP/SCTP/TCP/IP

- - NxAP/QUIC/UDP/IP

Proposal 3, RAN3 study the following options for service-based RAN architecture.

- full RAN service-based architecture

- partial RAN service-based architecture

Proposal 4, the protocol stack for SBI should be aligned with CT4.

- Data Plane Interface and protocol

- Observation 4, QUIC-based data plane is well-suited for IP-centric environments where simplicity, efficiency, and minimal network integration are prioritized. While the GTP-U-based data plane is preferable for scenarios requiring deep integration with core networks.

- Observation 5, Adopting QUIC for both control and data planes allows for a unified plane support both singllaing and high-throughput data transmission. Using GTP-U may also unify the user and data planes.

- Proposal 6, RAN3 study the following options for the data plane

- - Data PDU/QUIC/UDP/IP

- - Data PDU/GTP-U/UDP/IP

- Proposal 7, RAN3 agrees the pCR to 6G TR 38.760-3 in the Annex

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2]R3-258283, General principles and functions on RAN-CN interface, Xiaomi

5	Annex (pCR to 6G TR 38.760-3 )

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

6.1.3.x1	User Plane Interface and protocol

The 6G RAN-CN user plane interface is defined between the 6G RAN node and the 6G UPF. The user plane protocol stack of the 6G RAN-CN interface is shown on Figure 6.1.3.x1-1. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the user plane PDUs between the 6G RAN node and the 6G UPF.

Figure 6.1.3.x1-1: 6G RAN-CN User Plane Protocol Stack

6.1.3.x2	Control Plane Interface and protocol

In case of P2P architecture, The 6G RAN-CN control plane interfaceis defined between the 6G RAN node and the NF(s). The control plane protocol stack of the 6G RAN-CN interface is shown on Figure 6.1.3.x2-1 and . Figure 6.1.3.x2-2 The transport network layer is built on IP transport. In the transport, IP layer point-to-point transmission is used to deliver the signalling PDUs.

Editor’s Note x-1, the transport layer could be SCTP, QUIC, GTP-U, which need further study.

Editor’s Note x-2, FFS on whether the NF is only 6G AMF, or multiple NFs.

Editor’s Note x-3, FFS on whether there is a data plane for new services (e.g., sensing, AI)

Figure 6.1.3.x2-1: 6G RAN-CN Control Plane P2P Protocol Stack (SCTP-based)

Figure 6.1.3.x2-2: 6G RAN-CN Control Plane P2P Protocol Stack (QUIC-based)

In case of service-based architecture, service-based interface is defined between 6G RAN node and Core network NFs, as shown on Figure 6.1.3.x2-2.

Editor’s Note x-4, the transport layer of SBI should be aligned with CT4.

Figure 6.1.3.x2-2: Service-based Protocol Stack

6.1.3.x3	Data Plane Interface and protocol

The 6G RAN-CN data plane interface is defined between the 6G RAN node and the 6G NFs with large volume of data transmission requirements. The two alternative options on the data plane protocol stack of the 6G RAN-CN interface are shown on Figure 6.1.3.x3-1 (QUIC-based) and Figure 6.1.3.x3-2 (GTP-U/QUIC-based). The transport network layer is built on IP transport and QUIC and/or GTP-U is used on top of UDP/IP to carry the data plane PDUs between the 6G RAN node and the 6G NFs.

Editor’s Note x-5, FFS on whether the data plane can be merged to control plane or user plane.

Figure 6.1.3.x3-1: 6G RAN-CN Data Plane Protocol Stack (QUIC-based)

Figure 6.1.3.x3-1: 6G RAN-CN Data Plane Protocol Stack (GTP-U-based)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>