3GPP TSG-RAN WG3 Meeting #130	R3-258298

Dallas, USA, 17-21, November, 2025

Agenda Item:	10.2.1

Source:	ZTE Corporation

Title:	(TP to TR38.760-3) Initial discussion on functional framework for 6G AIRAN

Document for:	pCR

1	Introduction

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In RP-251881[1], following is the objectives of AI/ML for 6G Radio Access Network:

Functional framework and data collection are under discussion in RAN2, and in RAN3 meeting, and several companies propose to trigger initial discussion in RAN3. Therefore, this discussion paper has been prepared to explore the functional framework for 6G AI RAN.

2	Discussion

According to the SA1 6G use cases and requirements [2], 6G system data denotes the data generated and controlled by the 6G system, e.g., AI/ML data, sensing data, etc. Different from traditional user traffic data, which is transmitted through the 3GPP system for user-related services, 6G system data should be delivered between data providers and consumers within the 6G system. Here, the data providers and consumers may be the UE, RAN, CN, third-party application server etc.

As a matter of fact, various data collection methods are available in 5G, e.g., MDT, QoE, L3 measurement and report, L1 measurement and report, LPP, etc. The existence of multiple, distinct data collection methods creates trouble for operators to manage data in a unified and consistent manner. In the 5G-A phase, the data collection mechanism is still under discussion among RAN2, SA2, and SA5. Until now, no consensus has been reached on UE-side data collection, and it will be further discussed in the Rel-20 AI/ML for air interface WI. Therefore, in 6G, a uniform and efficient data collection and transmission mechanism is beneficial to support 6G AI/ML.

- Various data collection methods are available in 5G. The distinct data collection methods increase the specification efforts and create trouble for operators to manage data in a unified and consistent manner.

- Design a uniform and efficient data collection and transmission mechanism among network entities (e.g., UE, RAN, CN) in 6G era to support various AI/ML use cases.

Before determining the data collection mechanism, the functional framework for RAN intelligence can be firstly discussed in RAN3 for 6G AI RAN. After we have a clear understanding of the functional process for 6G AI, it will help us better study the data collection mechanism.

The topic "AI/ML for NG-RAN" has been discussed from Rel-17 to the current Rel-20. The functional framework for RAN intelligence (as shown in Figure 1) and AI/ML-assisted use cases have been studied and specified in RAN3. Additionally, other WGs have also introduced AI/ML topics in their discussions, and RAN1 & RAN2 also designed its functional framework for RAN intelligence for Air interface (as shown in Figure 2). RAN3 focused more on data transmission between AI/ML functions, while RAN1, in addition to data transmission, also emphasized model management within the LCM.

Functional Framework in RAN3:

Figure 1. Functional Framework for RAN Intelligence

Functional Framework in RAN1:

Figure 2. Functional framework for AI/ML for NR Air Interface

- During 5G-A phase, different functional frameworks for RAN intelligence were depicted across relevant WGs.

For the 6G AI RAN discussions, a unified RAN intelligence functional framework first needs to be agreed upon by involved RAN working groups, even better to align it for inter-TSG WGs, which will help with subsequent discussions on native AI in 6G.

- A unified RAN intelligence functional framework is defined to support data collection and model management across all involved WGs.

On the other hand, Agentic AI is a key and promising AI technology in 6G. SA1 and SA2 has already initiated discussions on AI agents; therefore, when designing the unified functional framework for RAN intelligence, it is necessary and beneficial to incorporate Agentic AI. Agentic AI can be deployed on all domains, including CN, RAN and UE sides, collaborating with each other to support AI-native networks. The Agentic AI consists of several key components which encompasses the following essential parts as shown in the Figure 3:

- Perception: This enables the Agent to perceive its network environment, allowing it to receive and process multimodal inputs such as text, images, audio, radio condition or other data types.

- Decision-Making (Knowledge): This empowers the Agent with autonomous decision-making and planning capabilities, enabling it to handle and execute more complex tasks effectively.

- Knowledge Storage: This provides the Agent with memory functions, where internal storage holds the Agent's accumulated knowledge, skills, and experiences for reference and learning.

- Action: This equips the Agent with the ability to interact with the network environment, allowing it to perform actions that, combined with perception, enable autonomous completion of increasingly complex tasks.

Agentic AI can be deployed in all domains, including CN, RAN and intelligent end devices

Figure 3. Agentic AI architecture in 6G

- Incorporate Agentic AI into the 6G functional framework for RAN intelligence.

Given this, we depict a new functional framework for 6G by combining the existing frameworks and the AI Agentic Architecture.

Figure 4: Functional Framework for RAN Intelligence in 6G

Data Collection is a function that provides input data to Model training and Model inference functions.

-	Training Data: Data needed as input for the AI/ML Model Training function.

-	Monitoring Data: Data needed as input for the Management of AI/ML models or AI/ML functionalities, including data retrieving in the network environment to monitor the performance of AI/ML models or AI/ML functionalities.

-	Inference Data: Data needed as input for the AI/ML Model Inference function.

Model Training is a function that performs AI/ML model training, validation, and testing which may generate model performance metrics which can be used as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

Model Inference is a function that provides outputs from the process of applying AI/ML models or AI/ML functionalities, using the data that is provided by the Data Collection function (i.e., Inference Data) as an input. The Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

-	Model Storage/Update: Trained or updated models can be delivered to the Management and Storage function. Trained models can be stored in the function awaiting invocation, while updated models can replace previously existing ones.

The Management & Storage is a function that oversees the operation and monitoring of AI/ML models or AI/ML functionalities, including the management of model training and inference, as well as the coordination of model collaboration between network elements. It also includes the knowledge storage. For example, in a distributed learning architecture, the management function needs to coordinate the model training and inference processes between various network elements.

-	Model Retraining Request: Based on model performance feedback and monitoring data, model retraining can be triggered if the model's performance does not meet the requirements.

-	Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

Furthermore, compared with legacy AI/ML and Agentic AI, the short-term memory and long-term memory are very important to achieve good performance of LLM. Therefore, the Storage function here includes not only the AI/ML models but also the knowledge storage.

Actor is a function that receives the output from the Model Inference function or Agentic AI, and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

-	Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

Agentic AI refers to an autonomous, goal-oriented functional framework that empowers AI agents to perceive, reason, plan, and execute actions in dynamic environments, particularly complex network systems. Agentic AI integrates advanced capabilities for model training and inference to enable real-time perception of the network environment. These models are grounded in a knowledge base and prior knowledge. Given that, Agentic AI can iteratively dynamically refine their decision-making, adapt to uncertainties, and orchestrate multi-entity collaborations to fulfil the complex task.

- Agree the functional framework for RAN intelligence for 6G AI RAN above.

3	Conclusions

Following is our proposals and observations:

- Various data collection methods are available in 5G. The distinct data collection methods increase the specification efforts and create trouble for operators to manage data in a unified and consistent manner.

- Design a uniform and efficient data collection and transmission mechanism among network entities (e.g., UE, RAN, CN) in 6G era to support various AI/ML use cases.

- During 5G-A phase, different functional frameworks for RAN intelligence were depicted across relevant WGs.

- A unified RAN intelligence functional framework is defined to support data collection and model management across all involved WGs.

- Incorporate Agentic AI into the 6G functional framework for RAN intelligence.

- Agree the functional framework for RAN intelligence for 6G AI RAN above.

6	Reference

[1]	RP-251881, New SID: Study on 6G Radio, June 2025.

[2]	3GPP TR 22.870 v0.4.0, Study on 6G use cases and requirements, September, 2025.

5	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

7	AI/ML for RAN

7.2 Functional Framework

Figure 4: Functional Framework for RAN Intelligence in 6G

Data Collection is a function that provides input data to Model training and Model inference functions.

-	Training Data: Data needed as input for the AI/ML Model Training function.

-	Monitoring Data: Data needed as input for the Management of AI/ML models or AI/ML functionalities, including data retrieving in the network environment to monitor the performance of AI/ML models or AI/ML functionalities.

-	Inference Data: Data needed as input for the AI/ML Model Inference function.

Model Training is a function that performs AI/ML model training, validation, and testing which may generate model performance metrics which can be used as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

Model Inference is a function that provides outputs from the process of applying AI/ML models or AI/ML functionalities, using the data that is provided by the Data Collection function (i.e., Inference Data) as an input. The Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

-	Model Storage/Update: Trained or updated models can be delivered to the Management and Storage function. Trained models can be stored in the function awaiting invocation, while updated models can replace previously existing ones.

The Management & Storage is a function that oversees the operation and monitoring of AI/ML models or AI/ML functionalities, including the management of model training and inference, as well as the coordination of model collaboration between network elements. It also includes the knowledge storage. For example, in a distributed learning architecture, the management function needs to coordinate the model training and inference processes between various network elements.

-	Model Retraining Request: Based on model performance feedback and monitoring data, model retraining can be triggered if the model's performance does not meet the requirements.

-	Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

Actor is a function that receives the output from the Model Inference function or Agentic AI, and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

-	Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

Agentic AI refers to an autonomous, goal-oriented functional framework that empowers AI agents to perceive, reason, plan, and execute actions in dynamic environments, particularly complex network systems. Agentic AI integrates advanced capabilities for model training and inference to enable real-time perception of the network environment. These models are grounded in a knowledge base and prior knowledge. Given that, Agentic AI can iteratively dynamically refine their decision-making, adapt to uncertainties, and orchestrate multi-entity collaborations to fulfil the complex task.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>