# 3GPP TSG-RAN WG2 Meeting #130				             	         R3-258310

# Dallas, US, 17 – 21, November 2025

Source:	CATT

Title:	(TP to BL pCR for TR 38.765) Discussion on signalling procedure between RAN and CN on ISAC

Agenda Item:	13.3

Document for:	Approval

1 Introduction

In October 2025, the first RAN3 discussion on Integrated Sensing and Communication (ISAC) was held in the RAN3#129bis meeting.

The necessary procedures of ISAC are described in agenda [1] as follow:

Procedures to support sensing: - a class 1 sensing procedure including sensing request/response/failure message - a class 2 sensing report procedure including sensing report message. - gNB/CN initiated sensing termination procedure(s), FFS on the details.

A BL pCR of ISAC including network architecture and RAN-CN procedures and signalling is provided in the last meeting. In this paper, we will continue the discussion on the signalling procedure between RAN and CN. Section 2 gives the details discussion, Section 3 summarizes the conclusion of this paper, and followed by Annex that provides pCR.

2 Discussion

2.1 Information in Sensing Request

In TR 23.700-14 [2] which is updated in the last SA2 meeting, the agreed principles for SE (gNB) selection are copied as below:

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area. NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

According to SA2 principles, the SE selection is performed by the SF based on the sensing service request and the information of gNBs. The sensing service request may include the requested service type, target sensing area, required sensing QoS and sensing results exposure type (one-time, periodic, or event-trigger).

OAM can configure the SF with information of the gNBs which support sensing. The information of gNB may include:

- Supported service type (e.g. motion monitoring, object detection or tracking, environment monitoring, as defined in TS 22.137 [2])

- Supported sensing area

- Supported sensing QoS

Sensing QoS means the sensing performance to be satisfied, which is evaluated by the key metrics defined in TS 22.137, such as accuracy, false alarm rate, missed detection probability, sensing resolution, sensing latency and so on.

After SE selection, the SF should send the request to the selected gNB for the gNB starting sensing measurement to collect the sensing data.

The agreed principles for Sensing Data and the Associated Information Collection and Transport in TR 23.700-14 [2]:

The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor’s Note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

It can be observed that the gNB determines how to collect the sensing data based on the sensing service requirement from SF. Thus, SF needs to send the Sensing Request for a specific sensing service to convey the requirement of the sensing service to the gNB, for gNB to collect the data for detected object/target that fulfills the service requirement.

The Sensing request may include following information:

- Requested service type, indicating the sensing object type, e.g., motion monitoring, UAV object detection/tracking or environment monitoring)

This information can help the gNB to filter the unnecessary data for the detected object(s) not required.

- Requested sensing area

The gNB can use this information to decide the scope of area to perform sensing measurement and collect sensing data.

- Requested sensing QoS, representing the performance metrics which should be fulfilled for the gNB to detect object/target.

The sensing QoS includes the parameters, such as accuracy, false alarm rate, missed detection probability, sensing resolution, sensing latency and so on. The gNB can take into account this information for allocating the sensing radio resources. The definition and description need RAN1 input.

- Requested sensing report mode, indicating how the gNB reports sensing data for the detected object/target.

This information is corresponding to the requirement of sensing results exposure.

Proposal 1:	The information in the Sensing Request message which is sent from SF to gNB may include:

Requested service type,

Requested sensing area,

Requested sensing QoS,

Requested sensing report mode.

Sensing report mode

There are three kinds of report modes:

Mode 1: One-time report

For this report mode, the gNB can include the sensing report in the Sensing Response message.

Mode 2: Periodic reporting

For this report mode, the gNB should be indicated with the time interval (e.g. 1s, 10s, 1 min) between two successive reporting, and the reporting number may also be configured.

Mode 3: Event-triggered reporting

For this report mode, the gNB is indicated about the condition to send the sensing data report. For example:

-	A new object (e.g. drone) is detected or disappears in the target sensing area.

-	Mobility is detected for an object in the target sensing area.

-	The velocity of an object in the target sensing area is above a defined speed threshold.

Proposal 2:	The requested sensing report mode indicated by the Sensing Request message includes one-time report, periodic report and event-trigged report.

Sensing area

Theres are potentially two ways to identify the sensing area:

Option 1: Using sensing area ID

The SF should know the physical area of each area ID, as well as the list of area ID(s) which the gNB supports. And the gNB is configured with the mapping relation between the cell(s)/TRP(s) and each area ID which the gNB supports.

Option 2: Using Cell ID or TAI

For this option, the sensing area is aligned with the cell boundary. The SF should know the physical area for the serving cell(s) of the gNB.

In our opinion, the sensing coverage of the gNB may not be the same as the coverage of the serving cell(s). The area specific for sensing is more appropriate. Therefore, sensing area ID is preferred.

RAN3 can discuss which option to use. Note that the SF does not indicate the information for TRP(s) which the gNB uses for sensing measurement in both options, the TRP(s) is up to the gNB’s determination.

Proposal 3:	The requested sensing area indicated by the Sensing Request can be identified by sensing area ID or Cell ID/TAI.

A SF may receive form the service consumer the requests of different service with different requirements, for example, with different sensing QoS or different sensing results exposure type, thus the SF may initiate multiple sensing reporting procedures to one gNB for different sensing service with different requirements. That would lead to paralleled on-going sensing reporting procedures between SF and one gNB. The the Sensing Request should include a correlation ID to identify the specific sensing service to the gNB, so that the gNB can include the same correlation ID in the corresponding Sensing Response and Sending Report.

Proposal 4:	The Sensing Request can include a correlation ID to identify the sensing service between the SF and the gNB.

A TP to the BL pCR is provided in the Annex to capture above proposals.

2.2 Sensing procedures

In the last meeting, a general message flow for sensing reporting procedure was introduced in the BL pCR. However, the details have not been discussed.

Figure 1. Message flow for sensing reporting

Based on the Sensing Request from the SF, the gNB can decide whether sensing for the relevant service can be conducted. If the sensing for the service can be carried out, the gNB responds to the SF with a Sensing Response message. Whereas, if the sensing cannot be conducted, for example, when the requested sensing area or requested sensing QoS is not supported or other reasons, the gNB should send a Sensing Failure message to the SF.

Proposal 5:	The gNB can send a Sensing Failure message to the SF in case sensing for the service indicated by the Sensing Request cannot be conducted.

The Sensing Response can include the sensing data for the detected object(s) if one-time report is indicated in the Sensing Request for the sensing service.

Proposal 6:	The gNB sends the Sensing Response to indicate acceptance for the Sensing Request, and the Sensing Response can include a one-time report to the SF.

For the periodic report or event-triggered report as requested by the Sensing Request, the Sensing Report can be used by the gNB to deliver the report of sensing data periodically or eventually.

Proposal 7:	The Sensing Report is used by the gNB to send the sensing data to SF periodically or eventually, as requested by the Sensing Request.

According to the key Issue #2 in TR 23.700-14 [2], the end-to-end procedure should support revocation of an on-going sensing service. For the periodic report or event-triggered report, the SF can initiate the sensing release to the gNB for an on-going sensing service, e.g., upon the request from the sensing consumer.

In addition, the gNB can also decide to terminate the on-going sensing for a service due to some reason, and indicate the release of sensing to the SF.

Proposal 8:	The SF can initiate the sensing release to the gNB for an on-going sensing service and vice versa.

The TP in Annex provides the revision to the sensing reporting message flow to capture above proposals.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1:	The information in the Sensing Request message which is sent from SF to gNB may include:

Requested service type,

Requested sensing area,

Requested sensing QoS,

Requested sensing report mode.

Proposal 2:	The requested sensing report mode indicated by the Sensing Request message includes one-time report, periodic report and event-trigged report.

Proposal 3:	The requested sensing area indicated by the Sensing Request can be identified by sensing area ID or Cell ID/TAI.

Proposal 4:	The Sensing Request can include a correlation ID to identify the sensing service between the SF and the gNB.

Proposal 5:	The gNB can send a Sensing Failure message to the SF in case sensing for the service indicated by the Sensing Request cannot be conducted.

Proposal 6:	The gNB sends the Sensing Response to indicate acceptance for the Sensing Request, and the Sensing Response can include a one-time report to the SF.

Proposal 7:	The Sensing Report is used by the gNB to send the sensing data to SF periodically or eventually, as requested by the Sensing Request.

Proposal 8:	The SF can initiate the sensing release to the gNB for an on-going sensing service and vice versa.

4 Reference

- RAN3#129bis meeting minutes, RAN3 Chair

- 3GPP TR 23.700-14, Study on Integrated Sensing and Communication, V0.1.0 (2025-10)

- R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR, China Telecom, Xiaomi, RAN#129bis

5 Annex (TP to BL pCR for TR 38.765)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.1	Information exchanged between SF and RAN

### 8.1.1	Sensing Reporting

The Sensing Request can be sent by the SF to a gNB for a specific sensing service. The Sensing Request may include the following:

- Correlation ID, which is used to identify the sensing service between the SF and the gNB.

- Requested service type (e.g., UAV object detection or tracking), which is used to indicate the required sensing object.

- Requested sensing area, which is used to indicate the scope of area to perform sensing and collect data for the detected objects.

The sensing area may be identified by sensing area ID, Cell ID or TAI.

- Requested sensing QoS, which indicates the metrics which should be fulfilled for the gNB to detect the object.

NOTE 1:	The definition of sensing QoS is FFS.

- Requested sensing report mode, which indicates how gNB reports the sensing data for the detected objects.

The sensing report mode may include one-time report, periodic report and event triggered report. For the periodic report, the time interval (e.g. 1s, 10s, 1 min) between two successive reports can be configured, and the reports number may also be configured to the gNB. The event-triggered report is used to indicate the condition for the gNB to send the sensing data.

The Sensing Response can be sent from the gNB to the SF for acceptance to the Sensing Request for the sensing service. The Sensing Response can include a one-time report of the sensing data for the detected object(s).

The Sensing Report can be sent by the gNB to the SF for delivering the sensing data for the detected object(s) periodically or eventually.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing request to the gNB.

2.	The gNB sends sensing response to the SF.

NOTE 1: In Step 2, the gNB may instead send Sensing Failure to the SF indicating that the gNB cannot conduct the sensing service.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

NOTE 2: Step 3 may happen periodically or be triggered by event, as requested in Step 1.

NOTE 3: If SF/gNB needs, the SF/gNB can send Sensing Release to the gNB/SF.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<