3GPP TSG-RAN WG3 Meeting #130	R3-258325

Dallas, US, 17 - 21 November 2025

Title:	Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM

Agenda Item:	9.2.5

Source:	Huawei, NEC,Jio Platforms, CMCC, China Telecom, Lenovo

Document for:	Discussion

- Introduction

In the Rel-19 new feature, inter-CU LTM, it was agreed that the CSI report configuration for early CSI acquisition is provided from the candidate gNB to the source gNB in the LTM CONFIGURATION UPDATE ACKNOLEDGE message.

There are still some issues that need further clarification for the transfer of CSI report configuration for early CSI acquisition.

- Whether an explicit indicator is needed or not in the LTM CONFIGURATION UPDATE message to request the candidate gNB provide the demanded information?

- Procedural text is missing in Xn on how and when the candidate gNB shall include the CSI report configuration for early CSI acquisition IE in the LTM CONFIGURATION UPDATE ACKNOLEDGE message

This contribution provides further clarifications and proposals for above issues.

2	Discussion

In inter-CU LTM, separate requests in the HANDOVER REQEUST message are defined for the source gNB to acquire the CSI-RS resources for L1 measurement and for CSI acquisition. In response in the HANDOVER REQEUST ACKNOLEDGE message, the candidate gNB also returns the CSI-RS resources for L1 measurement and for CSI acquisition separately.

For support of subsequent LTM, a second round LTM preparation procedure, namely LTM Configuration Update is introduced for the source gNB to send the common CSI resource configuration (the combination of CSI-RS resources from all candidate cells in case of CSI-RS) to the candidate gNB. And the candidate gNB may also utilize this new procedure to return the CSI report configuration for L1 measurement encoded in the LTM Candidate Configuration IE.

In Handover Request: >Request for CSI-RS Resource Configuration for L1 Measurements; O; ENUMERATED (true, …); –. >Request for CSI-RS Resource Configuration for Early CSI Acquisition; O; ENUMERATED (true, …); –. In Handover Request Ack: >CSI-RS Resource Configuration for L1 Measurements; O; 9.2.3.xx8; –. >CSI-RS Resource Configuration for Early CSI Acquisition; O; 9.2.3.xx8; –. In LTM Configuration Update: >CSI Resource Configuration; O; 9.2.3.xx7; –. In LTM Configuration Update Ack: >>LTM Candidate Configuration; M; OCTET STRING; Includes the ltm-CandidateConfig as defined in TS 38.331 [10].; –. >>Complete Candidate Configuration Indicator; O; ENUMERATED (complete, ...); –. >>CSI-RS Report Configuration for Early CSI Acquisition; O; OCTET STRING; Includes the ltm-CSI-ReportConfig as defined in TS 38.331 [10].; –.

The CSI-RS Report Configuration for Early CSI Acquisition IE (typo, should be CSI Report Configuration for Early CSI Acquisition) referencing to the ltm-CSI-ReportConfig RRC IE contains the configuration of CSI reporting used by the UE in the targe cell after LTM cell switch. In which it associates a LTM-CSI-ResourceConfigId  identifying the CSI-RS resource to be measured. The LTM-CSI-ResourceConfigId is assigned by the source gNB.

For CSI-RS based L1 measurement, based on the common CSI resource configuration received in the LTM CONFIGURATION UPDATE message, the candidate gNB includes the CSI report configuration for L1 measurement in the LTM Candidate Configuration IE.

While for CSI acquisition, there are two options for the source gNB to request the candidate gNB to provide the CSI report configuration for early CSI Acquisition.

- Option 1: relying on the current CSI resource configuration IE in the LTM CONFIGURATION UPDATE message

- Option 2: introducing a new indicator

Option 1 is workable with the prerequisite that the current CSI resource configuration which is introduced for L1 measurement also contains the CSI-RS resource for CSI acquisition. We think that the prerequisite is valid as there is only one LTM-CSI-ResourceConfig definition in RRC specification for both L1 measurement and CSI acquisition.

The drawback of option 1 is that it brings some functional limitation. Option 1 requires that the subsequent LTM and CSI acquisition measurement shall always be configured together. Taking F1 interface as an example, the following procedural text was captured in TS 38.473 for intra-CU LTM.	Considerring the case that only CSI-RS resource for CSI acquisition is configured, the receiving gNB-DU will return a LTM CSI reporting configuration for L1 measurement mistakenly.

[In UE Context Setup] If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell. [In UE Context Modify] If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE. If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

However, there is no such kind of restriction in either RAN1 and RAN2 design. Meaning that subsequent LTM can be performed without CSI acquisition measurement, and vice versa. Any change on above procedueal texts may impact on Rel-18 intra-CU LTM.

Another drawback of option 1 is that it cannot use separate criticality to track any potential errors in the CSI acquisition related part in the common CSI Resource Configuration.

With above in mind, it is proposed to adopt option 2.

To simplify the design, option 2 does not need to introduce a new entire CSI resource configure IE. It is sufficient for the source gNB just to provide the LTM-CSI-ResourceConfigId  which will be encoded in the CSI report configuration by the candidate gNB. Because the candidate gNB already knows the CSI-RS resources that it allocated for CSI acquisition in the first round LTM preparation.

With the same logic, a second LTM-CSI-ResourceConfigId for the CSI-IM resource which is to be introduced is needed as well.

- In the LTM CONFIGURATION UPDATE message, the source gNB provides the per candidate cell LTM-CSI-ResourceConfigId to the candidate gNB as the request indicator for CSI report configuration for CSI acquisition.

- In the LTM CONFIGURATION UPDATE message, the source gNB provides the per candidate cell LTM-CSI-ResourceConfigId to the candidate gNB as the request indicator for CSI report configuration for CSI-IM measurements.

A tentative implementation for option 1 is also provided below for RAN3 comparison:

[In TS 38.473] [In UE Context Modify] If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration for L1 measurement in the CellGroupConfig IE, and/or to generate the LTM CSI reporting configuration for CSI acquisition and/or CS-IM measurement in the CSI Report Configuration for CSI Acquisition IE for the requested LTM candidate cell identified by the SpCell ID IE. If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell. […] Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for conditional LTM and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for conditional LTM. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE and may include the TA Remaining Information List IE in the UE CONTEXT MODIFICATION RESPONSE message. If the CSI Resource Configuration IE received in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, and if the CSI Resource Configuration IE does not contain the CSI-RS resources for L1 measurement, the gNB-DU shall not use it to generate the LTM CSI reporting configuration for L1 measurement in the CellGroupConfig IE.

- If option 2 is not agreable, RAN3 to discuss and agree on option 1.

Note that even with above interaction text, there still are IOT issues between a Rel-18 gNB-DU and a Rel-19 gNB-CU.

3	Conclusion

This document proposes:

Proposal 1:	In the LTM CONFIGURATION UPDATE message, the source gNB provides the per candidate cell LTM-CSI-ResourceConfigId to the candidate gNB as the request indicator for CSI report configuration for CSI acquisition.

Proposal 2:	In the LTM CONFIGURATION UPDATE message, the source gNB provides the per candidate cell LTM-CSI-ResourceConfigId to the candidate gNB as the request indicator for CSI report configuration for CSI-IM measurements.

Proposal 3:	If option 2 is not agreable, RAN3 to discuss and agree on option 1.

Note that option 1 may have IOT issues between a Rel-18 gNB-DU and a Rel-19 gNB-CU.

The XnAP CR and F1AP CR for option 2 are provided in [1] and [3].

The XnAP CR and F1AP CR for option 1 are provided in [2] and [4].

Reference

[1] R3-258326	Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM (Option 1)

[2] R3-258328	Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM (Option 2)

[3] R3-258341	Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM (Option 1)

[4] R3-258342	Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM (Option 2)