3GPP TSG-RAN WG3 #130	R3-258369

Dallas, USA, 17th-21st November 2025

Agenda Item:	12.2.1

Source:	Lenovo

Title: 		(pCR to TR 38.745) Discussion on multi-hop UE trajectory

Document for:	pCR

# Introduction

In this paper, we discuss the use case of multi-hop UE trajectory.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order. The following solutions are considered for supporting multi-hop UE trajectory: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. Focus on cell-based granularity for measured and predicted multi-hop UE trajectory. Study multi-hop UE trajectory for L3 HO. The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network. The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop. Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message? Configuration of measurement feedback: - Data collection initiate procedure How to transfer measurement feedback: • Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1) • Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1) • Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1) • Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

# Discussion

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

With respect to how the multi-hop predicted UE trajectory is transferred, giving that the source gNB only knows if the predicted multi-hop UE trajectory is useful for the next hop target gNB when the handover is triggered, it is natural to follow legacy and transfer the predicted multi-hop UE trajectory using Handover Request message during subsequent inter-gNB handover.

The predicted multi-hop UE trajectory is transferred using Handover Request message during subsequent inter-gNB handover.

Besides, in the subsequent inter-gNB handover, it is upon the decision of the concerned new source gNB whether it will send the predicted multi-hop UE trajectory to the next hop target gNB. It may not send anything if, for example, the multi-hop UE trajectory received from previous hop gNB becomes totally wrong/invalid.

In addition, it is also upon the new source gNB if the multi-hop predicted UE trajectory sent to the next hop target gNB is different from the prediction received from the last hop gNB. For example, the new source gNB may have its own AIML model deployed capable of UE trajectory prediction and may produce a new/updated multi-hop UE trajectory and sent to the next hop target gNB.

For example, assuming RAN#2 owns Cell#1#2#3 and RAN#3 owns Cell#A#B#C,

- When RAN#1 handovers UE#1 to RAN#2, RAN#1 indicates to RAN#2 about the predicted UE trajectory: Cell#1 and Cell#A

- When RAN#2 handovers UE#1 further to RAN#3, RAN#2 may also send the predicted UE trajectory to RAN#3 with the following modification

  - Removing Cell#1 since it is no longer valid

  - Adding Cell#B, as RAN#2 predicts the UE may visit Cell#B after Cell#A

During subsequent inter-gNB handover, it is upon the concerned source gNB’s decision whether/what will be transferred to the next hop target gNB related to the predicted UE’s single/multi-hop trajectory.

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Regarding how to transfer the measurement feedback, Option 1&3 would require the initial source gNB to establish connection or initiate the data collection initiation procedure with each/final hop of gNB in advance, however, it is not realistic for the source gNB to know which gNBs UE will actually move to. Although it is possible to achieve based on source gNB’s prediction, it may become messy if the prediction is wrong.

Option 1 and Option 3 require the source gNB to trigger the data collection initiation procedure with the future gNB hop(s) before the handover happens, it will increase complexity if the UE does not move to the future gNB hop(s) as source gNB predicted.

Among the identified 4 options, Option 4 can be excluded following the agreement made last time that The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

To avoid impacting the core network, RAN3 confirms to not consider Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

On the other hand, we tend to believe that the multi-hop UE trajectory collection can mostly reuse the Rel18 one-hop UE trajectory collection approach with minimal standard impact (Option 2). A simple example is given in the figure below with high level procedure signaling.

RAN3 is suggested to support multi-hop UE trajectory collection based on the Rel18 one-hop UE trajectory collection approach, i.e., the actual UE trajectory is collected hop by hop via Xn interface (Option 2).

# Conclusion

Based on the discussion above, we observe:

Observation 1	Option 1 and Option 3 require the source gNB to trigger the data collection initiation procedure with the future gNB hop(s) before the handover happens, it will increase complexity if the UE does not move to the future gNB hop(s) as source gNB predicted.

Based on the discussion above, we propose:

Proposal 1	The predicted multi-hop UE trajectory is transferred using Handover Request message during subsequent inter-gNB handover.

Proposal 2	During subsequent inter-gNB handover, it is upon the concerned source gNB’s decision whether/what will be transferred to the next hop target gNB related to the predicted UE’s single/multi-hop trajectory.

Proposal 3	To avoid impacting the core network, RAN3 confirms to not consider Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Proposal 4	RAN3 is suggested to support multi-hop UE trajectory collection based on the Rel18 one-hop UE trajectory collection approach, i.e., the actual UE trajectory is collected hop by hop via Xn interface (Option 2).

# TP to TR 38.745

***************************** CHANGE STARTS *****************************

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor Note: Capture the description of use case

In Rel18 AI/ML based mobility optimization discussion, NG-RAN node is capable of predicting UE trajectory in the future. In order to collect enough ground truth data for the NG-RAN node to train or monitor the AI/ML model, the source NG-RAN node may request the target NG-RAN node to measure and report the UE trajectory back to the source NG-RAN node after the handover operation.

### 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

RAN3 understands that the single-hop UE trajectory collection approach supported in Rel18 can be extended to support the multiple-hop UE trajectory across gNBs with minimal standard impact, i.e., the actual UE trajectory is collected hop by hop via Xn interface. The detailed standard impact will be discussed in Rel19 normative phase.

An exemplary procedure with high level signalling flow is given below.

Step 1: NG-RAN node 1 triggers a Data Collection Reporting Initiation procedure to configure NG-RAN node 2 being prepared to provide UE trajectory measurement after handover.

Step 2: NG-RAN node 1 triggers the Handover Preparation procedure to handover the UE to NG-RAN node 2. NG-RAN node 1 also indicates NG-RAN node 2 to measure and report the UE trajectory after handover as configured in step 1.

Step 3: NG-RAN node 1 sends the handover command to UE

Step 4~6: NG-RAN node 2 may decide to further handover the UE to another NG-RAN node 3. NG-RAN node 2 may report the same Step 1 to Step 3 to request the UE trajectory measurement from NG-RAN node 3 after the handover.

Step 7: NG-RAN node 3 will measure the UE trajectory after UE is handed over to NG-RAN node 3 and report to NG-RAN node 2 as configured in step 4.

Step 8: Then NG-RAN node 2 will report the UE trajectory measurement, including both UE trajectory when UE is connected to NG-RAN node 2 and UE trajectory when UE is connected to NG-RAN node 3, to NG-RAN node 1 as configured in step 8.

***************************** CHANGE ENDS *****************************