3GPP TSG-RAN WG3 Meeting #130	R3-258371

Dallas, US, 17 – 21 November 2025

Agenda Item:	13.2

Source: 	Lenovo

Title: 	(TP to TR 38.765) Discussion on ISAC network architecture

Document for:	Other

- Introduction

A new SID for Integrated Sensing and Communication [1] has been approved to be discussed in Rel-20 and following detail objectives are identified for RAN3.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, following aspects are discussed for ISAC: connection between SF and Sensing gNB, protocol stacks for sensing, gNB-based mono-static sensing and CU-DU split architecture of sensing gNB.

- Discussion

Connection between SF and Sensing gNB

In SA2, there is another SI to discuss sensing, which was triggered before RAN3 and currently has some progress as specified in the TR 23.700-14 [2]. As agreed in SA2, a new Sensing Function (SF) is introduced to enable the 5GS system architecture to support sensing service, and both Direct Connectivity and Indirect Connectivity can be considered between the Sensing gNB and the SF.

-	Direct Connectivity: the SF communicates directly with the Sensing gNB

## 6.1 Solution #1: Support of Sensing operations with direct interfaces ### 6.1.0 High-level solution Principles The solution is based on the following general principles to support sensing service: - The Sensing Function performs a selection of the Sensing Entity(ies) based on the sensing service request. - The Sensing Function triggers each of the selected Sensing Entities to establish a direct connection with the Sensing Function, if the connection was not yet available. - The Sensing Function uses the direct connection to request the sensing service operation at the Sensing Entity(ies) and obtain the Sensing Data and Associated information from the Sensing Entity(ies).

-	Indirect Connectivity: the SF communicates indirectly with the Sensing gNB via an AMF

## 6.5 Solution #5: AMF-Inclusive Framework for 5GC Sensing ### 6.5.0 High-level solution Principles The high-level solution principles are as follows: - An AMF-inclusive framework where the AMF is responsible for: - Discovery and selection of sensing entities - Forwarding the sensing request to the sensing entities. - Introduction of a new network function for Sensing called Sensing Function (SF) which: - Authorizes the usage of sensing service and selects the relevant AMF(s) based on the sensing service request parameters. - Processes the sensing data to derive the sensing result. - The N1/N2 interface may need to be enhanced to support sensing data transmission.

For key issue 1: following principles have been agreed in SA2 for architecture for ISAC.

### 7.1.1 Agreed Principles for KI#1: System Architecture to Support Sensing To support KI#1 System Architecture to Support Sensing, the following principles are concluded: Principle 1: one new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. ### NOTE 5: Other KI’s interim conclusions will be aligned with KI#1 conclusions.

To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect connectivity for the Sensing gNB to communicate with the SF, and the SF can be split into the SCF and the SPF.

Proposal 1: To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect connectivity for the Sensing gNB to communicate with the SF, and the SF can be split into the SCF and the SPF.

Protocol Stacks for sensing

As for the protocol stacks between the Sensing gNB and the SF, for control plane, option #1 is to reference the protocol design for positioning, where a new protocol layer may be introduced for the transport of the sensing control information, e.g., sensing related configuration. For example, it is illustrated as XXAP as shown in the Figure 1 both for direct connectivity and indirect connectivity.

Direct Connectivity									 Indirect Connectivity

Figure 1: Option #1 for control plane protocol between Sensing gNB and SF

Another option #2 for control plane is to take the examples defined for A-IoT. In case of Direct Connectivity, the SF needs to transmit the sensing control information through a NGAP like protocol layer (illustrated as NXAP in Figure 2) established between the sensing gNB and the SF, or it can even reuse the NGAP. And In case of Indirectly Connectivity, the SF needs to transmit sensing control information to AMF via SBI interface and then the AMF sends the sensing control information to the sensing gNB through NGAP.

Direct Connectivity									 Indirect Connectivity

Figure 2: Option #2 for control plane protocol between Sensing gNB and SF

Both Option #1 and Option #2 may have impacts on the NGAP. However, Option #1, with additional architecture impacts, needs to introduce a new protocol specification. Considering R20 is the later phase the 5G, Option #2 is preferred to be used for control plane sensing information transport between the sensing gNB and the SF with less architecture impacts.

Proposal 2: Reuse NGAP or introduce a NGAP like protocol to transport control plane sensing information between sensing gNB and SF.

Proposal 3: Agree the protocol stacks for control plane in Figure 2 for both Direct Connectivity and Indirectly Connectivity between sensing gNB and SF.

As for the user plane, in case of direct connectivity, it is assumed that the Sensing gNB triggers a NG like interface (NX interface) establishment towards the SF. And the SF triggers data connection establishment towards the Sensing gNB. While for indirect connectivity, it is assumed that AMF notifies the Sensing Function about the existence of the Sensing gNB. Then the SF triggers data connection establishment towards the Sensing gNB. The protocol stack for user plane between the Sensing gNB and the SF is shown in Figure 3. Where a GTP-U/UDP/IP based protocol is used to align with other user plane protocol like NGAP/XnAP/F1AP for 5G. The sensing data at least includes the sensing report generated by the sensing gNB. While the format of sensing data, it requires further inputs from RAN1.

Figure 3: User plane protocol between Sensing gNB and SF

Proposal 4: Agree the protocol stack for user plane in Figure 3 between sensing gNB and SF, where the sensing DATA is encapsulated by GTPU/UDP/IP layers.

gNB monostatic sensing

As specified in the SID, R20 ISAC only considers gNB-based mono-static sensing for UAV sensing target use cases and no inter-gNB coordination will be studied for gNB bistatic sensing. For gNB-based mono-static sensing, the gNB which is selected as the sensing entity functions as both a Sensing Transmitter and a Sensing Receiver. In some cases, same TRP or same DU of the sensing gNB is selected as both the Sensing Transmitter and the Sensing Receiver. In some other cases, to alleviate the self-interference and avoid duplex issue on the sensing entity, different TRPs or different gNB-DUs of the sensing gNB may be selected as the Sensing Transmitter or the Sensing Receiver.

Proposal 5: For gNB-based mono-static sensing, same TRP or same DU of the sensing gNB may be selected as both the Sensing Transmitter and the Sensing Receiver.

Proposal 6: For gNB-based mono-static sensing, different TRPs or different gNB-DUs of the sensing gNB may be selected as a Sensing Transmitter or a Sensing Receiver.

CU-DU split architecture

A gNB may consist of a gNB-CU and one or more gNB-DU(s) in the current architecture. If the sensing procedure cannot be applied in the CU-DU split architecture, there are some limitations on the selection of sensing gNB as the sensing entity. In addition, CU-DU split architecture can achieve combination of sensing result from different DUs to improve sensing accuracy. And the case that different DUs as the Sensing Transmitter or the Sensing Receiver can also effectively reduce the interference of sensing signals.

Proposal 7: For gNB-based mono-static sensing, the sensing gNB supports CU-DU split architecture.

Sensing reporting

### 7.1.4 Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor’s Note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI’s interim conclusions will be aligned with KI#1 conclusions.

As a sensing entity, the sensing gNB at least supports the functionalities of sensing signal transmission and reception (or measurement). As agreed in SA2, the sensing gNB can determine how to perform the relevant sensing measurement to collect the Sensing data based on the received sensing service requirements from the SF. While for the sensing data processing, it might be completely processed by the SF. Alternatively, the sensing gNB needs to support the functionality to pre-process the sensing data and then sends the pre-processed data to the SF rather than to send the raw sensing measurement result to the SF. In this way, the volume of reported sensing data between the sensing gNB and the SF can be decreased. For the type of sensing reporting, the sensing gNB can at least support one time or periodical reporting based on the configuration from the SF.

Proposal 8: RAN3 to study whether a sensing gNB supports the functionality of pre-processing the sensing data.

Proposal 9: The sensing gNB supports one time or periodical reporting based on configuration from the SF.

# Conclusion

This contribution aims to discuss some architecture issues for 5G ISAC. And following observations and proposals are concluded.

Proposal 1: To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect connectivity for the Sensing gNB to communicate with the SF, and the SF can be split into the SCF and the SPF.

Proposal 2: Reuse NGAP or introduce a NGAP like protocol to transport control plane sensing information between sensing gNB and SF.

Proposal 3: Agree the protocol stacks for control plane in Figure 2 for both Direct Connectivity and Indirectly Connectivity between sensing gNB and SF.

Proposal 4: Agree the protocol stack for user plane in Figure 3 between sensing gNB and SF, where the sensing DATA is encapsulated by GTPU/UDP/IP layers.

Proposal 5: For gNB-based mono-static sensing, same TRP or same DU of the sensing gNB may be selected as both the Sensing Transmitter and the Sensing Receiver.

Proposal 6: For gNB-based mono-static sensing, different TRPs or different gNB-DUs of the sensing gNB may be selected as a Sensing Transmitter or a Sensing Receiver.

Proposal 7: For gNB-based mono-static sensing, the sensing gNB supports CU-DU split architecture.

Proposal 8: RAN3 to study whether a sensing gNB supports the functionality of pre-processing the sensing data.

Proposal 9: The sensing gNB supports one time or periodical reporting based on configuration from the SF.

# Reference

- RP-252819. Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR.

- TR 23.700-14. Study on Integrated Sensing and Communication; Stage 2 (Release 20).

# TP to TR 38.765

# 7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

For the Sensing gNB, architecture and protocol aspects of split RAN architecture could be studied. For gNB-based monostatic sensing, same/different TRP(s) or same/different DU(s) of the sensing gNB may be selected as the Sensing Transmitter and the Sensing Receiver.

Figure 7.2 shows protocol stack examples of control plane protocol between the Sensing gNB and the SF direct or via the AMF.

Figure 7.2 Control plane protocol between Sensing gNB and SF

For direct connectivity, the NXAP is terminated at the Sensing gNB and the SF. The signalling transport for NXAP at the Sensing gNB is SCTP/IP. NXAP can be realised by including sensing information in NGAP.

For indirect connectivity, the sensing information is included in the NGAP between the Sensing gNB and the AMF, and transported via SBI between the AMF and the SF.

Figure 7.3 shows protocol stack examples of user plane protocol between the Sensing gNB and the SF. There is a direct data connection between the Sensing gNB and the SF for both direct or indirect logical architecture between the Sensing gNB and the SF.

Figure 7.3 User plane protocol between Sensing gNB and SF