3GPP TSG-RAN WG3 Meeting #130	R3-258373

Dallas, USA, 17 – 21 November 2025

Agenda item:	14.2

Source: 	Lenovo

Title: 	On UE Reader Selection in Topology 2

Document for:	Approval

- 1. Introduction

The Rel-20 work item on solutions for A-IOT in NR phase 2 has the following objectives [1]:

- Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility.

This paper discusses the open issues on UE reader selection in Topology 2.

- 2. Discussion

For UE reader selection, there are three scenarios needs to be considered

- Scenario 1: AF requests a specific UE reader for AIoT service operation.

- Scenario 2: Any authorized UEs can be selected as UE reader within a area.

- Scenario 3: Fixed deployed UE reader.

In Scenario 1, a third-party AF intends to use specific UE reader(s) for AIoT service operations and includes the UE reader information (i.e., UE reader ID) in the service operation request sent to the core network. In Scenario 2, any authorized UE can be selected to act as a UE reader within a AIoT area.  In Scenario 3, the UE reader is assumed to be deployed as a fixed device within a specific area.

The scenario 1 and scenario 3 can be treated as UE-specific inventory request and scenario 3 is a area based inventory request.

Our understanding all the three scenarios have been confirmed by SA2 during the study. SA2 has concluded in the TR 23.700-13 as:

8.3.4 Principles on the procedures to support AIoT services To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported: - Inventory Procedure. - Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages. There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1). All the procedures follow have the following steps: 1. The AF makes a service request to the NEF, including parameters as defined in clause 8.3.5. 2. The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF. 3. For the requested operation the AIOTF: 1). Performs initial reader selection by either: - The AF provide information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader. NOTE 1: Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation. - If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader. - If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc. If no readers can be selected then the request is rejected.

Proposal 1: Both UE reader specific inventory request and area-based inventory request should be supported.

For the UE reader specific inventory, the UE reader ID needs to be provided to the NG-RAN node from AIOTF. Since the NGAP AIOT information between the AIOTF and NG-RAN is transparent to the AMF, which UE reader ID should AIOTF use to communicate with the NG-RAN node will become a problem. For example, when AF requests a specific UE reader in the form of GPSI, after AIOTF or NEF resolves it into SUPI (via UDM), AIOTF cannot directly use SUPI as the UE reader ID and send it to the NG-RAN since AMF in the between cannot read and let alone translate (e.g., translate SUPI into the ID that NG-RAN knows), and NG-RAN certainly cannot understand the SUPI. On the other hand, if the UE ID is outside the AIOT NGAP information and readable to the AMF, the AMF between the AIOTF and NG-RAN may not always be the AMF that currently serves the UE (i.e., serving AMF). Under this case, the middle/relay AMF may not be able to translate the UE ID (SUPI) into the ID that RAN can understand (e.g., NGAP UE ID, 5G-TMSI), and it will also cause the misalignment of the UE ID between NG-RAN and AIOTF.

To solve this problem, the UE reader ID needs to be aligned at both the NG-RAN and AIOTF, understood and used by both sides to exchange the messages that related to UE reader. What’s more, in order to forward the message towards the UE, the current serving NG-RAN, needs to be firstly discovered, especially considering the UE mobility. Therefore, it is proposed with the following 2 options:

Option 1: During UE reader registration procedure, the AMF actively sends the authorized UE reader information to the AIOTF, that contains the UE reader ID, the serving NG-RAN information (e.g., NG-RAN ID) and reader allowed/authorization indication. The NGAP UE ID is reused as the UE reader ID and sent to the AIOTF, which is associated with SUPI. Based on this, later on when a UE reader is determined (either by AIOTF, or indicated by AF), instead of using SUPI, the AIOTF will check the stored information, use the NGAP UE ID as the UE reader ID and send it to the serving NG-RAN of the UE.

Option 2: AIOTF obtains the serving AMF of the UE from UDM using UE ID and sends the UE reader related information retrieval request to the serving AMF, when a UE reader is determined (either by AIOTF, or indicated by AF). The serving AMF will respond with UE reader ID (AMF/RAN UE NGAP ID) associated with SUPI, the serving NG-RAN information (e.g., NG-RAN ID) and UE reader allowed/authorization indication to the AIOTF.

Based on the options listed above, the AIOTF can send the AIOT service request to the UE via the serving NG-RAN, that contains NGAP AIoT information (including UE reader ID (AMF/RAN UE NGAP ID)), NG-RAN ID and AIOTF ID.

Proposal 2: For UE reader specific inventory, the AIOTF sends the UE reader ID directly to the NG-RAN node. The NG-RAN node shall use the UE reader the inventory procedure.

Proposal 3: The AMF/RAN UE NGAP ID is used as the UE reader ID.

For area based inventory request, any authorized UE can be selected to act as a UE reader in the AIOTF area. Since the UE reader may be a regular UE, radio conditions must be taken into account in such cases. In other words, the final selection of the UE reader should reside with the NG-RAN node, as it is aware of each UE's radio conditions.

As the NG-RAN node is already aware of UE reader authorization status - for example, through the Initial UE Context Setup Request or UE Context Modification Request messages. For example, in the inventory request, the AIOTF needs to provide the inventory area information to the NG-RAN node. With the inventory area, the NG-RAN node selects a appropriate UE reader among the authorized UEs.

For the area based inventory, we think that the A-IOT area ID can be reused:

-	the A-IoT CN provides the A-IoT Area id list to the NG-RAN node in Inventory Request;

-	the NG-RAN node is configured with the mapping relationship between the A-IoT Areas and the NR Uu Cells;

-	the NG-RAN node selects the readers within the NR Uu Cells associated with the A-IoT Areas;

Proposal 4: For area-based inventory, AIOTF provides the A-IOT Area ID in the inventory request message to NG-RAN node

Proposal 5: NG-RAN node selects the UE readers among the authorized UEs within the inventory area associated with the A-IOT Area ID .

In last meeting, it was discussed that in case the core network provides an empty requested service area info, the gNB selects among all the served connected mode UE readers. Our understanding is that the gNB needs to consider the radio condition, mobility status and other aspects when select the UE readers.

Proposal 6: NG-RAN node may also consider radio condition and other aspects when selecting of the UE readers, even though a empty inventory area info is provided from AIOTF.

- 3. Conclusion

In this contribution, we discuss the potential issues to support A-IOT topology 2.

Proposal 1: The A-IOT related messages and IEs defined in Rel-19 NGAP, specifically for the Topology 1 indirect scenario, can be reused for Topology 2.

Proposal 2: Introduce AIOT UE reader authorization and revocation indications in NG-AP e.g., in UE associated signalling.

Proposal 3: For the scenario that AF requests a specific UE reader for AIOT service operation, how to convey the requested UE reader information from the AIOTF to the NG-RAN node needs to be addressed.

Proposal 4: For the scenario that any authorized UEs can be selected as UE reader, the NG-RAN node selects the UE readers among the authorized UEs within the inventory area.

Proposal 5: For the scenario that UE is deployed as fixed UE reader, the AIOTF can select the UE reader according to the pre-configuration information of the UE reader.

Proposal 6: RAN3 to discuss how service continuity can be maintained during the switch of UE readers for the same inventory session.

Proposal 7: RAN3 to discuss whether source the NG-RAN node should provide UE reader information to the target NG-RAN node during handover preparation.

- Reference

[1] RP-252867. Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3. ZTE Corporation, NEC.

[2] TS 38.300