3GPP TSG-RAN WG3 Meeting #130	R3-258389

Dallas, US, 17 – 21 November 2025

Agenda Item:	10.5.1

Source:	Fujitsu

Title:	Discussion on 6G AI/ML Network Energy Savings use case

Document for:	Discussion

1. Background

The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

In the RAN3 #129bis meeting, RAN3 discussed about 6G AI RAN use cases and the following aspects were captured:

In this contribution, we describe some aspects for further discussion on AI/ML based network energy savings.

2. Discussion

In 5G-Advanced, RAN3 discussed AI/ML based network energy savings (NES), e.g., prediction and finer granularity of energy cost at NG-RAN nodes but there was no consensus even though importance of the energy efficiency was obvious. In our understanding, one of the causes is difficulty to align common promising information among multiple nodes.

If RAN3 starts the same discussion in 6G, we expect the same dead lock happens unless new environments or assumptions are introduced. Therefore, it is desirable firstly to discuss agreeable scenarios where AI/ML based methodologies including prediction are useful.

Proposal 1 RAN3 should identify potential useful scenarios for 6G AI/ML based NES.

Generally, 6G NES features can be discussed in multiple WGs. Since RAN1 is currently studying evaluation methodology of energy efficiency in terms of signal transmission and UE power consumption, discussion for NES in terms of signal transmission reduction can be led by RAN1. Since RAN1 does not currently think about AI/ML application on NES, it is not clear whether AI/ML application on RAN1 NES is considered or not. Since overall framework is still not clear, RAN3 can wait for progress of the discussion in RAN1, to confirm if there are any clarification/discussion is necessary in RAN3.

Observation 1: For AI/ML based NES in terms of signal transmission reduction, RAN3 may discuss it if there are any requests from RAN1 based on output of the study on energy efficiency.

Instead, RAN3 can firstly focus on other aspects for energy efficient RAN design, e.g., energy saving by cell management. As an example, prediction of the daily change in traffic load or energy consumption on cells can help e.g., cell management and cloud resource optimization for base stations.

Proposal 2: AI/ML based cell management for NES can be considered as one of the application scenarios.

Although actual definition of the management of cells in 6G should be discussed across multiple WGs, we consider there can be at least an operation of turning on/off cells in multiple granularities. For example,

- Base station (BS) wise control

- Cell wise control

- Beam wise control

In case of base station wise control, cells under the base station are simultaneously turned on/off. Although such control is possible based on inter-BS IF or Core-RAN IF, applicability of such operation is not high in real deployment due to the big impact on coverage. In case of cell wise control, BS can turn on/off cells under its control. Such coordination can be possible based on messages among RAN internal nodes in disaggregated BS architecture in addition to the previously mentioned IFs. Lastly, regarding beam/frequency wise control, a RAN node may further control e.g., mapping of beams in space and frequency domains

Since RAN internal function split is currently under discussion, it is difficult to clearly state impacted nodes. However, relevant RAN nodes can be generally different depending on the granularities. In addition, appropriate operation periodicity can be also different for the granularities. Thus, RAN3 can firstly discuss target granularities for AI/ML based energy optimization.

Observation 2: Depending on the cell turn on/off time, frequency and spatial granularity, impacted nodes can be different.

Proposal 3 For AI/ML for RAN NES, RAN3 to discuss the granularity of cell turn on/off capability at 6G base stations.

3. Conclusion

In this paper, we further discussed potential target scenarios for network energy savings. Proposals and observations are as the following:

Proposal 1 RAN3 should identify potential useful scenarios for 6G AI/ML based NES.

Observation 1: For NES in terms of signal transmission reduction, RAN3 may discuss it if there are any requests from RAN1 based on output of the study on energy efficiency.

Proposal 2: AI/ML based cell management for NES can be considered as one of the application scenarios.

Observation 2: Depending on the cell turn on/off time, frequency and spatial granularity, impacted nodes can be different.

Proposal 3 For AI/ML for RAN NES, RAN3 to discuss the granularity of cell turn on/off capability at 6G base stations.