3GPP TSG-RAN WG3 Meeting #130                                                        R3-258409

Dallas, Texas, USA, 17-21 November 2025

Title:	Orange views on RAN internal functional split in 6G

Agenda item:	10.4

Source:	Orange

Document for:	Discussion

- Introduction

One of the objectives in the approved 6G SID [1] relates to the RAN architecture and includes the following points:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

During the RAN3 #129bis meeting, extensive discussions have been conducted regarding the CU‑DU split topic, the way forward was captured in the Chair’s note as follows [2]

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

To be continued...

In this contribution, we would like to present our views as an operator on the CU-DU split topic and highlight the concerns related to its deployment.

- Discussion

3GPP defines the 5G NR architecture with an optional functional split between the CU and DU. The CU typically handles higher-layer functions and centralized control, while the DU manages real-time, lower-layer processing closer to the radio units.  Although previous studies present potential benefits of this architecture, the deployment of CU-DU split in 5G networks remains very limited. In the following, we present the practical concerns from an operator’s perspective related to the deployment of CU-DU split.

Concern #1: Limited Resource Pooling Gain in Practice

One of the advantages frequently highlighted of the CU-DU split architecture is the potential for resource pooling [3]. The idea is that by centralizing control functions, network resources can be more efficiently shared and allocated across multiple gNBs. However, in practice, the CU accounts for only a very small proportion of the total number of servers compared to the DU, e.g., around 10%, which limits the actual resource pooling gain through CU centralization. Moreover, with latest single server configurations, spare capacity allows the collocation of the CUs and DUs on the same server without additional cost, e.g. on distributed sites, thereby further reducing the potential benefit from centralized resource pooling.

Observation 1: In practice, the CU accounts for only a very small proportion of the total number of servers compared to the DU, which limits the actual resource pooling gain through CU centralization.

Concern #2: Substantially Increased OPEX and CAPEX for Resilience Solutions

Another claimed benefit of the CU-DU split is its deployment flexibility [3]. Specifically, the CU can be located in a centralized or cloud environment, while the DUs can be deployed closer to the network edge. In this setup, a centralized CU controls a large number of DUs, each corresponding to a group of cells.

Observation 2: In the CU-DU split deployment, a centralized CU controls a large number of DUs, each corresponding to a group of cells.

The impact of a CU failure largely depends on the network architecture. In the non-split architecture, where each gNB is managed by its dedicated CU, a CU failure only disrupts the services of the gNB under that specific CU. In such a case, the overall service in the area may be partially maintained, if neighboring gNBs can compensate by providing fallback coverage and capacity. However, in the deployment with a centralized CU managing a large number of DUs, a CU failure can have significantly more severe consequences. Such a failure will cause a widespread service outage across the entire coverage area served by the DUs connected to this failed CU. Restoring service quickly through neighboring gNBs is unlikely, as they are also impacted by the failure.

Observation 3: In the non-split architecture, where each gNB is managed by its dedicated CU, a CU failure only disrupts the services of the gNB under that specific CU, the overall service in the area may be partially maintained. However, in the deployment with a centralized CU managing a large number of DUs, a CU failure can cause a widespread service outage across the entire coverage area served by the DUs connected to this failed CU. Restoring service quickly through neighboring gNBs is unlikely, as they are also impacted by the failure.

To enhance network resilience and ensure service continuity in the CU-DU split deployments, redundancy mechanisms are essential to mitigate the impact of potential CU failures. Two primary redundancy schemes are usually required:

- Hardware redundancy: typically, this involves deploying N+M servers within the same data center to ensure seamless service availability in case of localized hardware or software failures. The number of additional backup servers depends on the desired level of redundancy and the capacity of each server. However, hardware redundancy solution alone is insufficient to protect against site level failures.

- Geographical redundancy:  a non-collocated backup data center is provisioned to take over operations in the event of major site-level disruptions, such as complete power outages or transmission link failures. In this case, duplication of all connections between the CU and its managed DUs is necessary. This redundancy approach involves a significant number of additional fiber or transport links. It also requires investment in redundancy capable switches, along with more complex network management and configuration to properly handle the redundant links.

Consequently, the resilience solutions in the CU-DU split deployment substantially increases both OPEX and CAPEX costs for operators.

Observation 4: Resilience solutions in the CU-DU split deployment substantially increase both OPEX and CAPEX costs for operators.

Concern #3: Unnecessary Complexities of Multi-Vendor CU-DU Integrations

Multi-vendor interoperability is often cited as another motivation for deploying the CU-DU split architecture. However, until now, only a very limited number of commercial multi-vendor implementations have been realized in the industry. This is primarily due to different interpretations of split interface specifications and variations in supported features. Moreover, integration, testing and fault management across multiple vendors require extensive coordination and validation efforts. In addition, operators are required to pay for, manage and maintain two separate software systems. As a result, in practice, multi-vendor CU-DU implementations are often seen as adding unnecessary complexities with very minor gains.

Observation 5: In practice, multi-vendor CU-DU implementations are often seen as adding unnecessary complexities with very minor gains.

- Conclusion

We have the following observations:

Observation 1: In practice, the CU accounts for only a very small proportion of the total number of servers compared to the DU, which limits the actual resource pooling gain through CU centralization.

Observation 2: In the CU-DU split deployment, a centralized CU controls a large number of DUs, each corresponding to a group of cells.

Observation 3: In the non-split architecture, where each gNB is managed by its dedicated CU, a CU failure only disrupts the services of the gNB under that specific CU, the overall service in the area may be partially maintained. However, in the deployment with a centralized CU managing a large number of DUs, a CU failure can cause a widespread service outage across the entire coverage area served by the DUs connected to this failed CU. Restoring service quickly through neighboring gNBs is unlikely, as they are also impacted by the failure.

Observation 4: Resilience solutions in the CU-DU split deployment substantially increase both OPEX and CAPEX costs for operators.

Observation 5: In practice, multi-vendor CU-DU implementations are often seen as adding unnecessary complexities with very minor gains.

- References

[1] RP-252912 ‘Revised SID: Study on 6G Radio’, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] Chair Notes RAN3 #129bis

[3] R3-256596, ‘Discussion on RAN internal architecture’, Samsung, Verizon, NTT DoCoMo, Rakuten