3GPP TSG-RAN WG3 Meeting # 130	R3-258414

Dallas, USA,  17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1588 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on TNLA handling during Xn handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To clarify the NG-RAN node behavior when it receives Signalling TNL association address at source NG-C side IE in Handover Request message or Retrieve UE Context Request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure text in corresponding clauses to clarify NG-RAN node behavior as follows: When the target NG-RAN node has an available TNL association towards the TNL address as indicated by the Signalling TNL association address at source NG-C side IE, or when the target NG-RAN node does not have a TNL association towards the TNL address of the AMF received from the source NG-RAN node and establishes a TNL association towards it, the target NG-RAN node should select the TNL association to create an NGAP UE TNLA binding for the UE. Otherwise, the target NG-RAN node should select other available TNL association towards the same AMF or an AMF from the same AMF set to create an NGAP UE TNLA binding for the UE. Impact assesment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only add the missing behavior. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. It may cause IOT issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: This CR is revised for re-submission. |  |  |  |  |  |  |  |  |

Start of Change

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the LTM CFRA Resource Information IE is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [35].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

When the target NG-RAN node has an available TNL association towards the TNL address as indicated by the Signalling TNL association address at source NG-C side IE, or when the target NG-RAN node does not have a TNL association towards the TNL address of the AMF received from the source NG-RAN node and establishes a TNL association towards it, the target NG-RAN node should select the TNL association to create an NGAP UE TNLA binding for the UE. Otherwise, the target NG-RAN node should select other available TNL association towards the same AMF or an AMF from the same AMF set to create an NGAP UE TNLA binding for the UE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the Indication of Bitrate Adaptation IE set to "uplink" is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall, if supported, store the received indication and use it for uplink rate control, as defined in TS 38.300 [9].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the new NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, and the Geographical Area IE contains the MDT PLMN List IE, the new NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

When the target NG-RAN node has an available TNL association towards the TNL address as indicated by the Signalling TNL association address at source NG-C side IE, or when the target NG-RAN node does not have a TNL association towards the TNL address of the AMF received from the source NG-RAN node and establishes a TNL association towards it, the target NG-RAN node should select the TNL association to create an NGAP UE TNLA binding for the UE. Otherwise, the target NG-RAN node should select other available TNL association towards the same AMF or an AMF from the same AMF set to create an NGAP UE TNLA binding for the UE.

End of Change