3GPP TSG-RAN WG3 Meeting #130	R3-258415

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.1

Source:	Nokia

Title:	Handling of emergency PDU session and PEI

WID/SID:	NR_UE_pow_sav_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has received the LS in R3-258007 [1] where SA2 informs RAN3 that SA2 has considered the backwards compatibility issue raised by RAN3 and approved a CR.

SA2 thanks RAN3 for the LS in on the compatibility issue for PEI and emergency PDU session.

SA2 has approved the attached Rel-17 CR.

This paper analyses the CR agreed by SA2 and whether any further alignment is necessary by RAN3.

# 2	Discussion

Background

The issue started with an agreed CR in SA2 in S2-2502428 for TS 23.501 removing the restriction for UE to not signal PEISP support from release 19 onwards.

#### 5.4.12.2	Core Network Assistance for PEIPS

To support the Paging Early Indication with Paging Subgrouping (PEIPS), Paging Subgrouping Support Indication and the PEIPS Assistance Information is used by the AMF and NG-RAN to help determine whether PEIPS applies to the UE and which paging subgroup used when paging the UE (see TS 38.300 [27]).

.......

This means that a R19 UE could signal PEIPS support, then AMF could relay this to gNB and UE so that the UE would apply PEIPS. One of the backwards compatibility issue was that gNB release 17/18 will not apply the PEIPS as per the text in TS 38.300 clause 9.2.2.1:

The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28].

This desynchronization was leading to paging loss.

CT1 had also discussed the backwards compatibility but could not agree (see C1-255035) a stage 3 CR to align the stage 2 change which would have resolved the issue. It was postponed.

In the LS [1], SA2 informs of the new CR 6467 in S2-2509756 [2] against TS 23.501 agreed by SA2. It covers any situation where an emergency PDU has been established, regardless what was included in the registration request as shown below:

When an emergency PDU session is successfully established over 3GPP access, the UE and the network shall not use PEIPS assistance information, see details as described in TS 24.501 [47].

Comparing with the current TS 38.300 release 17 text, both gNB, UE and AMF applies now the same policy.

Observation 1: the new SA2 CR from SA2 aligns UE, RAN and CN behaviour.

From the above, we can infer from this that the backwards compatibility issue should be resolved.

It is therefore proposed to conclude this issue.

Proposal 1: agree that the backwards compatibility issue for emergency PDU session and PEI is solved and no further RAN3 CRs is needed.

# 3	Conclusion

This paper has recalled the past discussions on the risk of losing paging due to the backwards compatibility issue introduced by SA2 CR 2428 for TS 23.501 in S2-2502428.

It has shown that the new SA2 CR 6467 agreed at SA2#171 in S2-2509756 against TS 23.501[2] solves the issue and aligns with current RAN3 TS 38.300.

It is therefore proposed to conclude that RAN3 CRs are no more needed.

Proposal 1: agree that the backwards compatibility issue for emergency PDU session and PEI is solved and no further RAN3 CRs is needed.

Proposal 2: agree the reply LS in [3] accordingly.

# 4	References

- R3-258007, LS response on compatibility issue for PEI and emergency PDU session, S2-2509759

- S2-2509756, PEIPS for emergency PDU session, TS 23.501, CR 6467, SA2#171

- R3-258420, Reply LS on compatibility issue for PEI and emergency PDU session