3GPP TSG-RAN WG3 Meeting #130	R3-258417

Dallas, USA, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support for Continuous Management Based MDT in stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce Continuous management-based MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the following: - Definition of C-MDT - Added C-MDT as MDT mode - Functional description of C-MDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Continuous Management-based MDT is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 4.1, 5.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply.

C-MDT: MDT functionality involving measurements from the same UE across RRC Transitions and RRC Connected Mobility.

Immediate MDT: MDT functionality involving measurements performed by the UE in CONNECTED state and reporting of the measurements to RAN available at the time of reporting condition as well as measurements by the network for MDT purposes.

Logged MDT: MDT functionality involving measurement logging by UE in IDLE mode, INACTIVE state, CELL_PCH, URA_PCH states and CELL_FACH state when second DRX cycle is used (when UE is in UTRA) for reporting to eNB/RNC/gNB at a later point in time, and logging of MBSFN measurements by E-UTRA UE in IDLE and CONNECTED modes.

Management Based MDT PLMN List: MDT PLMN List applicable to management based MDT.

MDT measurements: Measurements determined for MDT.

MDT PLMN List: A list of PLMNs where MDT is allowed for a user. It is a subset of the EPLMN list and RPLMN at the time when MDT is initiated.

Signalling Based MDT PLMN List: MDT PLMN List applicable to signalling based MDT.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AICH	Acquisition Indicator CHannel

BLER	Block Error Rate

BSSID	Basic Service Set Identifier

CA	Carrier Aggregation

CDMA	Code Division Multiple Access

CHO	Conditional Handover

C-MDT	Continuous management-based MDT

CN	Core Network

<<<<<<<<<<<<<<<<<<<< Unmodified Text Skipped >>>>>>>>>>>>>>>>>>>>

SNPN	Stand-alone Non-Public Network

SNR	Signal to Noise Ratio

SON	Self Organizing/Optimizing Network

SRB	Signalling Radio Bearer

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# 4	Main concept and requirements

## 4.1	General

The general principles and requirements guiding the definition of functions for Minimization of drive tests are the following:

1.	MDT mode
There are two modes for the MDT measurements: Logged MDT and Immediate MDT. There are also cases of measurement collection not specified as either immediate or logged MDT, such as Accessibility measurements. C-MDT mode corresponds to a mode where the same UE provides Immediate or Logged MDT measurements within the same MDT job.

2.	UE measurement configuration
It is possible to configure MDT measurements for the UE logging purpose independently from the network configurations for normal RRM purposes. However, in most cases, the availability of measurement results is conditionally dependent on the UE RRM configuration.

3.	UE measurement collection and reporting
UE MDT measurement logs consist of multiple events and measurements taken over time. The time interval for measurement collection and reporting is decoupled in order to limit the impact on the UE battery consumption and network signalling load.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Skipped >>>>>>>>>>>>>>>>>>>>

The solutions for MDT shall take into account the following constraints:

1.	UE measurements
The UE measurement logging mechanism is an optional feature. In order to limit the impact on UE power consumption and processing, the UE measurement logging should as much as possible rely on the measurements that are available in the UE according to radio resource management enforced by the access network.

2.	Location information
The availability of location information is subject to UE capability and/or UE implementation. Solutions requiring location information shall take into account power consumption of the UE due to the need to run its positioning components.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.4.x	Support of Continuous MDT (C-MDT)

C-MDT is a management-based MDT functionality that enables continuous collection of MDT data within a certain area.

The OAM allocates a specific range of Trace References (TR) allowing for the identification of C-MDT jobs. C-MDT is configured by OAM over an area of gNBs either by two Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with the same TR to the gNB, or with one Trace Session activation request with Job Type (IMMEDIATE_MDT_AND_ LOGGED_MDT). OAM also indicates to each gNB in a given area a range of Trace Recording Session References (TRSR), within which a gNB may select its values.

The gNB shall configure the UE selected for Continuous Management-based MDT with Immediate MDT and Logged MDT by using the configuration provided by the OAM. A unique Trace Recording Session Reference is assigned to the UE by the gNB which is valid for both Immediate MDT and Logged MDT, allocated based on the received OAM configuration.

When a UE indicates availability of logged MDT measurements upon transition from RRC_Idle to RRC_Connected, a gNB configured with C-MDT shall request the Logged MDT report. If the gNB determines that the UE is configured with C-MDT through the reported TR in the log, and if the area-based selection and user consent conditions are satisfied, it shall reconfigure the UE with Immediate C-MDT.

When a C-MDT UE is handed over to a target gNB configured with C-MDT, the target gNB shall determine whether the UE is configured with C-MDT based on C-MDT information included in the HANDOVER REQUEST message and, if the area-based selection and user consent conditions are satisfied, it shall reconfigure the UE with an Immediate C-MDT configuration corresponding to the TR provided as part of the C-MDT information.

When a UE transits from RRC_Inactive to RRC_Connected in a gNB, the gNB shall determine whether the UE is configured with C-MDT based on C-MDT information included in the RETRIEVE UE CONTEXT RESPONSE message and, if the area-based selection and user consent conditions are satisfied, it shall reconfigure the UE with an Immediate C-MDT configuration corresponding to the TR provided as part of the C-MDT information.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>