3GPP TSG-RAN WG3 #130	R3-258418

Dallas, USA, 17th – 21st Nov. 2025

Agenda Item:	10.2.1 General principles and requirements

Source:	NTT DOCOMO INC.

Title:	Requirements and deployment scenarios toward 6G RAN

Document for:	Discussion

Introduction

At the previous meeting, 6G study phase was started, and pCR [1] was agreed. On the other hand, there are still many proposals discussed in the offline discussion [2] but not captured in the TR. In this contribution, we refer to 6G SID [3] and the requirements captured in TR 38.914 at RAN#109 [4], and propose the general principles and deployment scenarios that should be captured in TR 38.760-3 for the 6G study.

Discussion

General Principle

According to latest approved pCR for 38.914 on Requirements for architecture and migration for 6G [4] in RAN plenary#109, the following requirements should be taken into consideration into RAN3 6G study.

- The 6G RAN architecture shall support standalone RAN architecture.

- The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

- The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

- The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

- The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

- 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

- The 6G RAN architecture shall allow for control plane and user plane separation.

- The 6G RAN architecture shall support sharing of the RAN between multiple operators.

- The 6G RAN architecture shall allow for the operation of network slicing.

- The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

- The 6G RAN architecture shall support enhanced service awareness in RAN.

- The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

- The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

- The 6G RAN architecture shall allow non-public networks.

At the previous meeting, the requirements listed above were referred in the pCR as the basis for the RAN architecture design principles. To move forward with the RAN3 study on 6G, it is important to further clarify and elaborate these general principles. In particular, we believe that TR 38.760-3 should explicitly capture, at a high level, those principles derived from TR 38.914 that are especially critical for discussions on the 6G RAN architecture. By doing so, we can ensure that the ongoing and future studies within RAN3 are guided by a clear and consistent set of architectural principles, which will facilitate more effective and harmonized progress in the standardization of 6G RAN. Therefore, we propose to capture following from TR 38.914.

- For inter-RAT mobility between 6GR and NR: In the initial phase of 6G deployment, 5G and 6G are likely to coexist. To keep high performance while keeping good user experiences of 6G, or to ensure a smooth fallback of voice services, robust mobility functions between RATs are required.

- For open interfaces for multi-vendor interoperability: Defining specifications in 3GPP is rooted in the motivation to enable interoperability across different vendor equipment. Especially in RAN3, where protocols (e.g. RAN–CN interfaces) connect potentially different vendors, the specification design should aim for deployments that do not require or minimize extra coordination beyond the standards.

- For control/user plane separation: At a high level, RAN3 should enable appropriate priority control in the transport network according to traffic characteristics. Whether C/U separation applies to protocol design and logical entities depends on detailed architecture/protocol discussions in other agenda.

- For lower CAPEX/OPEX: An operator’s success in deploying 6G RAN depends on balancing user performance demands with the cost. Therefore, in addition to aiming for performance gains, reducing CAPEX/OPEX is also an important requirement. In particular, architecture design should afford sufficient flexibility so that operators can deploy 6G at a cost aligned with demand.

- For enhanced resilience: As highlighted in TR 38.914, enhancing resiliency is essential for both rapid recovery from RAN equipment failures—such as those caused by natural disasters—and for maintaining service continuity during routine operation and maintenance. This is particularly critical in disaster-prone regions like Japan, where the speed of network restoration can have a direct impact on public safety, as seen in emergency warning systems like ETWS. While 5G specified only limited resiliency functions, operators have relied on vendor-specific redundancy and, in some cases, manual operation for recovery. In the 6G era, it will be important to support resiliency functions in the specifications, enabling operators to leverage common hardware pools and reduce costs associated with redundant equipment. Furthermore, the integration of RAN virtualization, advanced monitoring and management using AI/ML, and automated recovery mechanisms—building on existing SON/MDT functions—will enable more robust, rapid, and stable cellular communications. Standardizing these resiliency mechanisms will help realize a more reliable, vendor-agnostic network infrastructure for the future.

Proposal 1:	RAN3 to capture following general principles for 6G study based on the 6G RAN architecture and migration requirements captured in TR 38.914.

- The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

- 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

- The 6G RAN architecture shall allow for control plane and user plane separation.

- The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

- The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

Regarding additional migration options (other than standalone, MRSS, and inter-RAT mobility between NR-6G), RAN will start the study in March 2026 and will make a decision by September 2026 whether expand WG SI scope to cover additional migration option(s) as stated in [3]. RAN will also task relevant RAN WGs for any specific technical analysis. Therefore, RAN3 should wait for liaison from RAN regarding additional migration options.

Observation 1:	RAN will start the study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal 2:	RAN3 should wait for liaison from RAN regarding additional migration options study.

In 5G, RAN nodes are deployed relying on OAM-based management and various pre-configurations. For example, capability mismatches across RAN nodes can cause issues in basic procedures such as handover—especially when nodes from different vendors. To enable more flexible deployments and vendor choice in 6G without huge effort for inter-operability, RAN3 should study functions that realize a plug-and-play RAN as a high-level principle.

Proposal 3:	6G RAN shall enable plug-and-play deployment of heterogeneous RAN nodes even from different vendors, minimizing pre-configuration and OAM dependencies.

Deployment scenarios

For each deployment scenario, some aspects affect overall RAN architecture (e.g., CU–DU split) while others affect protocol design (e.g., virtualization). Excluding those, we first address deployment scenarios that operators need. Before moving to detailed discussions of solutions linked to each requirement, considering that this is the study phase, the requirements necessary to deploy/operate 6G should be captured in the TR before discussing whether any specific solution should be supported.

Proposal 4:	RAN3 to capture potential deployment scenarios first. Architectural and functional impact on RAN3 specifications can be further discussed.

- Details on monolithic and split RAN deployments will be discussed in Section 10.4; hence we do not go into detail here. However, to reduce CAPEX/OPEX as mentioned above, architecture design should aim for flexible deployments that allow centralization and distribution depending on the deployment situations.

- Regarding the splitting layer of 6G RAN node, lower layer split (i.e. RU) was already discussed [5] in RAN#109 and they endorsed that 6G RAN includes a distinct logical Radio Unit (RU) and the interface for it will not be specified.

- Fig. 1. Non-centralized and centralized deployment

Proposal 5:	Support both monolithic RAN node and split RAN node architecture in 6G. In which layer do we split the RAN node can be further discussed.

Given the current 5G deployment and the fact that there will not be many new frequencies in 6G, it's possible that 4G/5G/6G could all be used to form coverage. Some country including Japan, 4G is used to form coverage, and NSA is used for higher capacity. In some areas with high traffic 5G SA is deployed, such as urban areas, for higher performance and/or advanced features. For early step of 6G deployment, it is expected that 6G will be deployed for further performance, i.e. in areas covered with 4G NSA or 5G NR. This means that in some area, 4G would be potentially migrated to/co-located with 6G skipping a migration to 5G SA. Considering this situation, from operator perspective, both 6G/5G and 6G/4G co-location should be considered. In actual deployment, 3 RAT co-location would happen in case that 4G would be still needed for legacy devices even if 6G/5G are overlaying or in case that neither VoNR nor 6G voice is supported. However, we are not so motivated to specify any enhancement to support that scenario, and in most cases, it can be considered as the combination of Site A and Site B in the Fig.2. It is also noted that MRSS for 6G and 4G and for 3 RAT is not approved in RAN plenary, while one for 6G and NR was approved. From RAN3 perspective, for these deployment scenarios, we should discuss whether and how to introduce an interface and procedures for coordination between RAN nodes across RATs. This coordination would include, e.g. for inter-RAT handover and MRSS. There are following approaches, therefore we suggest discussing which approach we should take.

- Single RAN node serves both 5G and 6G, or different RAN nodes serve each RAT, and the coordination would be done based on the implementation.

- Different RAN nodes serve each RAT, and related procedures would be done without the coordination between them.

- Different RAN nodes serve each RAT, and the interface between RAN nodes (similar as Xn) would be specified and used for the coordination between them.

- Different RAN nodes serve each RAT, and new interface between lower layer part of RAN nodes (similar as D2 specified in O-RAN) would be used for the coordination between them and specified, if necessary.

Fig. 3. Co-sited deployment with LTE/NR

Observation 2:	At least, co-site deployment for 6G/5G and 6G/4G would be needed in initial phase of 6G deployment.

Observation 3:	Co-site deployment for 4G/5G/6G would happen in initial phase of 6G deployment, however it can be considered as the combination of 6G/5G co-location and 6G/4G co-location in most cases.

Proposal 6:	Co-sited deployment with 6G-NR and with 6G-E-UTRAN should be supported. FFS on interworking between 6G and E-UTRAN pending on RAN plenary discussions.

Proposal 7:	RAN3 to discuss whether and how to specify interfaces and procedures for MRSS between RAN nodes.

For wider coverage and higher reliability, not only TN but also NTN should be utilized in the 6G era. We think that considering both TN and NTN and supporting smooth mobility and various topologies from day 1 will contribute to the success of deployment and commercialization. Mobility procedures would be discussed mainly in RAN2, and from RAN3 perspective, switch overs of RAN interfaces should be considered, because in NTN deployment, connection between RAN-CN and RAN nodes frequently switched, which characteristic is different from TN deployment. In 5G, suspend/resume/removal of NG interfaces are supported in release 19 as a fundamental function for gNB on board NTN, and at least it should be supported from 6G day 1.

Proposal 8:	Support both TN and NTN deployments.

Conclusions and proposals

Our proposals are summarized below.

[General principles]

Proposal 1:	RAN3 to capture following general principles for 6G study based on the 6G RAN architecture and migration requirements captured in TR 38.914.

- The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

- 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

- The 6G RAN architecture shall allow for control plane and user plane separation.

- The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

- The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

Observation 1:	RAN will start the study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal 2:	RAN3 should wait for liaison from RAN regarding additional migration options study.

Proposal 3:	6G RAN shall enable plug-and-play deployment of heterogeneous RAN nodes even from different vendors, minimizing pre-configuration and OAM dependencies.

[Deployment scenarios]

Proposal 4:	RAN3 to capture potential deployment scenarios first. Architectural and functional impact on RAN3 specifications can be further discussed.

Proposal 5:	Support both monolithic RAN node and split RAN node architecture in 6G. In which layer do we split the RAN node can be further discussed.

Observation 2:	At least, co-site deployment for 6G/5G and 6G/4G would be needed in initial phase of 6G deployment.

Observation 3:	Co-site deployment for 4G/5G/6G would happen in initial phase of 6G deployment, however it can be considered as the combination of 6G/5G co-location and 6G/4G co-location in most cases.

Proposal 6:	Co-sited deployment with 6G-NR and with 6G-E-UTRAN should be supported. FFS on interworking between 6G and E-UTRAN pending on RAN plenary discussions.

Proposal 7:	RAN3 to discuss whether and how to specify interfaces and procedures for MRSS between RAN nodes.

Proposal 8:	Support both TN and NTN deployments.

References

- R3-257323, “General principles and requirements”, Qualcomm

- R3-257292, “SoD CB # 19 6G RANArch”, Qualcomm

- RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo.

- RP-252870, “pCR for 38.914 on requirements for architecture and migration for 6G”, NTT Docomo.

- RP-252881, “Way Forward on 6G Fronthaul”

Annex: pCR for TR 38.760-3

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

The 6G RAN architecture shall allow for control plane and user plane separation.

The 6G RAN architecture shall enable lower CAPEX/OPEX with respect to current networks.

The 6G RAN architecture shall allow enhanced resilience compared to NR if/where applicable.

The 6G RAN architecture shall enable plug-and-play deployment of heterogeneous RAN nodes even from different vendors, minimizing pre-configuration and OAM dependencies.

## 5.2	Deployment Scenarios

Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914.

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

-  FFS whether all deployment scenarios of TR 38.914 can be supported.

Support both monolithic RAN node and split RAN node architecture in 6G.

-  FFS on in which layer we split the RAN node and how to specify the interface.

Co-sited deployment with 6G-NR and with 6G-E-UTRAN should be supported.

-  Interworking between 6G and E-UTRAN pending on RAN plenary discussions.

Support both TN and NTN deployments.