3GPP TSG-RAN WG3 #130	R3-258421

Dallas, USA, 17th – 21st Nov. 2025

Agenda Item:	10.3.1 General principles and requirements

Source:	NTT DOCOMO INC.

Title:	Requirements toward 6G RAN-CN functional split

Document for:	Discussion

Introduction

At the previous meeting, 6G study phase was started, and pCR [1] was agreed. On the other hand, there are still many proposals discussed in the offline discussion but not captured in the TR. In this contribution, we refer to the requirements captured in TR 38.914 at RAN#109 [2], align with the general principle discussed in another paper [3], and propose the requirements for RAN-CN interface that should be captured in TR 38.760-3 and a necessary consideration for the 6G study.

Discussion

Resiliency

As described in our other contribution [3], resilience is also an important viewpoint for operators. For rapid recovery from RAN/CN failures, it is very useful to specify functions for resilience in 3GPP for RAN-CN interfaces that are expected to be cross-vendor operations. In particular, dynamic/fast endpoint switching of RAN-CN interfaces should be discussed in RAN3 in relation to virtualization and feeder link switch over in NTN.

Proposal 1:	RAN-CN interface in 6G shall support functionalities for enhanced resiliency.

Proposal 2:	RAN3 to discuss dynamic switch over of the RAN-CN interface endpoints both in CN and RAN side, e.g. for virtualization, NTN feeder link switch over, and fast recovery from failures on NW entities.

RAN-CN connectivity options

In 5G study, several options were discussed for EPC/5GC and E-UTRAN/NR connectivity. Similarly, for 6G, 5GC/6G RAN or 6GC/5G RAN connectivity should be discussed including backward compatibility of RAN-CN interface. This point is also affected by the migration option. If UE aggregates 6G and NR based on single registration to the CN, RAN-CN cross connection is required between 6G and 5G. However, as described in 6G RAN SID [4], the migration discussion is decided by June 2026 in RAN plenary. In addition, whether CN in 6G has NBC for AMF/UPF in 5GC from C-plane/U-plane perspective should be discussed coordinating with SA2.

We think that the problem with the current RAN-CN interface is the concentration of load on the AMF for all C-plane signalings toward the CN. At present, most of the main functions of 5GC are basic functions related to access and mobility. However, if all the signaling for new services involving CN in 6G is signaled through the RAN-CN interface to the AMF as the end point, the load will be concentrated on the AMF. We think that the design that these new services put additional load on the NF essential for communication should be avoided. On the other hand, since we have not found a critical problem that requires NBC change for the existing NG-C/U signaling, reusing part of the existing NG-C/U in 6G, or changing only the transport network layer and reusing the overall procedure of legacy can be also potential solutions. RAN3 should discuss whether to reuse the existing NG-C/U or specify a new RAN-CN interface based on coordination with SA2. In some cases, the details of the fundamental functions of RAN, such as paging, mobility, UE context management, and so on, which are involved in RAN2 discussions, may also affect it.

From our perspective, we do not see any necessity to have NBC change on NG-U protocol design. Therefore, we think we can reuse 5G NG-U protocol to one for 6G.

Proposal 3:	RAN3 to discuss whether and how to reuse existing NGAP or specify new RAN-CN interface, based on the discussions for overall architecture in SA2 and for fundamental functionalities, e.g. paging, mobility, and UE context management in RAN2/SA2.

Proposal 4:	5G NG-U protocol can be reused for 6G.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN-CN interface in 6G shall support functionalities for enhanced resiliency.

Proposal 2:	RAN3 to discuss dynamic switch over of the RAN-CN interface endpoints both in CN and RAN side, e.g. for virtualization, NTN feeder link switch over, and fast recovery from failures on NW entities.

Proposal 3:	RAN3 to discuss whether and how to reuse existing NGAP or specify new RAN-CN interface, based on the discussions for overall architecture in SA2 and for fundamental functionalities, e.g. paging, mobility, and UE context management in RAN2/SA2.

Proposal 4:	5G NG-U protocol can be reused for 6G.

References

- R3-257324, “RAN-CN interface principles and functions”, Huawei.

- RP-252870, “pCR for 38.914 on requirements for architecture and migration for 6G”, NTT Docomo.

- R3-258418, “Requirements and deployment scenarios toward 6G RAN”, NTT Docomo.

- RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo.

Annex: pCR for TR 38.760-3

6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

-	the 6G RAN-CN interface supports functionalities for enhanced resiliency.