3GPP TSG-RAN WG3 #130	R3-258424

Dallas, USA, 17th – 21st Nov. 2025

Agenda Item:	10.3.2 RAN-CN interface options

Source:	NTT DOCOMO INC.

Title:	Necessary considerations for 6G RAN-CN interface options

Document for:	Discussion

Introduction

At the previous meeting, discussions under Agenda Item 10.3.1 focused on requirements and functions of RAN-CN interface, while Agenda Item 10.3.2 was not discussed. As indicated in 6G SID [1] as following, based on [2], this contribution discusses the advantages and challenges of existing solutions and potential solutions on the transport protocol stack design for RAN-CN interface.

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

Discussion

Analysis of current protocol design for C-plane

In current specifications, C-plane RAN interfaces including RAN-CN interfaces adopt a protocol stack using SCTP [3]. This assumes an end-to-end connection, and the IP address of the endpoint is signaled as a TNL address in NGAP and other RAN IFs’ protocols. SCTP has an integrated heart-beat function, and connection life management using this function is widely used by operators. Regarding security, 3GPP supports DTLS and IPsec [4], and these security functions can be used depending on the deployment environment of RAN nodes and transport networks. From the architecture perspective, the architecture in which RAN-CN interfaces are terminated by AMF enables simple procedures and management.

On the other hand, in the architecture where AMF is a single endpoint of RAN-CN interface, if signaling between RAN and NF other than AMF increases, the load may be concentrated on AMF which has fundamental functions related to access and mobility. From the security point of view, when signaling through SecGW for IPsec, additional latency of around tens of ms may occur because SecGW is usually placed centralized. In NG interface, this effect is relatively small because AMF is also placed centralized as well as SecGW, but it is especially noticeable for interfaces between RAN nodes such as Xn interface.

Observation 1:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following benefits:

- Integrated heart-beat function to monitor the connection health

- Flexible security options depending on deployments

- Simple architecture and procedures due to single endpoint of RAN-CN connection

Observation 2:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following issues:

- Not virtualization/cloud friendly due to end-to-end connection

- Load concentration on AMF

Analysis of potential solution for C-plane protocol

Among the existing protocol designs mentioned in the previous chapter, SBI can be a candidate solution, especially considering the compatibility for virtualization. Direct connectivity to NF other than RAN node and AMF can also be considered. This may avoid the architecture in which AMF is a single endpoint and separate the load of AMF from the new services that require signaling between RAN and NF.

On the other hand, possible drawbacks brought by the new design of transport protocol should be studied carefully at this stage. From operator perspective, use experience has first priority on option selection, therefore even if it contributes CAPEX/OPEX reduction, we cannot employ it without knowing the drawbacks would be experienced by customers.

Comparing to the existing protocol design, when a certain NW node is attacked, there are more NW nodes that can be accessed from that node. From a transport network point of view, it may be possible to access other nodes even in legacy, but from a security point of view, it is necessary to coordinate with SA3 when the protocol in the 3GPP specification allows access to more NW nodes. In addition, when considering the integration with the SBI chain of CN, it may be difficult to simply connect all RAN nodes directly to the SBI chain of CN because the existing 5GC typically assumes the connection of ~100 order NFs. Especially, the increased number of NFs managed by NRF and the load balancing of the transport network would be issues. Those issues need to be addressed in the study. Finally, from the learning in 5GC, the introduction of SBI may cause an increase of NFs and the complexity of procedures. Longer procedures may also potentially lead to slower signaling, lower performance, or IOT issues.

When it comes to the protocol stack, there are several options for adopting SBI. One of them is QUIC + HTTP/3 [5], which is one of the most modern and cloud-friendly protocols and might be a good candidate solution for virtualization. However, there is a concern about the use of QUIC [6], regardless of the use of HTTP/3 (i.e.SBI or not). Unlike SCTP, QUIC integrates TLS 1.3 and has the advantage of supporting multi-streams without restriction by endpoint IP addresses. However, this integration, i.e. mandating security protection might reduce the flexibility of the security protection option for operators. According to the existing specification [4], the use of security protection (DTLS and/or IPsec) is up to operator decision, and in secure environments, it may not be used. In some deployments, based on operator’s security policy and/or legal regulation, IPsec is preferred because it allows us to place security endpoint, i.e. SecGW, in a secure place under the operator’s control, and security protection by DTLS may not be needed. In that sense IPsec has an advantage over DTLS, and whether operators should have flexibility on this point needs to be discussed.

Regarding message encoding, we think that the existing ASN.1 should be reused even when introducing SBI. Text format is used in 5GC, but at the same time, we experience increased message size. Since ASN.1 is widely used in RAN interfaces, we do not see any problems, and it has advantages in terms of load on the transport network.

Observation 3:	SBI for C-plane RAN-CN interface transport protocol (e.g. QUIC + HTTP/3) has following potential benefits:

- Optimal for RAN virtualization

- Allow RAN nodes connection to other NFs in CN than AMF

- Flexible/dynamic TNL endpoint handling contributes enhanced feeder link switch over for NTN or resiliency functionalities, etc.

Observation 4:	SBI for C-plane RAN-CN interface transport protocol has following potential issues:

- Security threats due to more flexible connection between RAN nodes and NFs

- Some issues on integration with SBI chain in CN

- More complex architecture and longer procedures

Observation 5:	QUIC for C-plane RAN-CN interface transport protocol has following potential issue:

- No flexibility on the use of security protection even in case the operator can make it sure that the RAN nodes and transport network are deployed in secure environment

Observation 6:	There is no need to employ text format in RAN interfaces including RAN-CN interface.

Proposal 1:	In study of the new transport protocol for C-plane RAN-CN interface in 6G, following perspectives need to be addressed:

- Performance (e.g. slower C-plane signaling due to more interaction btw RAN nodes)

- Compatibility to RAN virtualization

- Potential security threats (coordination with SA3 is needed)

- Necessity on RAN nodes connections with other NFs than AMF

- Integration with virtualized/cloud-based CN

- Simple architecture and procedures

- Flexibility on security protection

Proposal 2:	RAN3 to study following options for encoding of C-plane RAN-CN interface protocol:

- Binary format (i.e. ASN.1)

- Text format (e.g. JSON)

Protocol design for U-plane

The currently used P2P connection between RAN node and UPF using GTP-U [7] is effective for handling large capacity data streams separately from C-plane with appropriate priority control. Since we do not see any issues in the current User-plane transport protocol design so far, we think that the baseline for the U-plane transport protocol in 6G should be the same protocol stack using GTP-U as in 5G. We are open to discuss the replacement of GTP-U if there are significant advantages.

Observation 7:	From operator perspective, there is no issue on current protocol design for U-plane RAN-CN interface (i.e. GTP-U).

Proposal 3:	For U-plane RAN-CN interface in 6G, GTP-U can be considered for transport protocol stack same as 5G.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following benefits:

- Integrated heart-beat function to monitor the connection health

- Flexible security options depending on deployments

- Simple architecture and procedures due to single endpoint of RAN-CN connection

Observation 2:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following issues:

- Not virtualization/cloud friendly due to end-to-end connection

- Load concentration on AMF

Observation 3:	SBI for C-plane RAN-CN interface transport protocol (e.g. QUIC + HTTP/3) has following potential benefits:

- Optimal for RAN virtualization

- Allow RAN nodes connection to other NFs in CN than AMF

- Flexible/dynamic TNL endpoint handling contributes enhanced feeder link switch over for NTN or resiliency functionalities, etc.

Observation 4:	SBI for C-plane RAN-CN interface transport protocol has following potential issues:

- Security threats due to more flexible connection between RAN nodes and NFs

- Some issues on integration with SBI chain in CN

- More complex architecture and longer procedures

Observation 5:	QUIC for C-plane RAN-CN interface transport protocol has following potential issue:

- No flexibility on the use of security protection even in case the operator can make it sure that the RAN nodes and transport network are deployed in secure environment

Observation 6:	There is no need to employ text format in RAN interfaces including RAN-CN interface.

Proposal 1:	In study of the new transport protocol for C-plane RAN-CN interface in 6G, following perspectives need to be addressed:

- Performance (e.g. slower C-plane signaling due to more interaction btw RAN nodes)

- Compatibility to RAN virtualization

- Potential security threats (coordination with SA3 is needed)

- Necessity on RAN nodes connections with other NFs than AMF

- Integration with virtualized/cloud-based CN

- Simple architecture and procedures

- Flexibility on security protection

Proposal 2:	RAN3 to study following options for encoding of C-plane RAN-CN interface protocol:

- Binary format (i.e. ASN.1)

- Text format (e.g. JSON)

Observation 7:	From operator perspective, there is no issue on current protocol design for U-plane RAN-CN interface (i.e. GTP-U).

Proposal 3:	For U-plane RAN-CN interface in 6G, GTP-U can be considered for transport protocol stack same as 5G.

References

- RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo.

- R3-257159, “Requirements and deployment scenarios toward 6G RAN”, NTT Docomo

- TS 38.412, “NG-RAN; NG signalling transport”

- TS 33.501, “Security architecture and procedures for 5G system”

- RFC 9114, “HTTP/3”

- RFC 9000, “QUIC: A UDP-Based Multiplexed and Secure Transport”

- TS 38.415, “NG-RAN; PDU Session User Plane Protocol”