3GPP TSG-RAN WG3#130	R3-258438

Dallas, U.S.A., 17-21 November 2025

Title: 	Stage 2 corrections for AI/ML based CCO

Source: 	Ericsson, Jio Platforms, InterDigital, Fibercop

Agenda item:	9.2.2

Document Type:	Discussion and Decision

# 1. Introduction

In this contribution to discuss stage 2 corrections to AI/ML based CCO specified in Rel-19.

# 2. Discussion

## 2.1 Behaviour when a CCO issue is predicted

In a split gNB1, when a CCO issue is predicted by a gNB-CU1, the gNB-CU1 sends the Predicted CCO Assistance Information IE to the gNB-DU1 controlling the predicted affected cells and beams. The gNB-DU1, in response to the received Predicted CCO Assistance Information, generates a future coverage configuration for its cells and beams, and sends it to the gNB-CU1 in the Future Coverage Modification Notification IE.
After receiving from gNB-DU1 the Future Coverage Modification Notification, gNB-CU1 may sends to another connected gNB-DU2 the Neighbour Future Coverage Modification IE with the future coverage configuration for the affected cells and beams served by gNB-DU1. The gNB-DU2, in response to the received Neighbour Future Coverage Modification, generates a future coverage configuration for its cells and beams and sends the information back to the gNB-CU1.

At this point, the gNB1 sends the future coverage modification to a neighbouring gNB2 (gNB-CU2) in the Future Coverage Modification List IE, and gNB-CU2 sends to its underlying gNB-DUs (e.g., gNB-DU3) a Neighbour Future Coverage Modification IE. Similarly to gNB-DU2, also the gNB-DU3, in response to the received Neighbour Future Coverage Modification generates a future coverage configuration for its cells and beams and sends the information back to the gNB-CU2.

Hence, when a CCO issue is predicted, there are two scenarios in which a gNB-DU “responds” to its gNB-CU, namely:

Scenario 1: the gNB-DU serves the predicted affected cells and beams, and it receives the Predicted CCO Assistance Information IE from a gNB-CU

Scenario 2: the gNB-DU does not serve the predicted affected cells and beams, and it receives the Neighbour Future Coverage Modification IE from a gNB-CU

To capture the above, the current stage 2 (TS38.401) specifies the following:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen.

A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

It can be appreciated how the first paragraph describes the information a gNB-DU receives in scenario 1, but no description is provided describing the gNB-DU response. The second paragraph describes the information received by a gNB-DU in scenario 2, and for that scenario the corresponding gNB-DU response is described.

The current text in TS38.401 is prone to be interpreted as if the gNB-DU “response” is specified for scenario 2, while the gNB-DU response is unspecified for scenario 1. On the other hand, according to the solution agreed for AI/ML based CCO in Rel-19, the gNB-DU “response” is exactly the same for both scenario 1 and 2, hence the specification is ambiguous in terms of gNB-DU behaviour for scenario 1.

Observation 1: In the current stage 2 text describing a gNB-DU behaviour when a CCO issue, it is not clear what is the behaviour of the gNB-DU controlling the predicted affected cells and beams, when receiving Predicted CCO Assistance Information from gNB-CU.

An easy way to solve this issue is to unify the two paragraphs, as shown below:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen. A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

With this simple correction, it becomes clear that the gNB-DU behaviour described in the sentence starting with “In response,” applies to both scenario 1 and 2, which is the behaviour agreed for AI/ML based CCO in Rel-19.

Proposal 1: In TS 38.401, section 7.11, unify the two paragraphs describing the received information at a gNB-DU when a CCO issue is predicted, and the corresponding gNB-DU response.

## 2.1 Behaviour when a predicted CCO issue is cancelled

When a predicted CCO issue is cancelled, the two following logical steps are used:

- a gNB-DU receives a notification that a previously predicted CCO issue is cancelled, and the list of associated predicted affected cells and beams

- if the gNB-DU has not yet applied the future coverage state change, it notifies the gNB-CU that the future coverage state change is cancelled

It is worth to note that step 2 is executed only if step 1 is executed.

Observation 2: A gNB-DU receives the notification that a predicted CCO issue is cancelled, and only after that, it informs the gNB-CU that the future coverage state associated with the cancelled predicted CCO issue (and not yet applied), is cancelled.

We also observe that the steps above allow to cancel a predicted CCO issue both at the gNB predicting the issue, as well as at a neighbour gNB, with the same logic being used when the gNB-DU controls the predicted affected cells and beams and when the gNB-DU does not control such cells and beams.

The current text in stage 2 (38.401) describes the cancelling of a predicted CCO issue with two separate paragraphs:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled.

The gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

The two paragraphs above are separate, hence the text above can be read as if a gNB-DU may notify the gNB-CU that a previously generated future coverage state is not applied, without having previously been informed by the gNB-CU that the predicted CCO issue is cancelled. Such behaviour is not in agreement with the Rel-19 solution, and a clarification is needed.

Observation 3: The current stage 2 text describing the cancelling of a predicted CCO issue allows a gNB-DU to cancel a future coverage modification without having previously received an indication that the corresponding predicted CCO issue is cancelled, which is not in agreement with the Rel-19 solution.

We think that an easy way to solve this issue is to unify the two paragraphs, and add an explicit indication that the gNB- DU notifies the gNB-CU that a previously non applied future coverage state has been cancelled in response to a cancelled predicted CCO issue, as in the example proposed below:

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled. In response, the gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

Proposal 2: In TS 38.401, section 7.11, unify the two paragraphs and indicate that gNB-DU notifies gNB-CU that a future coverage states not yet applied is cancelled in response to a cancelled predicted CCO issue.

# 3. Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: In the current stage 2 text describing a gNB-DU behaviour when a CCO issue, it is not clear what is the behaviour of the gNB-DU controlling the predicted affected cells and beams, when receiving Predicted CCO Assistance Information from gNB-CU.

Proposal 1: In TS 38.401, section 7.11, unify the two paragraphs describing the received information at a gNB-DU when a CCO issue is predicted, and the corresponding gNB-DU response.

Observation 2: A gNB-DU receives the notification that a predicted CCO issue is cancelled, and only after that, it informs the gNB-CU that the future coverage state associated with the cancelled predicted CCO issue (and not yet applied), is cancelled.

Observation 3: The current stage 2 text describing the cancelling of a predicted CCO issue allows a gNB-DU to cancel a future coverage modification without having previously received an indication that the corresponding predicted CCO issue is cancelled, which is not in agreement with the Rel-19 solution.

Proposal 2: In TS 38.401, section 7.11, unify the two paragraphs and indicate that gNB-DU notifies gNB-CU that a future coverage states not yet applied is cancelled in response to a cancelled predicted CCO issue.

Proposal 3: Agree to the CR for TS 38.401 in R3-258439.