3GPP TSG-RAN WG3 Meeting #130	R3-258483

Dallas, US, 17th – 21st November, 2025

Title: 	Further discussion on AI/ML assisted intra-CU LTM

Source: 	Huawei

Agenda item:	12.2.2

Document Type:	pCR

# 1. Introduction

A new SI “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” focusing on AI/ML-based mobility was approved for Rel-20 5G-Advanced (5G-A) at RAN #108 meeting [1], then revised in RAN#109 and eventually approved in [2], aiming at investigating mobility scenarios that could benefit from using AI/ML techniques.

This contribution focuses in particular on the “intra-CU LTM” scenario, providing our considerations and proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

## 2.1 Background

In the last RAN3 meeting the following agreements were captured in the Chair’s meeting minutes pertaining the AI/ML-based intra-CU LTM [3]:

Prioritize normal intra-CU LTM scenarios. For CU-DU split architecture, the following two scenarios are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. RAN3 to study the following - AI/ML assisted candidate cell selection for LTM Handover Preparation Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

While the following FFS aspects needs further discussion:

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM. RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM. Further discuss whether to study addition of beam prediction in UE Trajectory prediction. Further discuss predicting TA value in the next meeting. Further discuss prediction of validity time of a measured TA value.

We will discuss the above open aspects in the following subsections.

## 2.2 Scope of the study and potential new deployment scenarios

This subsection covers the following two open aspects, which we think are tightly related one to each other:

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM. RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

For L1 measurement based LTM, the L1 measurement report is terminated at the gNB-DU. However, for such type of LTM to actually benefit from AI/ML capabilities, we should take into account the framework for AI/ML-based LTM agreed in the last meeting which is in line with the general framework specified in Rel-18 for AI/ML in NG-RAN for which the AI/ML model resides in the gNB-CU, and such framework perfectly works for AI/ML-aided L3 measurement based LTM.

Observation1: The agreed framework for AI/ML-based LTM is in line with the general AI/ML for NG-RAN framework specified in Rel-18. Such framework perfectly works for L3 measurement based LTM.

As a general principle, we think that we should not change an already well-established framework for the sole purpose of supporting the AI/ML-based version of another flavour of the same functionality, i.e., L1 measurement based LTM. In fact, since the AI/ML model in gNB-CU cannot directly access the L1 measurement results, in order to support the AI/ML-aided L1 measurement based LTM, the model inference function may be located in the gNB-DU. However, typically the gNB-DU is considered resource-constrained, and such limitation implies that it is not feasible to perform inference in the gNB-DU.

Observation 2: A well-established framework should not be changed for the sole purpose of supporting the AI/ML-based version of another flavour of the same functionality.

Observation 3: Support for the AI/ML-aided L1 measurement based LTM may require the model inference function to be located in the gNB-DU which, however, is resource-constrained.

Proposal 1: RAN3 to acknowledge that the gNB-DU is resource-constrained, and hence it is not appropriate the locate the inference function there.

Based on the above, in our view supporting the AI/ML-aided L1 measurement based LTM should still be possible by keeping the inference function in the gNB-CU. Specifically, a possible approach is to enhance the legacy measurement reporting mechanism, e.g., the gNB-DU could be configured to forward the L1 measurement reports to the gNB-CU which, in turn, may use them as input to the AI/ML model. Or, alternatively, to make the L1 measurement results available at the gNB-CU, another way is to allow the UE to report L3 measurements derived from unfiltered L1 measurements to the gNB-CU (i.e., the filter coefficient of the L3 measurement could be adjusted to zero by the gNB-CU).

Proposal 2: RAN3 to agree that, to support the AI/ML-aided L1 measurement based LTM, the inference function is kept in the gNB-CU.

Proposal 3: RAN3 to discuss ways to enhance the legacy measurement reporting mechanism to enable inference in the gNB-CU for the AI/ML-aided L1 measurement based LTM.

## 2.2 Predicted information

This subsection covers the following remaining open aspects, that is:

Further discuss whether to study addition of beam prediction in UE Trajectory prediction. Further discuss predicting TA value in the next meeting. Further discuss prediction of validity time of a measured TA value.

In LTM, each candidate cell can be configured with up to 128 Transmission Configuration Indicator (TCI) states. During the early downlink synchronization procedure, multiple TCI states may be activated; however, only specific TCI state(s) are ultimately utilized via the cell switch command MAC CE. This results in redundant TCI state configurations and unnecessary activations in the initial synchronization phase.

The Rel-18 cell-based UE trajectory prediction is proposed to be used in this paper to assist cell selection and resource reservation. Nevertheless, this approach may be insufficient for LTM scenarios, where the selection of potential beams (TCI states) prior to cell switch is also required. Therefore, by considering beam-based predicted information e.g. in the UE trajectory prediction, it becomes possible to provide a more accurate TCI state configuration during LTM preparation and optimize TCI state activation in the early downlink synchronization procedure. Such an enhancement can effectively reduce redundant configurations and activations, thereby improving efficiency and resiliency of LTM operations. In our view, beam-based predicted information could be generated at the gNB-CU by, e.g., leveraging historical information about the accessed beams for the UE, in line with the agreed framework for the intra-CU LTM.

Proposal 4: RAN3 support to study addition of beam-level information predicted by the gNB-CU in the UE trajectory prediction.

In intra-CU LTM, after the early TA acquisition procedure, the gNB-CU can obtain TA values from each candidate gNB-DU(s) which determined the TA values of candidate cells controlled by such gNB-DU(s) in case of network-triggered TA acquisition. In fact, in case of inter-DU intra-CU LTM, the TA values are anyway provided to the gNB-CU which sends this information to candidate gNB-DU(s).

Observation 4: The existing LTM signalling framework already allows the gNB-CU to have visibility of the TA values determined via network-triggered TA acquisition.

By leveraging the existing LTM signalling framework, and by still keeping into consideration the general AI/ML framework, we think that the TA value can be predicted by the gNB-CU by leveraging historical TA values together with other related inputs (e.g., predicted UE trajectory, L3 measurement reports, previous LTM configurations etc.).

Proposal 5: RAN3 to agree that the gNB-CU can predict the TA values for candidate cells by leveraging historical TA values together with other related inputs (e.g., predicted trajectory, L3 measurement reports, etc.).

In the current LTM mechanism, the decision on the validity time of a measured TA value is considered as a gNB-DU implementation-specific information. Therefore, still based on the general principle previously stated that we should not change an already well-established AI/ML framework just for supporting a single aspect of an overall feature, and that locating the inference function in the gNB-DU is not an appropriate design choice due to the gNB-DU’s constraints in terms of computing resources, it should be assessed whether the gNB-CU could obtain sufficient information as input to the AI/ML model, so to predict the validity time of a measured TA value. In our view, similarly as per the TA value prediction, there are scenarios where the gNB-CU is provided with TA values (and corresponding validity times), e.g., in inter-DU intra-CU LTM.

Observation 5: The validity time of a measured TA is determined by the gNB-DU based on implementation.

Observation 6: There are scenarios in current LTM where the gNB-CU is provided with measured TA values and corresponding validity times as determined by the gNB-DU.

Therefore, by leveraging historical information (i.e., measured TA values and corresponding validity times) available at the gNB-CU along with other information such as the UE’s velocity and direction, we think that predicting the validity time of a measured TA value is feasible to be done in the gNB-CU.

Proposal 6: RAN3 to agree that the gNB-CU can predict the validity time of a measured TA value by leveraging at least historical measured TA values and corresponding validity times provided by the gNB-DU.

## 2.3 Other aspects

Intra-CU LTM is designed to enable low-latency cell switching by pre-configuring candidate cells for a UE and allowing the UE to perform early uplink/downlink synchronization with each candidate cell before the actual cell switch is executed. Once the gNB-CU decides to configure LTM for a UE, all candidate cells must reserve dedicated resources (e.g., configured grant resources, PRACH resources, CSI-RS) for that UE as soon as the gNB-CU instructs the candidate gNB-DU(s) to prepare for LTM. These reserved resources remain unavailable for other UEs until the candidate cell is explicitly released. If the UE does not switch to certain candidates, the pre-allocated resources in those cells remain unused, resulting in low resource utilization. In addition, a UE can be configured with up to eight LTM candidate cells, each reserving dedicated resources for that UE. As the UE eventually connects to only one target cell, resource reserved in multiple candidates may remain unused, leading to avoidable resource locking and reduced overall efficiency.

- Observation 7: Pre-configured LTM candidate cells must reserve dedicated resources for a UE until released, and if unused, these locked resources lead to poor overall utilization.

- Observation 8: A UE may be configured with up to eight LTM candidate cells, each reserving dedicated resources, but only one becomes the target, leaving several candidate cells’ resources unused and hence wasting capacity.

- An AI/ML model can be leveraged to determine the appropriate timing for each candidate gNB-DU to reserve specific resources for a UE ahead of the corresponding event (e.g., PRACH resources for early uplink synchronization, CG resources after the UE accesses the target cell). By leveraging time predictions for key LTM events such as cell switch or early synchronization, candidate gNB-DUs can reserve the necessary resources only when needed, according to implementation policy. Until the reservation is triggered, those resources remain available for other UEs or services, improving overall resource utilization and reducing unnecessary resource locking.

- Observation 9: Indicating predicted time information for key LTM events (e.g., early synchronization, cell switch) to candidate gNB-DUs enables gNB-DUs themselves to reserve LTM resources (e.g., PRACH for early synchronization, CG after target cell access) only when needed, hence minimizing resource waste.

- Proposal 7: RAN3 to agree to transfer the time information for key LTM events (e.g., early synchronization, cell switch) predicted by the AI/ML model located in the gNB-CU to candidate gNB-DUs for enabling optimized LTM resources reservation.

- In terms of the predicted time for cell switch, the predicted UE trajectory spanning cells belonging to different DUs but still controlled by the same CU – hence the same type of prediction introduced in Rel-18 as specified in TS 38.300 [4] – could provide information about the subsequent cells where the UE is predicted to connect. In particular, the expected time of stay in each cell predicted to be visited may be used to determine the time information for future subsequent cell switches.

- Proposal 8: RAN3 to agree that the expected time of stay in each cell of the predicted UE trajectory may be used to determine the time information for future subsequent LTM cell switches.

Based on Observation 8, to identify the LTM candidate cells which are more likely to become the actual serving cell in the future, the Rel-18 predicted cell-based UE trajectory can indicate the future UE mobility information which can be beneficial for the gNB-CU to determine the candidate cells for LTM or subsequent LTM events.

Proposal 9: RAN3 to agree that the list of candidate cells for LTM or subsequent LTM can be determined based on the Rel-18 predicted cell-based UE trajectory.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation1: The agreed framework for AI/ML-based LTM is in line with the general AI/ML for NG-RAN framework specified in Rel-18. Such framework perfectly works for L3 measurement based LTM.

Observation 2: A well-established framework should not be changed for the sole purpose of supporting the AI/ML-based version of another flavour of the same functionality.

Observation 3: Support for the AI/ML-aided L1 measurement based LTM may require the model inference function to be located in the gNB-DU which, however, is resource-constrained.

Proposal 1: RAN3 to acknowledge that the gNB-DU is resource-constrained, and hence it is not appropriate the locate the inference function there.

Proposal 2: RAN3 to agree that, to support the AI/ML-aided L1 measurement based LTM, the inference function is kept in the gNB-CU.

Proposal 3: RAN3 to discuss ways to enhance the legacy measurement reporting mechanism to enable inference in the gNB-CU for the AI/ML-aided L1 measurement based LTM.

Proposal 4: RAN3 support to study addition of beam-level information predicted by the gNB-CU in the UE trajectory prediction.

Observation 4: The existing LTM signalling framework already allows the gNB-CU to have visibility of the TA values determined via network-triggered TA acquisition.

Proposal 5: RAN3 to agree that the gNB-CU can predict the TA values for candidate cells by leveraging historical TA values together with other related inputs (e.g., predicted trajectory, L3 measurement reports, etc.).

Observation 5: The validity time of a measured TA is determined by the gNB-DU based on implementation.

Observation 6: There are scenarios in current LTM where the gNB-CU is provided with measured TA values and corresponding validity times as determined by the gNB-DU.

Proposal 6: RAN3 to agree that the gNB-CU can predict the validity time of a measured TA value by leveraging at least historical measured TA values and corresponding validity times provided by the gNB-DU.

- Observation 7: Pre-configured LTM candidate cells must reserve dedicated resources for a UE until released, and if unused, these locked resources lead to poor overall utilization.

- Observation 8: A UE may be configured with up to eight LTM candidate cells, each reserving dedicated resources, but only one becomes the target, leaving several candidate cells’ resources unused and hence wasting capacity.

- Observation 9: Indicating predicted time information for key LTM events (e.g., early synchronization, cell switch) to candidate gNB-DUs enables gNB-DUs themselves to reserve LTM resources (e.g., PRACH for early synchronization, CG after target cell access) only when needed, hence minimizing resource waste.

- Proposal 7: RAN3 to agree to transfer the time information for key LTM events (e.g., early synchronization, cell switch) predicted by the AI/ML model located in the gNB-CU to candidate gNB-DUs for enabling optimized LTM resources reservation.

- Proposal 8: RAN3 to agree that the expected time of stay in each cell of the predicted UE trajectory may be used to determine the time information for future subsequent LTM cell switches.

Proposal 9: RAN3 to agree that the list of candidate cells for LTM or subsequent LTM can be determined based on the Rel-18 predicted cell-based UE trajectory.

# 4. Reference

- RP-251868, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#108

- RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” (ZTE, NEC), 3GPP RAN#109

- 3GPP RAN3#129bis Chairman’s notes (RAN3_129bis_agenda_20251017_eom2.doc)

- 3GPP TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

# Annex – pCR for TR 38.745 (based on TR 38.745 V0.1.1 (2025-11))

<<<<<<<<<< Start of Changes >>>>>>>>>>

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

*** skipped unmodified text ***

#### 4.1.2.2		Input data of AI/ML based Intra-CU LTM

To optimize LTM performance, a gNB-CU may need the following information as input data for AI/ML assisted intra-CU LTM:

From local node:

-	Measured/Predicted radio resource status per cell/SSB area

-	Measured/Predicted UE trajectory

-	UE history information

-	Historical UE’s candidate LTM cell and beam list(s)

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level 		UE measurements

<<<<<<<<<< Next Change >>>>>>>>>>

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

-	Candidate cell and beam for LTM HO Preparation

-     Predicted time of stay of the UE in each predicted candidate LTM cell and beam

-	Predicted time for key LTM events (e.g., cell switch, early synchronization)

#### 4.2.2.4	Feedback of AI/ML assisted Intra-CU LTM

To optimize the performance of the AI/ML model used for AI/ML assisted intra-CU LTM, the following feedback can be considered to be collected from gNB-DU(s):

-	Actual LTM cells and beam for each cell switch event

-    Actual time of stay of the UE in each LTM cell and beam of each cell switch event

-    Actual time of occurrence of key LTM events (e.g., cell switch, early synchronization)

#### 4.2.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-20 normative work compared with what was specified during Rel-19.

F1 interface:

- Enable transfer of the following predicted information from CU to DU:

- Predicted candidate LTM cell(s) and beam(s) of the corresponding DU

- Predicted time of stay of the UE in each predicted candidate LTM cell and beam of the corresponding DU

- Predicted time for key LTM events (e.g., cell switch, early synchronization) in the corresponding DU

- Enable transfer of the following measured/actual information from DU to CU:

- Actual LTM cell(s) and beam(s) of the corresponding DU for each cell switch event

- Actual time of stay of the UE in each LTM cell and beam of the corresponding DU for each cell switch event

- Actual time of occurrence of key LTM events (e.g., cell switch, early synchronization) in the corresponding DU for each cell switch event

<<<<<<<<<< End of Changes >>>>>>>>>>