3GPP TSG-RAN WG3 Meeting #130	R3- 258501

Dallas, US, 17 – 21 November 2025

Agenda item:	13.2

Source:	Hanbat National University

Title:	Network architecture enhancements for NR ISAC

Document for:	Discussion

Introduction

The study item on “Study on integrated sensing and communications (ISAC) for NR” was approved in RAN#108 meeting and it contains the network architecture aspects in its scope [1], as follows:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we further discuss considerations for network architecture aspects to support NR ISAC.

Architecture enhancements for ISAC

At RAN3#129-bis meeting, the ISAC network architecture discussion raised several issues around how a gNB reports sensing information to the core network. The handling of the protocol stack for sensing reporting was left FFS pending decision from SA2’s conclusion. RAN3 is now expected to discuss the protocol stack solution for sensing data reporting in alignment with the stage-2 architecture. SA2’s ongoing work has defined the Sensing Function (SF) as a new core network function and the gNB as a Sensing Entity (SE).

During RAN3#129-bis, several options were discussed for transporting sensing reports from the gNB (Sensing Entity) to the core network’s Sensing Function.

- Option 1: New Interface with Dedicated Protocol

- Option 2 (Ambient IoT-like Reuse of Existing Signaling)

- Option 3 (NRPPa-Like Application Protocol over NGAP)

Option 1 defines a completely new interface between the gNB and the SF, carrying a specialized sensing application protocol over new transport layers. This direct interface approach eliminates involvement of intermediate nodes (e.g., AMF), offering the shortest signaling path and potentially lower latency for time-critical sensing services. However, it would require defining an entirely new protocol stack and procedures from scratch, increasing standardization and implementation effort. Option 2 reuses the existing 5G core interfaces and protocols for sensing, treating sensing as an extension of current network services. The SF residing in the core would communicate with the AMF over standard service-based interfaces, and the AMF would forward or host new NGAP message extensions for sensing data toward the gNB. This approach mirrors the strategy used for Ambient IoT in Rel-19, where new device types and functions were supported over existing NG interfaces with minimal changes. Option 3 defines a new application-layer protocol called NrxPa, analogous to how NR Positioning Protocol (i.e., NRPPa) works for location services. In this model, the gNB and the SF would exchange dedicated sensing messages end-to-end, but these messages are encapsulated within NGAP signaling over the existing N2 interface.

With the respect the above three options, we need to consider the following aspects:

- Alignment with SA2 Architecture: SA2 has considered both direct and indirect SF-gNB interfaces. RAN3 can stay consistent with their conclusions.

- Low Specification and Implementation Impact: Reusing the NG control plane greatly minimizes new standardization efforts. It will also offer backward compatibility and incremental extension of a proven protocol.

- Sufficient Performance for Rel-20 Use Cases: In the gNB-based monostatic sensing scenario, it is expected that data volumes are moderate, which can be considered to decide direct vs. indirect interface.

Proposal 1: RAN3 is encouraged to consider the following aspects for the network architecture study for NR ISAC:

- Alignment with SA2 Architecture

- Low Specification and Implementation Impact

- Sufficient Performance for Rel-20 Use Cases

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN3 is encouraged to consider the following aspects for the network architecture study for NR ISAC:

- Alignment with SA2 Architecture

- Low Specification and Implementation Impact

- Sufficient Performance for Rel-20 Use Cases

References

- RP-251861 “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108.