3GPP TSG-RAN3 Meeting #130	R3-258504

# Dallas, US, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of OAM support for on-demand SIB1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Telecom, Jio Platforms, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the Rel-19 WI on enhancements of NES for NR, stage-2 details for support of on-demand SIB1 were introduced in TS 38.300. The potential ambiguity of OAM aspect regarding the Cell A can be further clarified in stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification that the cell(s) transmitting the OD-SIB1 configuration for a cell supporting OD-SIB1 may be chosen by the requesting gNB. If not indicated, it can be determined by OAM configuration in the assisting gNB. - Addition of an optional OAM requirement on configuring the cell(s) transmitting the OD-SIB1 configuration for a cell supporting OD-SIB1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification may cause ambiguity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.4.2.9, 15.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

BEGIN OF CHANGE

## 15.4	Support for Energy Saving

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 15.4.2.9	On-demand SIB1

To facilitate reducing gNB downlink transmissions, instead of always periodically transmitting SIB1, the gNB can provide on-demand SIB1, i.e., upon receiving an OD-SIB1 request from a UE supporting OD-SIB1. OD-SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, where MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. OD-SIB1 request configurations of one or more cells which support OD-SIB1 are included in SIB26, which can be broadcasted in any cell, including cell’s own OD-SIB1 request configuration. UE may request SIB1 based on the OD-SIB1 request configuration from SIB26 in order to determine the suitability of a cell during and after cell reselection as specified in TS 38.331 [12].

A gNB may request neighbour gNB(s) over the Xn interface to transmit or stop transmitting the OD-SIB1 configuration for a cell supporting OD-SIB1. The requesting gNB may indicate the particular cell(s) of the neighbour gNB(s) to transmit the OD-SIB1 configuration. If not indicated, the neighbour gNB(s) select(s) the appropriate cell(s) based on O&M configuration. The neighbour gNB(s) deciding to stop the OD-SIB1 configuration transmission informs the requesting gNB over Xn. The inter-gNB coordination procedures are defined in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 15.4.3	O&M requirements

Operators should be able to configure the energy saving function.

The configured information should include:

-	The ability of an NG-RAN node to perform autonomous cell switch-off;

-	The ability of an NG-RAN node to request the re-activation of a configured list of inactive cells owned by a peer NG-RAN node.

O&M may also configure:

-	Policies used by the NG-RAN node for cell switch-off decision;

-	Policies used by peer NG-RAN nodes for requesting the re-activation of an inactive cell;

-	The minimum time an NG-RAN node's cell should remain activated upon reception of a re-activation request from an eNB;

-	The cells of an NG-RAN node that should transmit the OD-SIB1 configuration for a cell supporting OD-SIB1.

END OF CHANGE