3GPP TSG-RAN WG3 Meeting #130													R3-258523

Dallas, US, 17th-21st November, 2025

Agenda item:		10.5.1

Source:				Jio Platforms

Title:					AI/ML-based use cases and unified framework for 6G

Document for:		Discussion

# Introduction

During the 3GPP TSG-RAN WG3 Meeting #129bis, held in Prague, the following items were discussed.

High-level principles for AI/ML RAN in 6G:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope.

AI/ML based mobility optimization and network energy saving propose to be supported with high priority in 6G Day 1, since the network operation and user experience guarantee are two critical aspects for operators.

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

Other use cases can be discussed in future meetings based on contributions.

In 6G AI/ML study, the following new use cases can be considered:

-	AI/ML assisted QoE

-	Cross-domain collaboration for AI/ML

-	Agentic AI

- AI computing resource management

- AI/ML-enabled RAN Digital Twin for leveraging environmental data

- AI/ML-assisted resilience

- Network for AI

This document highlights that current 5G AI/ML frameworks (Rel-17/18) are isolated, leading to conflicting actions. It proposes a unified 6G framework to allow key use cases (NES, LB, CCO) to share measurements and coordinate decisions.

# Discussion

Earlier releases (Rel-18 and Rel-19) worked on features like Network Energy Saving (NES), Load Balancing (LB), and Coverage & Capacity Optimization (CCO) — but each worked separately as isolated loops.

This caused issues such as conflicting actions, redundant signalling, and sub-optimal performance.

To solve this, 6G aims to build a unified AI/ML framework that coordinates all these functions together.

This framework will also include Federated Learning, Slice-aware optimization, cross-domain energy coordination, and AI/ML model lifecycle management so that AI/ML functions across the RAN, transport, and core network can work together smoothly.

## Unified Framework for AI/ML‑led RAN3 Use Cases

In Release 18, RAN3 enabled AI/ML support in NG-RAN for Network Energy Saving (NES), Load Balancing (LB) and Mobility Optimization, primarily by specifying data-collection and reporting over existing interfaces (e.g., Xn) to feed model training/inference. In Release 19, the scope expanded to AI/ML-assisted Network Slicing and AI/ML-assisted Coverage & Capacity Optimization (CCO) (together with split-architecture and continuous-MDT support).

The proposed unified framework builds upon Rel-19 AI/ML architectural hooks (data collection, model inference signalling), extending them toward multi-objective coordination and slice-aware intelligence in 6G. While these capabilities allow AI/ML to be applied to each use case, Rel-18/19 did not define a multi-use-case coordinator; in deployments the loops typically execute independently, each maintaining its own inputs, models and actuation logic.
This fragmented approach introduces several operational challenges:

- Conflicting actions: A NES algorithm might deactivate a low-traffic cell to save power at the same time a LB algorithm tries to redirect users to that cell, resulting in ping-pong behaviour and lost energy savings.

- Redundant signalling: Independent loops often request similar measurements and issue overlapping control messages, increasing signalling load and system overhead.

- Limited holistic optimization: Since NES, LB, and CCO share common inputs (traffic load, energy consumption, mobility patterns), optimizing them separately can lead to inconsistent or suboptimal outcomes.

- No conflict resolution: There is no standardized mechanism for resolving competing objectives across these loops, leading operators to depend on vendor-specific heuristics.

These issues arise from treating NES, LB, MOB, and CCO as isolated SON functions, despite their interdependencies: energy saving often requires user offloading (LB/MOB), which in turn affects radio coverage and quality (CCO). A unified optimization framework is thus required to coordinate these objectives effectively, aligning with Rel-19's emphasis on AI/ML integration for NG-RAN.

Key technical requirements identified are:

- Common Data Layer: All functions rely on shared measurements, such as real-time cell load, predicted traffic demand, energy usage, and UE mobility statistics. A standardized shared data model and prediction interface (e.g., extending Rel-18 IEs in F1AP/XnAP) enables consistent inputs for AI/ML models, reducing duplication and enhancing reliability.

- Multi-Objective Decision Engine: A single AI/ML decision engine at the gNB (e.g., in the CU) should evaluate NES, LB, and CCO objectives together. It can generate an optimized action plan balancing energy savings, load distribution, and coverage quality, guided by operator-defined policy weights or slice-specific priorities.

- Unified Control Procedures: Decisions must be executed atomically across the network to prevent inconsistent actions. A unified control procedure over Xn/F1/E1 interfaces can deliver composite instructions — such as cell on/off commands, offload targets, and coverage tuning parameters — ensuring all affected nodes act coherently.

- Model versioning and identification: Each AI/ML model should carry a unique identifier and version so that the network can manage multiple models and track updates.

- Performance reporting: UEs and gNBs should report AI/ML performance metrics (e.g. prediction error, energy savings achieved, QoS impacts) back to the training function.

- Feedback triggers: If performance falls outside defined thresholds, the network should automatically trigger retraining or switch to a fallback mode.

- Coordinated fallback: When a gNB falls back from AI/ML operation to deterministic control, neighbours must be notified to adjust their own actions. This ensures consistent operation across the network.

Given the distributed nature of AI/ML deployment across layers, coordination between RAN1 (signal and beam optimization), RAN2 (control and RRC procedures), and RAN3 (interface and data exchange mechanisms) remains critical. The unified framework can serve as the architectural anchor to ensure consistency across these domains.

Figure 1(a) (b) Unified Framework for AI/ML‑led RAN3 Use Cases

Observation 1: Energy saving, load balancing, mobility, and coverage control exhibit strong interdependencies that necessitate coordinated, policy-driven optimization.

Proposal 1: It is proposed to RAN3 to study the development of a unified, multi-objective AI/ML framework with discrete goals for the identified use cases, based on a shared data model to minimize signalling overhead,

Proposal 2: RAN3 is proposed to define unified control procedures to execute coordinated, atomic actions across nodes according to operator-weighted objectives.

Proposal 3: Model lifecycle aspects (Model-ID/versioning, performance reporting, feedback triggers, coordinated fallback) should be included in the framework and aligned with RAN2’s AI/ML lifecycle signalling.

The proposed AI/ML framework aligns with the architectural objectives defined for TR 38.760, particularly regarding functional modularity, cross-domain intelligence, and energy-efficient operation

## AI/ML Use Cases in RAN3

Building on the unified framework principles in Section 2.1, this section outlines the key AI/ML use cases within RAN3’s scope for 6G, identifying their interdependencies and coordination requirements under the proposed common data and control framework.

RAN3’s initial AI/ML study (TR 37.817, Rel-17) explicitly focused on NES, LB, MRO, and CCO as high-priority use cases for NG-RAN intelligence. However, these were analyzed and enhanced individually in Rel-18/19, with limited consideration of interactions among themselves or with RAN2-led (e.g., RRC-based mobility prediction) or RAN1-led (e.g., beam management) AI/ML features.

The 6G AI/ML landscape should expand these intelligent functions, building on 5G-Advanced foundations while introducing predictive, slice-aware, and cross-domain capabilities:

- Network Energy Saving (NES): reduce RAN energy consumption by deactivating carriers, beams or cells while maintaining service quality. Current design relies on thresholds and simple heuristics; predictive metrics are missing.

- Load Balancing (LB): redistribute traffic to avoid congestion. Existing algorithms are reactive and focus on handover offsets; AI/ML can predict load shifts and coordinate with NES to avoid reactivation of dormant cells.

- Coverage & Capacity Optimization (CCO): adjust beamforming, power and tilt to improve coverage and capacity. AI/ML can proactively prevent coverage holes when NES and LB actions change network topology.

- Mobility Optimization: predict UE trajectories and select handover targets. RAN2 and RAN3 studied separate approaches, but coordination is lacking.

- Network Slicing: allocate resources based on per‑slice SLA requirements. AI/ML can monitor slice performance and adapt RAN configurations (e.g. NES aggressiveness) accordingly.

Currently, RAN3-studied AI/ML use cases in Rel-18/19 do not fully specify slice-aware optimization, a dedicated model lifecycle management procedure, or cross-domain collaboration (e.g., integrating core network traffic data via SA2/SA5). Moreover, prioritization and coordination among multiple objectives (e.g., NES vs. MRO trade-offs) lack clear mechanisms, relying on vendor-specific policies.

For 6G, RAN3 must delineate standardized AI/ML use cases and their interactions, per Rel-20 scoping (post-RAN#107, March 2025). NES, LB, MRO, and CCO remain core, with potential additions like QoE optimization, UE energy saving (e.g., AI/ML-assisted DRX), and cross-domain AI/ML cooperation. RAN3 should define its scope: RAN3-led functions (e.g., NES, LB, CCO, RAN slicing interfaces), RAN2-led (e.g., mobility prediction, RRM measurement), and coordination protocols. Overlapping data needs (e.g., UE trajectories for MRO and slicing) highlight the value of a shared framework.

A common use-case taxonomy can refine this scoping, aligned with 3GPP's AI4Net (AI/ML empowering network efficiency) and Net4AI (network supporting AI/ML services). For example, RAN3-led AI/ML use cases could group into: (i) RAN resource optimization (NES, LB, CCO, RAN slicing), (ii) mobility and handover optimization (coordinated with RAN2), and (iii) cross-domain AI/ML cooperation (liaised with core network or OAM via SA5). Each can leverage the unified framework proposed in Section 2.1 for shared data collection and multi-objective inference.

Observation 2: Core 6G RAN3 use cases (i.e. NES, LB, MRO, CCO) require updates for predictive, slice-aware, and cross-domain operations, including inter-WG coordination (e.g., RAN3 actuation vs. RAN2 signaling) along with elevating model lifecycle management towards a standardized RAN3 procedure.

Proposal 4: RAN3 is proposed to define core 6G RAN3 AI/ML use cases (NES, LB, MRO, CCO, etc.) with a clear WG taxonomy, enabling coordinated, slice-aware optimization via per-slice policies.

Proposal 5: RAN3 to study a standardized model lifecycle management for seamless deployment, updates, and feedback across NG-RAN.

Moreover Agentic AI represents the next evolution of network intelligence, where autonomous AI agents perform goal-oriented reasoning, collaborate across network domains, and proactively adapt network configurations. In the 6G context, such agents could autonomously coordinate between NES, LB, CCO, and mobility functions, negotiating trade-offs and executing composite actions under operator-defined policies.

Unlike traditional model-inference loops, Agentic AI systems incorporate planning, reasoning, and interaction capabilities, allowing them to decompose high-level objectives (e.g., “minimize total energy consumption while preserving per-slice QoS”) into executable sub-goals distributed across gNBs.

Integrating Agentic AI into the unified framework requires standardized interfaces for inter-agent communication, shared situational awareness (common data context), and safety constraints to prevent conflicting or unstable actions. Such an approach could further enhance adaptability, reduce manual coordination, and pave the way for a self-evolving RAN.

Observation 3: Agentic AI enables autonomous and goal-driven coordination across multiple RAN functions (NES, LB, CCO, MRO), but requires standardized mechanisms for agent collaboration, goal negotiation, and safe execution within the unified AI/ML framework.

Proposal 6: RAN3 is proposed to study the incorporation of Agentic AI principles into the unified 6G RAN intelligence framework, defining inter-agent communication interfaces, shared context models, and control safety policies to support autonomous, goal-oriented network operation.

## Rel‑19 NES Challenges and Proposed Solutions

Release 19 introduced baseline energy saving features (e.g. on‑demand SIB1, micro‑sleep of carriers, energy consumption metrics) and AI/ML hooks (predictive CCO and energy cost reporting). Nonetheless, several critical gaps remain:

- Reactive thresholds and rules: Current NES algorithms rely on simple thresholds (e.g. cell load below X%) to trigger sleep/wake decisions. This causes frequent toggling (dozens of on/off events per day), wasted energy and unstable performance.

- Static energy savings: Techniques such as OD‑SIB1 reduce broadcast overhead but do not exploit deeper savings (e.g. beam‑level or sub‑slot sleep); they also lack predictive logic to decide when to sleep.

- No predictive data: There is no standardized way to exchange predicted load, energy cost or user mobility information across nodes. gNBs make NES decisions based on current state rather than future conditions.

- Fragmented coordination: NES decisions are taken locally without consulting load balancing or CCO functions. There is no unified procedure to prepare neighbor cells for offloading or to adjust coverage accordingly.

- Limited sleep granularity: Current standards define cell/carrier sleep, but do not support fine‑grained states such as beam‑level or sub‑slot deactivation, which could unlock further energy savings.

- No slice awareness: NES actions do not consider slice‑specific SLA requirements (e.g. URLLC slice may forbid sleeping a cell) or energy budgets per slice.

Overcoming these limitations requires both predictive intelligence and enhanced interface capabilities. NES decisions should be based on predicted metrics such as predicted cell load, predicted energy consumption and predicted mobility patterns. Exchanging such predictions over Xn and F1 allows nodes to coordinate and avoid premature or unnecessary deactivations. AI/ML models can forecast both traffic and energy costs and compute the optimal sleep/wake schedule, reducing toggling from ~50 times/day to ~5 times/day in simulations.

The NES framework should also adopt fine‑grained sleep states: beam‑level or sub‑slot dormancy can save energy while maintaining baseline coverage. Fine‑grained states require gNB‑DU support and new information elements to signal partial activation.

Integration with LB and CCO is essential: deactivating a cell must trigger load redistribution and coverage adjustments. This implies a new procedure on Xn (e.g. EnergySavingRequest/Response) whereby a node announces its intent to sleep, neighbours provide feedback on their capacity to absorb traffic, and the network collectively agrees on the action.

Many existing energy-saving proposals (like increasing SSB periodicity) reduce power usage at the network side but increase power drain at the UE side because UEs must wait longer or repeat searches to find the cell. 5GA lacks a standard metric to measure combined UE + network energy efficiency. For 6G, we propose defining a Total Energy Efficiency Index (TEE)—a holistic KPI that accounts for both network-side energy savings and the cost borne by UEs. This metric can be derived as the ratio of useful data throughput to the sum of network and UE power consumption per bit. A holistic KPI (e.g., Total System Energy Efficiency = Network Power + UE Power per bit throughput) is missing.

Finally, slice‑aware NES should be enabled. Each slice can specify an energy saving aggressiveness or prohibitions (e.g. critical slices forbidding sleep). NES decisions must respect these policies, potentially by weighting the importance of serving each slice versus energy reduction.

Figure 2 Proposed NES Framework

Observation 4: NES requires deep-sleep AI/ML-driven control, cross-function coordination with LB/CCO, and integration of slice-specific policies to protect QoS in order to fix any potential reactive inefficiencies due to a lack of proactive, predictive metrics, which causes excessive toggling.

Proposal 7: RAN3 is proposed to standardize predictive metrics and new cooperative procedures for cell sleep requests between gNBs, integrated into a unified loop with LB/CCO,

Proposal 8: RAN3 is proposed to support slice-aware policies/metrics that combines both UE and network energy consumption per useful bit transmitted.

In summary, the proposed unified AI/ML framework enables coordinated optimization across RAN functions, improving efficiency, stability, and adaptability. It also paves the way for predictive, slice-aware intelligence consistent with 6G architectural design principles under TR 38.760. Further discussions may refine data model extensions (e.g., prediction information elements) and cross-WG coordination procedures.

# Conclusion

Observation 1: Energy saving, load balancing, mobility, and coverage control exhibit strong interdependencies that necessitate coordinated, policy-driven optimization.

Proposal 1: It is proposed to RAN3 to study the development of a unified, multi-objective AI/ML framework with discrete goals for the identified use cases, based on a shared data model to minimize signalling overhead,

Proposal 2: RAN3 is proposed to define unified control procedures to execute coordinated, atomic actions across nodes according to operator-weighted objectives.

Proposal 3: Model lifecycle aspects (Model-ID/versioning, performance reporting, feedback triggers, coordinated fallback) should be included in the framework and aligned with RAN2’s AI/ML lifecycle signalling.

Observation 2: Core 6G RAN3 use cases (i.e. NES, LB, MRO, CCO) require updates for predictive, slice-aware, and cross-domain operations, including inter-WG coordination (e.g., RAN3 actuation vs. RAN2 signaling) along with elevating model lifecycle management towards a standardized RAN3 procedure.

Proposal 4: RAN3 is proposed to define core 6G RAN3 AI/ML use cases (NES, LB, MRO, CCO, etc.) with a clear WG taxonomy, enabling coordinated, slice-aware optimization via per-slice policies.

Proposal 5: RAN3 to study a standardized model lifecycle management for seamless deployment, updates, and feedback across NG-RAN.

Observation 3: Agentic AI enables autonomous and goal-driven coordination across multiple RAN functions (NES, LB, CCO, MRO), but requires standardized mechanisms for agent collaboration, goal negotiation, and safe execution within the unified AI/ML framework.

Proposal 6: RAN3 is proposed to study the incorporation of Agentic AI principles into the unified 6G RAN intelligence framework, defining inter-agent communication interfaces, shared context models, and control safety policies to support autonomous, goal-oriented network operation.

Observation 4: NES requires deep-sleep AI/ML-driven control, cross-function coordination with LB/CCO, and integration of slice-specific policies to protect QoS in order to fix any potential reactive inefficiencies due to a lack of proactive, predictive metrics, which causes excessive toggling.

Proposal 7: RAN3 is proposed to standardize predictive metrics and new cooperative procedures for cell sleep requests between gNBs, integrated into a unified loop with LB/CCO,

Proposal 8: RAN3 is proposed to support slice-aware policies/metrics that combines both UE and network energy consumption per useful bit transmitted.

# References

- 3GPP TR 38.843 – Study on AI/ML for NR (Rel-18/19)

- 3GPP TR 23.754 – Study on Network Energy Efficiency for 5GS

- 3GPP TS 28.104 – Energy Efficiency Management

- 3GPP TR 38.840 – Study on NR Energy Saving

- 3GPP TS 38.300 – NR overall description

- 3GPP TS 23.503 – Policy and charging control framework

- 3GPP TR 23.700-70 – AI/ML-based RAN optimization

- O-RAN Alliance WG2/WG3 specifications for AI/ML and RIC integration