3GPP TSG RAN WG3 Meeting #130	R3-258533

Dallas, TX, USA, 17th – 21st November, 2025

Agenda item:	13.3

Source:	Ericsson

Title:	Discussion on RAN-CN procedures and signaling to support ISAC with TP to pCR

Document for:	Other

# 1	Introduction

In [1], a study item on Integrated Sensing and Communication (ISAC) for NR was approved listing the following RAN3 related objectives:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] - Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In RAN3#129-bis meeting, RAN3 discussed basic signalling to support ISAC and a Sensing Reporting procedure has been captured in [2].

This contribution discusses the sensing procedure and the interface’s functions in more details.

# 2	Discussion

## 2.1	Sensing Reporting Procedures

In [2], the sensing reporting is described as below with an EN stating that further refinement is needed:

## 8.x Sensing Reporting Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF. <<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

In the SENSING REQUEST message, the SF provides information to assist the gNB in the sensing measurement task. For instance, the SF can include the following sensing service requirements: Target Sensing Area, Target Type (e.g., ‘drone’, …), Reporting mode for sensing results (e.g. one-time or periodic reporting ), FFS on other information.

Proposal 1: In the SENSING REQUEST message, the SF includes assistance information for gNB sensing task, such as: Target Sensing Area, Target Type (e.g., ‘drone’, …), reporting mode for sensing results (e.g. one-time or periodic reporting).

When the gNB receives the SENSING REQUEST message, it can take the information therein into account to determine how to configure the relevant sensing measurements, also based on the local resource conditions of the gNB. For example, depending on the current interference and hardware limitations encountered by the gNB, it may decide to select two TRPs, one for transmission and one for reception, rather than a single TRP, in order to avoid the self-interference problem [3]. Therefore, it is always up to the gNB implementation to decide on the selection and configuration of TRPs, as the SF does not have visibility of local gNB information (e.g., the level of self-interference), nor does it need to.

Proposal 2: It is up to gNB implementation to make decision on TRP(s) selection and configuration.

For simplicity, we assume that the gNB can perform the requested sensing measurement and obtain the final results in the requested target sensing area based on RAN processing. The list of results would include a list of objects and, for each object detected as present in the requested target sensing area, the gNB includes the estimated 3D velocity and estimated 3D position. The list of results is included in the SENSING RESPONSE message if a single report is requested, or in the SENSING REPORT message if periodic reports are requested.

Proposal 3: When the gNB is determining the final sensing results, the gNB provides a list of sensing results (e.g., 3D velocity estimate and 3D position estimate) per detected object for a list of objects in the SENSING RESPONSE and SENSING REPORT messages.

Note: whether the SF in CN can perform further processing, e.g., to store and filter the targets from the reported objects, is up to SA2 group.

In case of measurement failure, RAN3 signalling should enable the gNB to reply with a SENSING FAILURE message, that is part of the sensing reporting procedure (in case of one-time reporting); or a SENSING FAILURE INDICATION (in case of periodic reporting) from the gNB to SF.

Proposal 4: RAN3 to support the Sensing Failure for both reporting modes (one-time and periodic)

During Periodic Reporting, the SF should be able to abort an on-going sensing task with a dedicated message, e.g., SENSING ABORT message.

Proposal 5: The SF may abort an ongoing Sensing task by sending a SENSING ABORT message.

We propose a TP to the pCR in Annex of this paper to capture the above proposals.

## 2.2	New Interface management procedures between gNB and SF for Direct interface connection

At the last meeting, it was discussed online that the need for management procedures for the new Nx interface between the SF and the gNB should be investigated. In fact, to support the exchange of signalling messages between the SF and the gNB, the SF needs to be aware of NG-RAN address and its sensing coverage area, so that the nodes can correctly interoperate.

Proposal 6: The SF and gNB need to be aware of each other’s relevant information (e.g., sensing coverage area) via management procedures for the Nx interface.

At first, the gNB can be provided with SF-related information based on OAM-provisioned information. The gNB initiates an association establishment request with SF by sending an Association Establishment Request procedure indicating the gNB’s covered sensing area(s). One example of such procedure is described in Figure 2 below.

Fig. 2: NG-RAN initiates the association establishment towards SF. - The NG-RAN is configured with the SF address by OAM. - The NG-RAN sends an Association Establishment Request to the SF for the sensing service. It includes (among other parameters) its sensing area(s), which is stored by SF. - The SF sends an acknowledgement message to indicate the successful establishment of the association with the NG-RAN. - The SF and the NG-RAN use the established association for further sensing procedures.

In case, e.g., the covered sensing area(s) of the NG-RAN has changed, the NG-RAN sends a Configuration Update request message to the SF. The details are provided below in Figure 3.

Fig.3 Configuration Update of the direct sensing connection between SF and NG-RAN. - The NG-RAN sends a sensing Configuration Update request to the SF if, e.g., the covered sensing area(s) of NG-RAN node has been changed. The SF updates its stored sensing area if it is required. The SF sends an acknowledgement to the gNB to indicate the successful configuration update.

Finally, the gNB may decide to remove the direct sensing association with the SF, e.g., to save on SCTP transport resources in case no signalling is needed. In that case, A Sensing Association Removal Procedure will be needed. The details are described in Figure 4 below.

Fig.4 Removal of the direct sensing connection between SF and NG-RAN. - The NG-RAN sends a Sensing Association Removal request to the SF. The SF updates its stored serving sensing area if it is required. The SF sends an acknowledgement message to NG-RAN about the removal.

Proposal 7: RAN3 to study dedicated procedures for Nx Sensing interface management: e.g., Association Establishment, Configuration Update and Association Removal between the NG-RAN and SF to support signalling connection setup, update and removal.

# 3	Conclusion

This contribution has provided the following observations and conclusions:

Proposal 1: In the SENSING REQUEST message, the SF includes assistance information for gNB sensing task, such as: Target Sensing Area, Target Type (e.g., ‘drone’, …), reporting mode for sensing results (e.g. one-time or periodic reporting).

Proposal 2: It is up to gNB implementation to make decision on TRP(s) selection and configuration.

Proposal 3: When the gNB is determining the final sensing results, the gNB provides a list of sensing results (e.g., 3D velocity estimate and 3D position estimate) per detected object for a list of objects in the SENSING RESPONSE and SENSING REPORT messages.

Proposal 4: RAN3 to support the Sensing Failure for both reporting modes (one-time and periodic).

Proposal 5: The SF may abort an ongoing Sensing task by sending a SENSING ABORT message.

Proposal 6: The SF and gNB need to be aware of each other’s relevant information (e.g., sensing coverage area) via management procedures for the Nx interface.

Proposal 7: RAN3 to study dedicated procedures for Nx Sensing interface management: e.g., Association Establishment, Configuration Update and Association Removal between the NG-RAN and SF to support signalling connection setup, update and removal.

# 4	Reference

- RP-251861, Study on Integrated Sensing And Communication (ISAC) for NR

- SA2 TR 23.700-14 V0.3.0, Study on Integrated Sensing and Communication; Stage 2, (Release 20)

- Y. Lyazidi et al., “ISAC Architecture and Sensing Topology Switching in 6G Cellular Networks”, in proceedings of IEEE CSCN 2024, November 2024. DOI: 10.1109/CSCN63874.2024.10849754

# 5	TP to TR 38.765

START OF CHANGES

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x1	Sensing Reporting

Sensing Reporting Procedures are used to transfer information for sensing measurement request, sensing results reporting and sensing measurement abortion.

Figure 8.x1-1: Message flow for sensing service procedures

1.	The SF sends a Sensing Request to the gNB, including Target Sensing Area, requested reporting mode for sensing results, and requested object type for sensing.

2.	The gNB makes decision on TRP(s) selection and measurements configuration. The gNB then provides a list of sensing results (e.g., 3D velocity estimate and 3D position estimate) per detected object for a list of objects in the Sensing Report message. In case of failure, the gNB sends a Sensing Failure message.

3.	If requested in Step 1, the gNB sends additional sensing reports to the SF in a Sensing Report message. In case of failure, the gNB sends a Sensing Failure Indication message.

4.	In case the SF decides to terminate the on-going Sensing Report, it sends a Sensing abort message.

## 8.x2	Sensing Management Procedures

The Sensing Management Procedures are used to establish an association between the SF and gNB prior to the Sensing Request, update the association or remove it.

Figure 8.x2-1: Message flow for Association Establishment, configuration and removal between gNB and SF

- The gNB is configured with the SF address by OAM.

2.	The gNB sends an Association Establishment Request to the SF for the sensing service and includes its sensing area(s).

3.	The SF sends an acknowledgement message to indicate the successful establishment of the association with the gNB.

4.	The SF uses the established association to initiate the Sensing Reporting procedure as described in clause 8.x1.

5.	In case the covered sensing area(s) of the gNB has changed, the gNB sends a Sensing Configuration Update request to the SF.

6.	The SF updates its stored sensing area and sends an acknowledgement to the gNB to indicate the successful configuration update.

7.	In case the gNB decides to remove the association, it sends an Association Removal request to the SF.

8.	The SF sends an acknowledgement message to gNB to confirm the removal.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

Rel-20 is limited to drone detection. This information can be extended in the future for sensing other target types (e.g., humans, vehicles, etc.)

Object detected as “absent” is still considered a sensing result and by no means a sensing failure. In such scenario, the velocity and position estimates are not provided.