3GPP TSG-RAN WG3 Meeting #130	R3-258600

Dallas, US, 17th – 21st November, 2025

Agenda Item:	12.2.1

Source:	CMCC

Title:	(pCR to TR 38.745) Discussion on AI/ML based Multi-hop UE trajectory across gNBs

WID/SID: 	FS_NR_AIML_NGRAN_Ph3

Document for:	Approval

# 1 Introduction

In RAN3#129bis, the following agreements were achieved for Multi-hop UE trajectory across gNBs in AI/ML-based mobility [1].

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

The following solutions are considered for supporting multi-hop UE trajectory:

-  AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-  AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-  AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-  AI/ML Model Training and Model Inference are both located in the gNB-CU.

Focus on cell-based granularity for measured and predicted multi-hop UE trajectory.

Study multi-hop UE trajectory for L3 HO.

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

The open issue was captured in last meeting as below and need to be continued in next meeting:

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

Configuration of measurement feedback.

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

In this contribution, we would continue the discussion on the solutions of AI/ML based multi-hop UE trajectory across gNBs and some proposals.

# 2 Discussion

2.1 Prediction information transmission

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

Regarding to the prediction transmission for Multi-hop UE trajectory across gNBs, the AI/ML model in source node has the capability to generate the multi-hop UE trajectory prediction which may include a list of cells related to multiple target nodes. That means the prediction information should be transmitted from the source node to the related target nodes in time. We think there may be two options:

- Option 1 (Hop-by-hop): The source node transmits the multi-hop prediction information to the first hop Target node (e.g., Target node-1 in Figure 1). And then Target node-1 convey the prediction information to its next hop Target node (e.g., Target node-2) when Target node-1 make sure UE will handover to Target node-2. And then Target node-2 send the related prediction information to its next hop Target node (e.g., Target node-3) when Target node-2 make sure UE will handover to Target node-3. That means the prediction information will be transmitted from one node to the next hop node in order.

Figure 1: UE trajectory prediction transmission-Hop-by-hop

- Option 2 (Point-to-multi-point): When the source node generates the multi-hop UE trajectory prediction, it transmits the prediction information to every potential target node directly (Option 2 is illustrated in Figure 2).

Figure 2: UE trajectory prediction transmission-Point-to-multi-point

Compare the above two options, we think Option 1 (Hop-by-hop) has more advantages. For example, for Hop-by-hop, if Target node-1 does not handover the UE to Target node-2, the prediction information will not be sent to Target node-2 to avoid the unnecessary signaling overhead for both Target node-1 and Target node-2. And since the procedure of prediction transmission is similar with that of one hop target NG-RAN node in Rel-18, UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the multi-hop UE trajectory prediction. Moreover, if Target node-1 or Target node-2 also have the deployment of AI/ML model for UE Trajectory prediction, it could choose to either forward the prediction from the source node to its next hop Target node or generate a new prediction information and send to its next hop Target node to ensure the performance of UE Trajectory prediction.

Observation 1: Option 1 (Hop-by-hop) of multi-hop UE trajectory prediction transmission has the following advantages, e.g.,

- To avoid the unnecessary signaling overhead

- UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the multi-hop UE trajectory prediction, since the procedure of prediction transmission is similar with that of one hop target NG-RAN node in Rel-18

- To enable the update or replacement of the prediction information from the source node and the lasted prediction information could be sent to the next hop target node to ensure the performance of UE Trajectory prediction

But for Option 2 (Point-to-multi-point), on one hand, it may cause the large signaling overhead and wasting of resources since the potential target nodes could get the prediction information from the source node to reserve resources for the coming UE but the UE may not be handed over to the nodes. On the other hand, the prediction forwarding may be limited by Xn interface between the source node and the target nodes. This may cause some target nodes may not get the prediction or make the forwarding procedure complex. Furthermore, the UE Trajectory Prediction cannot be sent in HANDOVER REQUEST message since the prediction will be sent to the nodes before UE is handed over. Therefore, a new procedure or message may be needed.

Observation 2: Option 2 (Point-to-multi-point) of UE trajectory prediction transmission may cause the large signaling overhead and wasting of resources. The prediction forwarding may be limited by Xn interface between the source node and target nodes which will make the procedure complex. And a new procedure or message for UE trajectory prediction transmission may be needed.

Based on the above analysis, we prefer to use Option 1 (Hop-by-hop) as the baseline of the UE trajectory prediction transmission from the source node to the target node for Multi-hop UE trajectory across gNBs.

Proposal 1: The solution Hop-by-hop that the prediction information is transmitted from one node to the next hop node in order could be considered as the baseline for the prediction transmission for Multi-hop UE trajectory across gNBs.

Proposal 2: UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the multi-hop UE trajectory prediction from the source node to the next hop node, and from the intermediate hop node to its next hop node.

If Proposal 1 is agreed, it may have been a long time from the generation of the predicted information of multi-hop UE trajectory at the source node to its acquisition by some target nodes in the UE trajectory. Generally, it is commonly accepted that the prediction accuracy would decrease as the prediction time increases. After a long time of forwarding, the predicted information may become outdated or invalid for the target nodes. Therefore, we propose that the time when the UE Trajectory prediction is generated, or the time of validity could be considered as the additional information to be provided together with the UE Trajectory prediction to avoid the confusion of HO decision or resource reservation caused by outdated/invalid prediction information. Based on the additional information, the target nodes could determine whether to use the predicted information or continue forwarding it to its next-hop node.

Observation 3: The prediction accuracy would decrease as the prediction time increases. After a long time of forwarding, the predicted information may become outdated or invalid for the target nodes.

Observation 4: For Multi-hop UE trajectory scenario, some additional information (e.g., the generation time of the UE trajectory prediction or its time of validity) may be needed for the target nodes to determine whether to use the predicted information or continue forwarding it to its next-hop node.

Proposal 3: The following information could be included in the prediction information from the source node:

- List of cells where the UE is predicted to connect (mandatory in Rel-18 UE Trajectory prediction)

- Predicted Time UE Stays in Cell (optional in Rel-18 UE Trajectory prediction)

- Additional information, e.g., the generation time of the UE trajectory prediction or its time of validity

When the intermediate hop node receives the prediction information generated by the source node, we identify two potential scenarios where the prediction information transmitted by the intermediate hop node may differ from the multi-hop UE trajectory prediction generated by the source node.

- Scenario 1: If the prediction information generated by the source node is correct, that is to say the predicted UE trajectory is the same as the actual UE trajectory, and the validity time of the prediction information has not expired, the intermediate hop node could update the prediction that only includes the cell list the UE is predicted to connect after it leaves this intermediate hop node. Furthermore, based on the actual time the UE stayed in the intermediate hop node, the intermediate hop node could update the remaining validity time of the prediction.

- Scenario 2: If the prediction information generated by the source node is not correct, that is to say the predicted UE trajectory is different from the actual UE trajectory (e.g., the predicted UE trajectory includes Node 1 Node 2Node 3, but the actual UE trajectory is Node 1 Node 2 Node 4), or the validity time of the prediction information has expired, the intermediate hop node (e.g., Node 2) could generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction) to replace the old one from the source node, and transfer the new prediction information to its next hop node. In this case, we think it is beneficial for the source node to receive feedback from the intermediate hop node (e.g., Node 2) about the actual next hop target node after the UE leaves this intermediate hop node.

Observation 5: The prediction information transferred by the intermediate hop node may be different from the multi-hop UE trajectory prediction generated by the source node, since the intermediate hop node could update the prediction based on the actual status or generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction).

Proposal 4: If the prediction information generated by the source node is correct and the validity time of the prediction information has not expired, the following updated information could be included in the multi-hop UE trajectory prediction transferred by the intermediate hop node:

- Updated list of cells where the UE is predicted to connect after it leaves from this intermediate hop node

- Predicted Time UE Stays in Cell which is included in the updated list of cells

- Additional information, e.g., the generation time of the UE trajectory prediction or the updated time of validity of the prediction

Observation 6: If the prediction information generated by the source node is not correct or the validity time of the prediction information has expired, the intermediate hop node could generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction) and transfer the new one to its next hop node. It is beneficial for the source node to receive feedback from the intermediate hop node about the actual next hop target node after the UE leaves this intermediate hop node.

Proposal 5: If the prediction information generated by the source node is not correct or the validity time of the prediction information has expired, the intermediate hop node could:

- generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction) and transfer the new prediction information to its next hop node;

- send the feedback to the source node to the source node about the actual next hop target node after the UE leaves this intermediate hop node.

2.2 Measured information collection and transmission

Similar with one hop target NG-RAN node scenario in Rel-18, we think the feedback (e.g., Measured UE trajectory and Measured UE performance) from the target nodes to the source node is also needed for AI/ML model monitoring and retraining.

Proposal 6: For Multi-hop UE trajectory scenario, the measured information from the target nodes to the source node is also needed for AI/ML model monitoring and retraining. The following information could be considered as the starting point:

- Measured UE trajectory

- Measured UE performance

For the measured information transfer, the following 4 options were captured in the last meeting:

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

• Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Based on the agreement achieved in the last meeting, we think Option4 will not be considered in measured information transfer, and we could focus on evaluating other options.

The delivery of the measured & predicted cell-based multi-hop UE trajectory should not impact the core network.

Observation 7: Based on the agreement achieved in the last meeting, Option4 will not be considered in measured information transfer.

From our perspective, Option3 is not feasible because it is difficult to define the final hop node during UE movement. That is to say, it is hard to determine which node is the final node that needs to feedback the measurement information to the source node. On the other hand, there may be no Xn interface between the source node and the final node.

Observation 8: Option3 is not feasible because it is difficult to define the final hop node during UE movement and there may be no Xn interface between the source node and the final node.

In Option1, the feedback of measured information is limited by Xn interface. If there is no Xn interface between the source node and some target nodes, the measured information related to the target nodes cannot be sent to the source node directly, that may result in the missing of partial measured information or great impact on specs to introduce new feedback procedure.

In Option2 (Hop-by-hop), the intermediate hop nodes (e.g., Target node-1 or Target node-2 in Figure1) needs to collect the measured information in the cells where the UE connect within Target node and receive the measured information that is from its next hop node; and then send the information to its previous hop node. The intermediate hop node needs to keep UE context after handover execution for the duration of the measured information collection from its next hop node. It brings the implementation complexity for the intermediate hop nodes.

Based on the above analysis, we think Option1 and Option2 have their own application scenarios. For example, in the case where there is Xn interface between the source node and target nodes, Option 1 could be used, that means the target nodes could send the measured information to the source node directly to avoid the related intermediate hop nodes keep UE context for a long time after handover execution. Otherwise, in the case where there is no Xn interface between the source node and target nodes, Option 2 could be used. With the help of the intermediate hop nodes, the measured information could be reported to the source node to avoid the measured information missing.

Observation 9: Option1 and Option2 could be used in different scenarios:

- Option 1 could be used in the case where there is Xn interface between the source node and target nodes

- Option 2 could be used in the case where there is no Xn interface between the source node and target nodes

From the above analysis, we observe that the information of Xn interface between the source node and target nodes is useful for target nodes to choose the appropriate feedback path. For example, when there is no Xn interface between a target node and the source node, with the help of the info of Xn interface the target node could find one intermediate hop node in the UE trajectory, which has the Xn interface with the target node and with the source node, respectively.

Observation 10: The information of Xn interface between source node and target nodes is helpful for target nodes to choose the appropriate feedback path to the source node for the measured information.

Proposal 7: RAN3 considers to introduce the information of Xn interface between the source node and target nodes in the UE Trajectory Prediction for Multi-hop UE trajectory scenario.

Configuration of measurement feedback.

In Rel-18, Collection Time Duration is used as one of the UE Trajectory Collection Configuration which is the condition to trigger the target node to stop the UE Trajectory collection after successful handover. For multi-hop UE trajectory, the actual collection duration of each target node will be different due to the time UE stayed in previous target node(s).

Observation 11: For multi-hop UE trajectory, the actual collection duration of each target node will be different due to the time UE stayed in previous target node(s).

Therefore, the intermediate hop node(s) could update the Collection Time Duration for its next hop node when the UE leaves this intermediate hop node and transfers the Collection Time Duration in UE associated message (i.e., HANDOVER REQUEST message).

Proposal 8: The Collection Time Duration could be updated by each intermediate hop node when the UE leaves this intermediate hop node.

Proposal 9: UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the Collection Time Duration, at least from the intermediate hop node to its next hop node.

# 3 Conclusion

In this paper, we discuss on the solutions for AI/ML based multi-hop UE trajectory across gNBs, we have the following observations and proposals:

For Prediction information transmission

Observation 1: Option 1 (Hop-by-hop) of multi-hop UE trajectory prediction transmission has the following advantages, e.g.,

- To avoid the unnecessary signaling overhead

- UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the multi-hop UE trajectory prediction, since the procedure of prediction transmission is similar with that of one hop target NG-RAN node in Rel-18

- To enable the update or replacement of the prediction information from the source node and the lasted prediction information could be sent to the next hop target node to ensure the performance of UE Trajectory prediction

Observation 2: Option 2 (Point-to-multi-point) of UE trajectory prediction transmission may cause the large signaling overhead and wasting of resources. The prediction forwarding may be limited by Xn interface between the source node and target nodes which will make the procedure complex. And a new procedure or message for UE trajectory prediction transmission may be needed.

Proposal 1: The solution Hop-by-hop that the prediction information is transmitted from one node to the next hop node in order could be considered as the baseline for the prediction transmission for Multi-hop UE trajectory across gNBs.

Proposal 2: UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the multi-hop UE trajectory prediction from the source node to the next hop node, and from the intermediate hop node to its next hop node.

Observation 3: The prediction accuracy would decrease as the prediction time increases. After a long time of forwarding, the predicted information may become outdated or invalid for the target nodes.

Observation 4: For Multi-hop UE trajectory scenario, some additional information (e.g., the generation time of the UE trajectory prediction or its time of validity) may be needed for the target nodes to determine whether to use the predicted information or continue forwarding it to its next-hop node.

Proposal 3: The following information could be included in the prediction information from the source node:

- List of cells where the UE is predicted to connect (mandatory in Rel-18 UE Trajectory prediction)

- Predicted Time UE Stays in Cell (optional in Rel-18 UE Trajectory prediction)

- Additional information, e.g., the generation time of the UE trajectory prediction or its time of validity

Observation 5: The prediction information transferred by the intermediate hop node may be different from the multi-hop UE trajectory prediction generated by the source node, since the intermediate hop node could update the prediction based on the actual status or generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction).

Proposal 4: If the prediction information generated by the source node is correct and the validity time of the prediction information has not expired, the following updated information could be included in the multi-hop UE trajectory prediction transferred by the intermediate hop node:

- Updated list of cells where the UE is predicted to connect after it leaves from this intermediate hop node

- Predicted Time UE Stays in Cell which is included in the updated list of cells

- Additional information, e.g., the generation time of the UE trajectory prediction or the updated time of validity of the prediction

Observation 6: If the prediction information generated by the source node is not correct or the validity time of the prediction information has expired, the intermediate hop node could generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction) and transfer the new one to its next hop node. It is beneficial for the source node to receive feedback from the intermediate hop node about the actual next hop target node after the UE leaves this intermediate hop node.

Proposal 5: If the prediction information generated by the source node is not correct or the validity time of the prediction information has expired, the intermediate hop node could:

- generate new prediction information (if it is deployed with the AI/ML model for multi-hop UE trajectory prediction) and transfer the new prediction information to its next hop node;

- send the feedback to the source node to the source node about the actual next hop target node after the UE leaves this intermediate hop node.

For Measured information collection and transmission

Proposal 6: For Multi-hop UE trajectory scenario, the measured information from the target nodes to the source node is also needed for AI/ML model monitoring and retraining. The following information could be considered as the starting point:

- Measured UE trajectory

- Measured UE performance

Observation 7: Based on the agreement achieved in the last meeting, Option4 will not be considered in measured information transfer.

Observation 8: Option3 is not feasible because it is difficult to define the final hop node during UE movement and there may be no Xn interface between the source node and the final node.

Observation 9: Option1 and Option2 could be used in different scenarios:

- Option 1 could be used in the case where there is Xn interface between the source node and target nodes

- Option 2 could be used in the case where there is no Xn interface between the source node and target nodes

Observation 10: The information of Xn interface between source node and target nodes is helpful for target nodes to choose the appropriate feedback path to the source node for the measured information.

Proposal 7: RAN3 considers to introduce the information of Xn interface between the source node and target nodes in the UE Trajectory Prediction for Multi-hop UE trajectory scenario.

Observation 11: For multi-hop UE trajectory, the actual collection duration of each target node will be different due to the time UE stayed in previous target node(s).

Proposal 8: The Collection Time Duration could be updated by each intermediate hop node when the UE leaves this intermediate hop node.

Proposal 9: UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the Collection Time Duration, at least from the intermediate hop node to its next hop node.

# Reference

- RAN3_129bis_agenda_20251017_EOM, Oct 2025.

Annex: pCR for TR 38.745

4.1	Multiple-hop UE trajectory

4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.1.2.X	Output data of AI/ML based Multi-hop UE trajectory

The prediction information of multi-hop UE trajectory across gNBs is necessary for AI/ML based multi-hop UE trajectory, which could be used to assist the target nodes for HO decision and resource reservation.

The following information can be considered as output data from the source node:

- List of cells where the UE is predicted to connect

- Predicted Time UE Stays in Cell

- Additional information, e.g., the generation time of the UE trajectory prediction or its time of validity

For the prediction transmission for Multi-hop UE trajectory across gNBs, the solution Hop-by-hop that the prediction information is transmitted from one node to the next hop node in order is taken as the baseline.

UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the multi-hop UE trajectory prediction.

4.1.2.Y	Measurement feedback of AI/ML based Multi-hop UE trajectory

For Multi-hop UE trajectory scenario, the measured information from the target nodes to the source node is needed for AI/ML model monitoring and retraining.

The measured information at the target node may include:

- Measured UE trajectory

- Measured UE performance

The information of Xn interface between source node and target nodes may be needed for target nodes to choose the appropriate feedback path to the source node for the measured information.

UE associated message (i.e., HANDOVER REQUEST message) could be used to transfer the Collection Time Duration, at least from the intermediate hop node to its next hop node.