3GPP TSG-RAN WG3 Meeting #130	R3-258609

Dallas, USA, 17th – 21th Oct 2025

Title: 	Discussion on AI/ML based intra-CU LTM use cases

Source: 	CMCC

Agenda item: 	12.2.2

Document Type:	Discussion and Decision

# Introduction

A new R20 study item has been agreed in RAN#109:  RP-252867, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

And in RAN3#129-bis meeting, the following discussion conclusions have been captured in chair note:

Deployment of AI/ML model for intra-CU LTM:

Prioritize normal intra-CU LTM scenarios.

Cover both L1 and L3 Measurement Report based Intra-CU LTM?

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation

RAN3 to study TA value and validity predication for Intra-CU LTM in spatial and/or temporal domain

Beam prediction should also be considered?

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

This discussion paper will focus on the AI/ML based intra-CU LTM use cases.

# Discussion

This discussion paper will focus on the output, input and feedback information for AI/ML based intra-CU LTM use cases.

## 2.1 Predicated output

In LTM HO preparation phase, the LTM configurations are exchanged between gNBs. The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-CU LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. The candidate prepares and provides the LTM configuration(s) to the source gNB. During this procedure, the candidate cells prediction/determination can be optimized using AI/ML.

And in last meeting, it has been agreed that:

- AI/ML assisted candidate cell selection for LTM Handover Preparation

It is straightforward that predicated candidate cells can be transferred from gNB-CU to gNB-DU to assist candidate cell selection for LTM Handover Preparation.

Proposal 1: It is beneficial to predicate candidate cells in gNB-CU and transfer this information to gNB-DU, to assist candidate cell selection for LTM Handover Preparation.

To improve the accuracy of predicated candidate cells, the predicated UE trajectory can be input as assistance information. However currently predicated UE trajectory introduced in R18 is only cell level, which can’t provide enough high granularity information, we suggest to introduce beam level UE trajectory predication in this use case.

Meanwhile, beam level UE trajectory is also useful in predicating TA value for each candidate cell which is a high precision value. Using only cell level trajectory is not enough to predicate TA.

Proposal 2: It is suggested to introduce predicated Cell/Beam level UE trajectory for AI/ML based LTM, to improve the accuracy of predicated candidate cells.

Except to improve the efficiency of LTM preparation through predicating candidate cells, AI/ML can also be used to optimize the procedures of early UL synchronization.

TS 38.401 describes how the early UL synchronization is triggered in gNB-DU for candidate cells before cell switch over.

When configured by the network, it is possible to initiate UL TA acquisition (called early TA) procedure of one or multiple cells that are different from the current serving cells. If the cell has the same NTA as the current serving cells or NTA=0, early TA acquisition procedure is not required. The network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition procedure is triggered by PDCCH order as specified in clause 9.2.6 or realized through UE-based TA measurement as configured by RRC. In the former case, the gNB/gNB-DU to which the candidate cell belongs calculates the TA value and sends it to the gNB/gNB-DU to which the serving cell belongs via gNB-CU. The serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch. In the latter case, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. The UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command, if it does not include any valid TA value. The network may also send a TA value in the LTM cell switch command MAC CE without early TA acquisition.

In current procedure, for the candidate cell, if NTA=0 or NTA is same as the serving cell, acquisition procedure is not required. Otherwise, the early TA acquisition procedure will be triggered by gNB-DU to UE. And all the cells need to calculate the TA respectively.

Another way to get TA is to predicate it on gNB-CU through AI/ML, based on the historical TA measurements gNB-CU collected, and some other information such as the UE trajectory, UE location information. And gNB-CU can transfer the predication TA measurement to gNB-DU for each candidate cells. In this way, early TA procedure can be skipped, which can further improve the efficiency of cell switch over.

And since the TA value is a predicated value, it shall be with a validity time. The early TA acquisition procedure may still be triggered after the validity time.

However, according to the previous discussion, we understand that the TA value is a key parameter and very sensitive for cell switch over. For RACH-less LTM, an invalid TA value may cause the LTM switch over failure and can’t recover immediately. Relying on the predicated value to execute the cell switch over may be risky now. So we suggest to use this value as an assistant information, and up to the gNB-DU’s implementation whether to use it or not.

Proposal 3: For intra-DU LTM, it is beneficial to introduce predicated TA value for each candidate cells as the output of AI/ML model. However, whether using the predicated TA value for cell switch over is up to gNB-DU implementation.

Meanwhile, the predicated validity time of measured TA value is also important. This value can help gNB-DU to select the target cells from candidate cells. Based on the historical measured TA value and UE trajectory information, AI/ML model can also predicate the validity time for a given measured TA value.

Proposal 4: It is proposed to transfer predicated validity time of measured TA value from gNB-CU to gNB-DU, to help select the target cell.

Currently in early UL synchronization stage, gNB-DU decide whether and when to trigger UL TA acquisition procedures based on L1/2 UE measurement configuration and reporting. Too early or too later to trigger the procedures will cause the inaccurate TA measurement and lead to cell switch over failure. We do believe AI can assist in predicating the timing based on more L3 measurement and history data. To avoid invalid early UL synchronization procedures, it is helpful to choose the best timing to activate early UL synchronization.

Proposal 5: It is suggested to use AI to predicate the timing whether and when to trigger early UL synchronization, to improve the efficiency of TA acquisition and assist cell switch over.

## 2.2 Input

To predicate the TA value for each candidate cells, gNB-CU can use the following information from gNB-DU as input for AI/ML inference model:

- Historical TA value measured by UE for candidate cells

- L3 Measurement

We do believe L1/L2 measurement is helpful in selecting candidate cells and target cell, but since the L1/L2 measurements have already been used in gNB-DU for the decision, we think L3 measurements shall be prioritized first as input for AI/ML.

gNB-CU can also use the local input to predicate TA value as below:

- Predicated Cell/Beam level UE trajectory

- UE location predication, this location includes the geo, coordination information, not the same granularity as UE trajectory to cells, gNB can use the location to predicate TA value from UE to each candidate cells

Proposal 6: It is proposed to prioritize the usage of L3 measurement as input for AI/ML model, than L1/L2 measurements.

Proposal 7: It is proposed to introduce beam level UE trajectory predication as a local input to predicate the candidate cells, and time to trigger cell switch over.

## 2.3 Feedback

In order to provide information to evaluate and improved the predication as above, the measured value for these predications shall be collected as feedback.

And after cell switch over, it is better to provide the feedback information back to AI/ML model for verification and evaluation:

- Measured Cell/Beam level UE trajectory

- Measured early UL synchronization trigger time (if has)

- TA value measured by UE for target cell and other candidate cells

- UE performance

Proposal 8: In intra-DU LTM, it is suggested to introduce the following feedback information for AI/ML model:

- Measured Cell/Beam level UE trajectory

- Measured early UL synchronization trigger time (if has)

- TA value measured by UE for target cell and other candidate cells

- UE performance

# 3	Summary

In this contribution, we provide additional input considerations on AI/ML intra-CU LTM use cases, following proposals are made:

Proposal 1: It is beneficial to predicate candidate cells in gNB-CU and transfer this information to gNB-DU, to assist candidate cell selection for LTM Handover Preparation.

Proposal 2: It is suggested to introduce predicated Cell/Beam level UE trajectory for AI/ML based LTM, to improve the accuracy of predicated candidate cells.

Proposal 3: For intra-DU LTM, it is beneficial to introduce predicated TA value for each candidate cells as the output of AI/ML model. However, whether using the predicated TA value for cell switch over is up to gNB-DU implementation.

Proposal 4: It is proposed to transfer predicated validity time of measured TA value from gNB-CU to gNB-DU, to help select the target cell.

Proposal 5: It is suggested to use AI to predicate the timing whether and when to trigger early UL synchronization, to improve the efficiency of TA acquisition and assist cell switch over.

Proposal 6: It is proposed to prioritize the usage of L3 measurement as input for AI/ML model, than L1/L2 measurements.

Proposal 7: It is proposed to introduce beam level UE trajectory predication as a local input to predicate the candidate cells, and time to trigger cell switch over.

Proposal 8: In intra-DU LTM, it is suggested to introduce the following feedback information for AI/ML model:

- Measured Cell/Beam level UE trajectory

- Measured early UL synchronization trigger time (if has)

- TA value measured by UE for target cell and other candidate cells

- UE performance

# 4	Reference

[1] RP-252867, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

[2] TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

[3] TS 38.401, NG-RAN Architecture description, Stage 2

[4] RAN3_129bis_agenda_20251017_EOM, Chairnote