3GPP TSG-RAN WG3 Meeting # 130	R3-258634

Dallas, US, 17 – 21 November, 2025

# Title: 	Discussion on R20 A-IoT for Topology 2

# Source: 	ZTE Corporation

Agenda item:	14.2

# Document for:	Discussion and Decision

- Introduction

The following progress has been made in previous RAN3 meeting:

- For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied.

- NGAP procedures specified for Topology 1 are base line for Topology 2.

- The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as base line.

- UE Reader Authorization is performed at 5GC

- The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

- FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

- In case the core network provides an empty requested service area info, the gNB selects among all the served connected mode UE readers.

This contribution intends to further discuss the remaining issues we left in previous meeting.

- Discussion

  - Signaling for Topology 2

RAN3 has confirmed to consider NGAP procedures in  topology 1 as the baseline for topology 2. More specifically, In R19, we have introduced the following AIOT procedures for Topology 1.

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (node initiated).

In R20, these AIOT procedures are suggested for Topology 2.

Proposal 1: The following AIOT procedures are introduced for Topology 2.

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (node initiated).

2.2 UE reader selection

In addition, RAN3 may also need to discuss the UE Reader selection

Moreover, UE Reader selection also impacts the related NGAP messages. At least the following alternatives are on the table:

- CN based solution

In this alternative, CN is fully responsible for the UE Reader selection. CN shall provide all needed information to gNB in the AIoT related NGAP message (e.g., Inventory Request message). Based on the received information, gNB shall select dedicated UE for received UE info list or select all involved UE Readers for received AIoT service area info, cell/TAI list for the upcoming procedure.

- CN assisted solution

CN can provide the UE Reader selection related information(e.g. UE list, AIoT service area info, Cell ID list, or TAI list) to gNB. But in this alt, gNB may consider the received info as reference and make its own decision on which UE Reader(s) will be involved for next progress.

- RAN based solution

gNB is fully responsible for the UE Reader selection. This alternative has the minimum NGAP impact in all three alternatives

Compared with alt#1, the other 2 alternatives are more flexible and can handle more situations in the deployment. Hence, we encourage RAN3 to down prioritize the alt#1.FFS the CN assisted solution or RAN based solution.

Proposal 2: It is proposed for RAN3 to discuss how to perform the UE Reader selection and make the down priority:

  - CN based solution: CN fully handles UE Reader selection.

  - CN assisted solution: CN provides assistance information, RAN makes final decision.

  - RAN based solution: RAN fully handles UE Reader selection.

In previous meeting, companies discussed how gNB handles the received empty requested service area information and make no consensus. In our view, this is a corner case and will rarely happens. So from simplicity point of view, when CN provides en empty requested service area info to gNB, gNB may select the the UE reader(s) from all the served connected mode UE readers by implementation, which is the alt#2 or alt#3 described in above proposal.

Proposal 3: gNB may selects UE readers among all its served connected mode UE readers when core network provides an empty requested service area info.

2.3 Specification of UE reader authorization

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

In current NGAP/XnAP/F1AP specifications, in order to support V2X services via the PC5 interface, the authorization status of the UE to use the NR sidelink for V2X services has been specified as “V2X Services Authorized”.

NR V2X Services Authorized This IE provides information on the authorization status of the UE to use the NR sidelink for V2X services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Vehicle UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Vehicle UE. Pedestrian UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Pedestrian UE. LTE V2X Services Authorized This IE provides information on the authorization status of the UE to use the LTE sidelink for V2X services. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Vehicle UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Vehicle UE. Pedestrian UE; O; ENUMERATED (authorized, not authorized, ...); Indicates whether the UE is authorized as Pedestrian UE.

In NGAP specification, the authorization status of the UE is sent from AMF to NG-RAN node, via the following messages.

-	INITIAL CONTEXT SETUP REQUEST

-	UE CONTEXT MODIFICATION REQUEST

-	HANDOVER REQUEST

-	PATH SWITCH REQUEST ACKNOWLEDGE

In XnAP specification, the authorization status of the UE is sent from source/old NG-RAN node to the target/new NG-RAN node, via the following messages.

-	HANDOVER REQUEST

-	RETRIEVE UE CONTEXT RESPONSE

In F1AP specification, the authorization status of the UE is sent from node-CU to node-DU, via the following messages.

-	UE CONTEXT SETUP REQUEST

-	UE CONTEXT MODIFICATION REQUEST

Proposal 4: The UE V2X service authorization mechanism can be as reference to specify the UE reader authorization.

2.4 Inter-gNB RRC-connected UE reader mobility

For large area storage places(e.g. port) or portable storage(marine/railway transport), the whole AIoT deployed area may be covered by multiple gNBs. When UE reader performs the inventory and/or command procedure(assume Topology 2 has such procedures) for the AIoT devices placed at the boundary between two gNBs, the UE reader may performs the inter-gNB mobility. Hence, we see benefit to support the inter-gNB RRC_CONNECTED UE reader mobility. More specifically, when UE reader handovers to target gNB, the UE reader may need to keep uploading the inventory reports which has been kept locally if needed. The general procedure is shown below:

Proposal 5: UE reader may need to report the kept inventory report information to target node after inter-gNB RRC-connected mobility.

- Conclusion

Based on the above analysis, we provide the following procedures.

Proposal 1: The following AIOT procedures are introduced for Topology 2.

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Session Release (AIoT CN Initiated);

-	A-IoT Session Release Request (node initiated).

Proposal 2: It is proposed for RAN3 to discuss how to perform the UE Reader selection and make the down priority:

  - CN based solution: CN fully handles UE Reader selection.

  - CN assisted solution: CN provides assistance information, RAN makes final decision.

  - RAN based solution: RAN fully handles UE Reader selection.

Proposal 3: gNB may selects UE readers among all its served connected mode UE readers when core network provides an empty requested service area info.

Proposal 4: The UE V2X service authorization mechanism can be as reference to specify the UE reader authorization.

Proposal 5: UE reader may need to report the kept inventory report information to target node after inter-gNB RRC-connected mobility.