3GPP TSG-RAN WG3 #130                                                    R3-258648

Dallas, US, 17th – 21th November, 2025

Agenda Item:	11.2.1

Source:	CATT

Title:	Discussion on SON enhancement for inter-CU LTM

Document for:	Discussion

Introduction

In last RAN3 meeting, the following agreements and FFS have been achieved:

Use cases:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.:

BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA;

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

Solution options:

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as baseline.

For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as baseline.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

L3 based inter-CU LTM is within the scope of the WID

Here we provide some analysis on MRO enhancement for inter-CU LTM.

Discussion

In last RAN3 meeting, following use cases have been agreed:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.:

BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA;

LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell;

And the solution for inter-CU LTM should take Rel-19 as baseline:

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

In R19, ACCESS AND MOBILITY INDICATION message is enhanced to provide BFR, failure SSB and TA information from target DU to source DU via CU. While for inter-CU LTM, current ACCESS AND MOBILITY INDICATION message in XN interface should be enhanced to align with F1 interface.

Proposal 1: It is proposed to enhance ACCESS AND MOBILITY INDICATION message in XN interface to align with F1 interface for MRO for inter-CU LTM.

For the case of LTM connection failure (too late and too early LTM cell switch, LTM cell switch to wrong cell), in R19, the solution for intra-CU LTM is based on RLF Report. For the case of inter-CU LTM, Transferring RLF Report between NG-RAN has been supported by current specification. So, there is no stage3 impact for the case of LTM connection failure.

Proposal 2: For the case of LTM connection failure (too late and too early LTM cell switch, LTM cell switch to wrong cell), there is no stage3 impact to support MRO for inter-CU LTM.

As for stage2 impact, subsequent LTM is supported in case of inter-CU LTM. So, MRO for subsequent LTM scenarios should be considered.

For example, gNB1 triggers LTM preparation, candidate cells in gNB2 and gNB3 is configured. Later, UE move from gNB1-> gNB2-> gNB3. If LTM failure occurs from gNB2 to gNB3, the failure may be cause by gNB1 (wrong candidate cell selection) or gNB2 (wrong handover triggering point). Not only last serving gNB2 but also gNB1 as initiating node is involved in MRO.

Proposal 3: It is proposed for RAN3 to discuss the subsequent inter-CU LTM scenario in which not only last serving gNB but also the initiating gNB should be involved to perform optimization.

In last RAN3 meeting, MRO for Ping-Pong case is captured:

Successful case: ping/pong

In R19, CU is responsible for detecting LTM Ping-Pong occurs and then inform DU to solve the issue. In release 20, for inter-CU LTM, UHI in e.g., handover request message should be enhanced to include LTM information in order for target CU to detect LTM Ping-Pong. Handover type (L3 handover, LTM) should be included in UHI.

Proposal 4: It is proposed to include handover type (L3 handover, LTM) in UHI.

In R19 mobility, L3 triggered LTM is supported. Based on L1 or L3 measurement report, network triggers LTM. The information is necessary for MRO to decide whether to optimize L3 or L1 configuration. It can be recorded by network as a kind of LTM configuration.

Proposal 5: It is proposed to use network based solution to identify L1 or L3 triggered LTM.

Conclusions

Proposal 1: It is proposed to enhance ACCESS AND MOBILITY INDICATION message in XN interface to align with F1 interface for MRO for inter-CU LTM.

Proposal 2: For the case of LTM connection failure (too late and too early LTM cell switch, LTM cell switch to wrong cell), there is no stage3 impact to support MRO for inter-CU LTM.

Proposal 3: It is proposed for RAN3 to discuss the subsequent inter-CU LTM scenario in which not only last serving gNB but also the initiating gNB should be involved to perform optimization.

Proposal 4: It is proposed to include handover type (L3 handover, LTM) in UHI.

Proposal 5: It is proposed to use network based solution to identify L1 or L3 triggered LTM.

Reference

[1]