3GPP TSG-RAN WG3 #130                                                 R3-258649

Dallas, US, 17th – 21th November, 2025

Agenda Item:	11.2.2

Source:	CATT

Title:	(TP for 38.300) Discussion on SON enhancement for conditional intra-CU LTM

Document for:	others

Introduction

In last RAN3 meeting, the following agreements and FFS have been achieved:

Use cases:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.:

BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA;

FFS whether both L1 and L3 apply to beam related selection optimization.

LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

L3 based intra-CU conditional LTM is within the scope of the WID

Solution options:

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable.

For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as baseline.

For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as baseline.

For the connection failure cases, the last serving gNB-CU performs initial analysis.

Additional information required from UE

Here we provide some analysis on MRO enhancement for conditional intra-CU LTM.

Discussion

In last RAN3 meeting, on beam selection, following FFS is captured:

FFS whether both L1 and L3 apply to beam related selection optimization.

In case of L1 triggered C-LTM, L1-execution condition allocated by source DU is used to perform beam selection. In case of L3 triggered C-LTM, L3 report is used to select candidate cell and CU will configure a RSRP during LTM preparation is also used to beam selection. The wrong beam selection will impact the C-LTM. So, both L1 and L3 C-LTM should consider beam selection optimization.

Proposal 1: Both L1 C-LTM and L3 C-LTM apply to beam selection optimization.

R19 LTM use cases have been discussed and following agreements are captured:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.:

BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA;

In R19, network solution is introduced to support MRO on the cases: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA. For legacy LTM, it is always source DU to perform optimization for above cases.

But for C-LTM, it is listed as below:

CU is responsible for the following configuration:

- Candidate cell selection.

- Execution condition in case of L3 triggered LTM.

- Early TA acquisition based on L3 measurement report.

Target DU is responsible for the following configuration:

- Execution condition in case of L1 triggered LTM.

- Candidate beam selection.

Source DU is responsible for the following configuration:

- Execution condition in case of L1 triggered LTM for initiating C-LTM.

- Triggering early TA acquisition based on L1 measurement report.

Observation 1: For C-LTM, source DU, target DU and CU are responsible for above configurations.

In R19, target DU perform initial analysis based on CU-DU/DU-CU CELL SWITCH NOTIFICATION message. While for C-LTM, without CU-DU/DU-CU CELL SWITCH NOTIFICATION message, target DU is not aware:

- The time UE triggers the initiation of C-LTM.

- The target cell and beam toward which UE triggers C-LTM.

- After receiving RRCReconfigurationComplete message, target DU cannot detect successful handover access or recovery access.

- TA information.

Observation 2: In case of C-LTM, without CU-DU/DU-CU CELL SWITCH NOTIFICATION message target DU is not aware of the following information:

- The time UE triggers the initiation of C-LTM.

- The target cell and beam toward which UE triggers C-LTM.

- After receiving RRCReconfigurationComplete message, target DU cannot detect successful handover access or recovery access.

- TA information.

Without above information, target DU cannot perform initial analysis as for legacy LTM. R19 network solution should be enhanced for C-LTM.

In our understanding, CU should be responsible for collecting failure information from successful accessed DUs, perform initial analysis and then inform related DU to make optimization.

CU would receive ACCESS SUCCESS and ACCESS AND MOBILITY INDICATION message from DU for the following case:

- UE perform successful LTM to cell in DU1. From network side, it can be recovery after HOF or direct successful LTM case. Neither CU nor source DU nor target DU can differentiate these two cases. But the successful access cell, beam and TA still can be used as the suitable selection to make optimization.

- UE selects cell in DU1 as handover target cell, RLF occurs shortly after successful handover. Then successful recovery to DU2 or DU1. Network is not aware of RLF, but after receiving ACCESS SUCCESS and ACCESS AND MOBILITY INDICATION message from DU2 or DU1 for successful access, CU may deduce RLF occurs or quick switch (due to “shortly”) and the final successful access cell, beam and TA can be used as the suitable selection to make optimization..

- UE selects cell in DU1 as handover target cell, RLF occurs shortly after successful handover. Then RRC re-establish to DU1 or DU2. The re-establish procedure implicitly indicates failure happens. If the re-establish cell is not within candidate cell, CU itself performs MRO on candidate cell selection. Otherwise, CU may inform related DU to optimize execution condition.

Observation 3: The obstacle for network solution includes:

- Network cannot differentiate successful LTM from recovery procedure.

- Network cannot detect RLF occurs directly.

Proposal 2: CU collects failure information from DU, perform initial analysis and then inform related DU to perform optimization.

Based on above analysis, the network solution for R19 use cases are captured below:

- BFR：Successful LTM from Source cell 1 beam 1 to target cell 2 beam 2, then BFR occurs. Recovery to target cell 2 beam 3 successfully

Target cell 2 provides ACCESS SUCCESS and ACCESS AND MOBILITY INDICATION message to CU for successful LTM including target cell/beam (target cell 2 beam 2) and BFR information (target cell 2 beam 3). CU identifies the source cell. In case of L3 C-LTM, CU performs RSRP of beam selection optimization. In case of L1 C-LTM, CU provides BFR information (target cell 2 beam 3) to source cell1. Source cell 1 optimizes L1 execution condition.

Proposal 3: For BRF, target DU provides target cell/beam and subsequent BFR information (recovery cell/beam) to CU, in case of L1 C-LTM, CU further provide them to source DU.

- Failure due to wrong beam: LTM fails from Source cell 1 beam 1 to target cell 2 beam 2. Recovery/re-establish to target cell 2 beam 3 successfully

- Failure due to wrong TA: LTM fails from Source cell 1 beam 1 to target cell 2 beam 2. Recovery/re-establish to target cell 2 beam 2 or other beam successfully

Target cell 2 provides beam and TA information to CU. CU also can acquire TA value from previous DU-CU/CU-DU TA INFORMATION TRANSFER message. CU detects wrong TA issue if the TA value in DU-CU/CU-DU TA INFORMATION TRANSFER message is not equal to the TA provided by DU for the same candidate cell. Otherwise, it would be wrong beam issue.

Proposal 4: Target DU provides recovery/re-establish cell/beam and TA to CU, CU can be responsible for detecting wrong TA issue or wrong beam issue if the TA value in DU-CU/CU-DU TA INFORMATION TRANSFER message is not equal to the TA provided by DU for the same candidate cell.

For wrong TA issue, in case of L3 C-LTM, CU performs TA optimization. In case of L1 C-LTM, CU identifies the latest trigger early TA acquisition cell and further forward TA of recovery or re-establish cell to latest trigger early TA acquisition cell.

For wrong beam issue, In case of L3 C-LTM, CU performs RSRP of beam selection optimization. In case of L1 C-LTM, CU provides target cell 2 and beam information to source cell1. Source cell 1 optimizes L1 execution condition for (cell 1 beam 1 -> cell 2 beam 3). Due to that the failed target cell 2 beam 2 is only known by UE in case of HOF, reporting failed target cell 2 beam 2 in RLF report can be considered in case of the L1 execution condition of (cell 1 beam 1 - cell 2 beam 2) would like to optimized.

Proposal 5: After CU detects failure type (wrong TA issue or wrong beam issue), in case of L1 C-LTM, CU provides recovery/re-establish cell/beam and TA to DU. Whether UE reports failed target cell/beam can be further discussed.

The RACH-less may be failed due to too long TAT configuration. UE perform RACH-less within TAT but failed. Whether the addition information required by UE can be further discussed.

Proposal 6: Support MRO on RACH-less failure due to too long TAT.

Another solution is the UE based solution, i.e. RLF report should be enhanced to include TA, beam and BFR information which have RAN2 impact.

Comparing network and UE based solution, network solution bases on R19 method which has less specification impact while UE based solution may impact RAN2.

Proposal 7: It is proposed to reuse R19 network solution as much as possible.

For the following case captured in last meeting:

Near failure case: SHR successful HO

SHR has been included in current ACCESS AND MOBILITY INDICATION message in F1 interface. So, there is no stage3 impact for RAN3.

Proposal 8: To support near failure case for conditional intra-CU LTM, there is no stage 3 impact.

In release 19, Ping-Pong issue in case of intra-CU LTM has been discussed. The solution is CU detects LTM Ping-Pong occurs and then inform DU to solve the issue. In release 20, Ping-Pong issue should be considered in conditional intra-CU scenarios. For conditional LTM, CU is also aware of the UE stay cell and stay time, so, the solution for R19 intra-CU LTM could be reused.

Proposal 9: It is proposed to reuse the solution on Ping-Pong issue of R19 intra-CU LTM for that of R20 conditional intra-CU LTM.

For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line.

For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

As agreed in last meeting, the definition of connection failure and detection mechanism in 38.300 can be taken as base line. The current specification does not cover the condition LTM case. We propose a TP as Annex.

- In case of conditional LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late conditional LTM cell switch, Too Early conditional LTM cell switch and conditional LTM cell switch to Wrong Cell.

- -    Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/ (conditional) LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

- The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/(conditional) LTM execution until connection failure.

Proposal 10: Agree TP for 38.300 to cover conditional LTM case in definition of connection failure and detection mechanism.

Conclusions

Proposal 1: Both L1 C-LTM and L3 C-LTM apply to beam selection optimization.

Observation 1: For C-LTM, source DU, target DU and CU are responsible for above configurations.

Observation 2: In case of C-LTM, without CU-DU/DU-CU CELL SWITCH NOTIFICATION message target DU is not aware of the following information:

- The time UE triggers the initiation of C-LTM.

- The target cell and beam toward which UE triggers C-LTM.

- After receiving RRCReconfigurationComplete message, target DU cannot detect successful handover access or recovery access.

- TA information.

Observation 3: The obstacle for network solution includes:

- Network cannot differentiate successful LTM from recovery procedure.

- Network cannot detect RLF occurs directly.

Proposal 2: CU collects failure information from DU, perform initial analysis and then inform related DU to perform optimization.

Proposal 3: For BRF, target DU provides target cell/beam and subsequent BFR information (recovery cell/beam) to CU, in case of L1 C-LTM, CU further provide them to source DU.

Proposal 4: Target DU provides recovery/re-establish cell/beam and TA to CU, CU can be responsible for detecting wrong TA issue or wrong beam issue if the TA value in DU-CU/CU-DU TA INFORMATION TRANSFER message is not equal to the TA provided by DU for the same candidate cell.

Proposal 5: After CU detects failure type (wrong TA issue or wrong beam issue), in case of L1 C-LTM, CU provides recovery/re-establish cell/beam and TA to DU. Whether UE reports failed target cell/beam can be further discussed.

Proposal 6: Support MRO on RACH-less failure due to too long TAT.

Proposal 7: It is proposed to reuse R19 network solution as much as possible.

Proposal 8: To support near failure case for conditional intra-CU LTM, there is no stage 3 impact.

Proposal 9: It is proposed to reuse the solution on Ping-Pong issue of R19 intra-CU LTM for that of R20 conditional intra-CU LTM.

Proposal 10: Agree TP for 38.300 to cover conditional LTM case in definition of connection failure and detection mechanism.

Reference

[1]

Annex

- 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

In case of conditional LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late conditional LTM cell switch, Too Early conditional LTM cell switch and conditional LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/(conditional) LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/(conditional) LTM execution until connection failure.