3GPP TSG-RAN WG3 Meeting #130                              R3-258680

Dallas, US, 17-21 November , 2025

Agenda Item:	11.2.2

Source:	ZTE Corporation

Title:	MRO for Conditional LTM

Document for:	Discussion and Approval

Introduction

In the Rel-20 WID in RP-251869, the following objective was identified.

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

During RAN3#129bis meeting, following agreements are reached:

1. Use cases At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA; FFS whether both L1 and L3 apply to beam related selection optimization. LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell; Near failure case: SHR successful HO Successful case: ping/pong L3 based intra-CU conditional LTM is witin the scope of the WID 2. Solution options Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable. For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line. For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line. For the connection failure cases, the last serving gNB-CU performs initial analysis. Additional information required from UE To be continued...

This contribution provide our views focus on MRO for conditional LTM.

Discussion

Overview on intra-CU Conditional LTM

The Conditional L1/L2 Triggered Mobility (C-LTM), defined as an LTM cell switch procedure executed only when defined execution condition(s) are met, is a Rel-19 mobility mechanism. It aims to combine the high robustness achieved by conditional mobility procedures (like CHO) with the short interruption time offered by L1/L2 triggered mobility (LTM).

Unlike normal LTM, C-LTM relies on UE evaluation of specific conditions before performing a cell switch. These conditions are configurable based on L1 conditions determined by gNB-DU or L3 conditions determined by gNB-CU. In other word, the Network needs to decide not only candidate cells but also associated Execution Condition(s) for each candidate cell. Therefore, the most import task for MRO is to help Network know whether the selected C-LTM candidate cell(s) and associated Execution Condition(s) are suitable.

Proposal 1: MRO for CLTM not only needs to consider whether the selected C-LTM candidate cell(s) are suitable, but also needs to decide whether the configured Execution Condition(s) are suitable.

R20 MRO User case

According to agreements reached in last RAN3#129bis meeting, at least following use cases should be supported:

- BFR shortly after successful C-LTM cell switch execution

- LTM Cell Switch failure due to wrong beam and outdated TA;

- LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

BFR shortly after successful C-LTM cell switch execution

In legacy MRO for LTM, the target gNB-DU identifies that a Beam Failure Recovery (BFR) happened in UE side after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold. The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU. Then the source gNB-DU can do optimization of target beam selection based on the information of the recovery beam used at successful Beam Failure Recovery and the beam indicated to the UE in the cell switch command.

Unlike normal LTM, the CLTM cell switch is not triggered by the source gNB-DU and the beam is not selected by the source gNB-DU. However, this does not mean the source gNB-DU does not need to do optimization. Although the beam is not direct selected by the source gNB-DU, the condition which leads UE to select the wrong beam is determined by the source gNB-CU or gNB-DU. So it is reasonable that the source gNB-CU or gNB-DU should do optimization of CLTM Execution Condition(s) related to beam selection.

As we know, the UE can be configured with L1 conditions or L3 conditions for CLTM, the L1 conditions are determined by gNB-DU and L3 conditions are determined by gNB-CU. So when gNB-CU acquire BFR related information, it performs initial analysis, if it find the CLTM condition is L1 based, then the gNB-CU should send the BFR related information to source gNB-DU. In this case, the source gNB-DU can do optimization of CLTM condition configuration. On the other hand, if it find the CLTM condition is L3 based, it can do optimization of CLTM condition configuration by its own.

- Although the beam used in CLTM cell switch is not direct selected by the source gNB-DU, the configured L3 Execution condition which leads UE to select the wrong beam is determined by the gNB-CU, and the configured L1 Execution condition which leads UE to select the wrong beam is determined by the source gNB-DU.

- The source gNB-DU or gNB-CU do optimization of CLTM condition configuration in case of BFR shortly after successful C-LTM cell switch execution.

In addition, since the LTM cell switch is not triggered by the source gNB-DU, the source gNB-DU does not know the beam selected by the UE when the cell switch is triggered. Even if it receives recovery beam information from the target gNB-DU, it can not know how to do optimization. RAN3 can consider to discuss whether additional information is needed in order to do optimization for this BFR case.

- Unlike normal LTM, the source gNB-DU does not know the beam selected by the UE when the cell switch is triggered. Even if it receives recovery beam information from the target gNB-DU, it may not know how to do optimization.

- RAN3 to discuss whether additional information is needed besides recovery beam information in order to do optimization for BFR case.

LTM Cell Switch failure due to wrong beam and outdated TA

In legacy MRO for LTM, the target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU. Then the gNB-DU can do optimization of target beam selection or TA related parameters based on the information of SSB Index and the TA value used at successful Random Access during LTM recovery or re-establishment after a Cell Switch failure in same cell.

This mechanism can be workable due to the target gNB-DU can receive cell switch notification message including TCI State ID and TA Information from the source gNB-DU. So it is easy for the target gNB-DU to determine whether the UE performing LTM recovery or re-establishment after a Cell Switch failure in the same cell. However, for CLTM, since there is no cell switch notification message from the source gNB-DU, when a target gNB-DU identifies that the UE successfully performs a RACH-based access, it cannot distinguish the following scenarios:

- Scenario 1: The UE perform RACH-based CLTM cell switch successful, it does not perform RACH-less CLTM cell switch since the condition of performing RACH-less is not met.

- Scenario 2: The UE perform RACH-less CLTM cell switch failure, then perform RACH-based CLTM cell switch successful to the same cell.

- Scenario 3: The UE perform RACH-less or RACH-based CLTM cell switch failure, then perform RACH-based CLTM cell switch successful to another cell.

For Scenario 1, the CLTM configuration (e.g., TAT timer ) may be not suitable which leads UE cannot perform RACH-less CLTM cell switch.

For Scenario 2, LTM Cell Switch failure may be caused by wrong beam selection or outdated TA.

For Scenario 3, LTM Cell Switch failure may be caused by wrong cell selection.

Since the target gNB-DU cannot distinguish the different possible scenarios, it is certainly that it cannot identify the reason of RACH-less CLTM cell switch failure

- when a target gNB-DU identifies that the UE successfully performs a RACH-based access, it cannot distinguish the following different scenarios which caused by different reasons:

  - Scenario 1: The UE perform RACH-based CLTM cell switch successful, it does not perform RACH-less CLTM cell switch since the condition of performing RACH-less is not met.

  - Scenario 2: The UE perform RACH-less CLTM cell switch failure, then perform RACH-based CLTM cell switch successful to the same cell.

  - Scenario 3: The UE perform RACH-less or RACH-based CLTM cell switch failure, then perform RACH-based CLTM cell switch successful to another cell.

Therefore, unlike normal LTM, network solution can not work without UE’s report information. RAN3 can further discuss which information from UE are needed to do CLTM optimization for wrong beam and outdated TA issue.

- Network solution can not work without UE’s report information for CLTM. RAN3 to further discuss which information from UE are needed to do CLTM optimization for wrong beam and outdated TA issue.

CLTM connection failure due to too late, too early or to wrong cell

The use case of connection failure for C-LTM is similar as MRO for CHO in Rel-17, such as "too early," "too late," or "wrong cell". It is straightforward to consider similar use cases from MRO for CHO as potential user case for MRO for C-LTM.

Potential user case for too late C-LTM

Figure 1  Potential cases for too late C-LTM

Case 1: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

This case provides the most straightforward evidence that the initial C-LTM configuration was flawed Too Late. MRO requires the RLF-Report (generated after RLF detection) to capture the configured C-LTM parameters, the latest radio measurements in the source cell, and the crucial time elapsed since configuration (timeSinceCHOReconfig equivalent). Analyzing these data points allows the network to relax the C-LTM execution conditions (L1/L3 thresholds or TTT) for future similar mobility situations.

Case 2: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery in a C-LTM candidate cell.

This case is vital because it tests the robustness of the C-LTM recovery mechanism itself, not just the initial trigger. The MRO system needs to know: 1) Which candidate cell (choCellId equivalent) was chosen for recovery, and 2) Did the C-LTM execution procedure fail during recovery? Since C-LTM can be RACH-less, recovery failure here could be linked to an invalid Timing Advance (TA) value for the candidate cell, which is unique to LTM/C-LTM procedures. The MRO report must capture the candidate cell ID and the failure cause, even if the eventual recovery is successful.

Case 3: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

This is the most valuable failure chain for MRO. Rel-17 MRO for CHO mandated reporting consecutive failures to track the complexity of conditional mobility issues. For C-LTM, MRO needs to: 1) Confirm the initial "Too Late" diagnosis; 2) Analyze why the C-LTM recovery attempt (Failure 2) failed (e.g., incorrect L2 configuration, security failure, or invalid TA); 3) Track the timeUntilReconnection from the first RLF until the eventual success. Optimizing the C-LTM configuration requires understanding the full system impact of the initial configuration error.

- For too late C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Case 2: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery in a C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Potential scenarios for too early C-LTM

Figure 2  Potential scenarios for too early C-LTM

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in the source cell.

The failure in Case 1 is the immediate and explicit consequence of a "Too Early" configuration error. MRO must study this case to:

Diagnose Premature Triggering. Confirm that the configured L1/L3 execution condition was satisfied when the target cell link was still unstable or resources were unavailable. The configured thresholds (e.g., ltm-Threshold, ltm3-Offset) likely need to be tightened or the time filtering (timeToTrigger) needs to be lengthened.

Verify L1/L2 Execution Robustness: C-LTM execution involves a MAC reset and attempts to complete the switch quickly. MRO must analyze if the failure was related to the execution step, particularly the RACH-less procedure.

Case 2: the UE receives C-LTM configuration; the C-LTM execution successes; an RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish the radio link connection in the source cell.

This case 2 signifies a "Too Early" configuration that incorrectly deemed the target cell suitable for long-term use.

Validate Execution Condition and Target Quality: This failure provides critical post-execution data. MRO needs to analyze the reported latest radio measurement results (especially RSRP/RSRQ) of the newly connected cell right before the RLF.

Determine Time of Robustness Collapse: MRO must use the time information to confirm the "shortly after" duration.

- For too early C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in the source cell.

Case 2: the UE receives C-LTM configuration; the C-LTM execution successes; an RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish the radio link connection in the source cell.

Potential scenarios for wrong cell C-LTM

Figure 3 Potential scenarios for C-LTM to wrong cell

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

This case 1 confirms that the C-LTM execution was triggered towards a Wrong Cell (T1) which led to failure (HOF equivalent). The most critical MRO input is identifying the cell where the connection was ultimately successful, via the reconnectCellId field in the final RLF report.

Case 2: the UE receives C-LTM configuration; the C-LTM execution fails; the UE successfully performs C-LTM recovery in another C-LTM candidate cell.

This case proves that the network's low-latency recovery mechanism (attemptLTM-Switch) works when navigating between configured LTM candidates post-failure. MRO needs to capture the target cell ID (T1, the failed attempt) and the recovery cell ID (T2, the successful recovery destination) to differentiate between initial misconfiguration and recovery success.

Case 3: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

This case is a successive failure scenario where the network needs to diagnose two failures from a single configuration set. The initial failure at T1 confirms "Wrong Cell" selection. The second failure (recovery attempt at T2) confirms T2 was also unsuitable, or that the recovery procedure itself failed (e.g., invalid TA/security context).

Case 4: the UE receives C-LTM configuration; the C-LTM execution fails;  the UE attempts to C-LTM recovery to a C-LTM candidate cell and successes; a RLF occurs shortly after C-LTM recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

This case diagnoses a "Wrong Cell" failure (T1) followed by a quality-based failure ("Too Early" migration) in the subsequent recovery cell (T2). Even though the C-LTM recovery (Success 1) succeeded, T2’s quality rapidly collapsed, forcing re-establishment to C3.

Case 5: the UE receives C-LTM configuration; the C-LTM execution successes; a RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish or recovery the radio link connection in a cell other than the source cell and the previously selected target cells.

This is the clearest indicator of a Wrong Cell configuration despite a technical success. The configured condition (L1/L3) was satisfied, the UE executed the switch (successful C-LTM), but the link immediately failed in T1 (new PCell). MRO must analyze the precise measurements and execution condition fulfillment that led to the successful but ultimately poor migration to T1.

- For wrong cell C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 2: the UE receives C-LTM configuration; the C-LTM execution fails; the UE successfully performs C-LTM recovery in another C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 4: the UE receives C-LTM configuration; the C-LTM execution fails;  the UE attempts to C-LTM recovery to a C-LTM candidate cell and successes; a RLF occurs shortly after C-LTM recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 5: the UE receives C-LTM configuration; the C-LTM execution successes; a RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish or recovery the radio link connection in a cell other than the source cell and the previously selected target cells.

As we know, UE based solution is used for CHO to identify "too early," "too late," or "wrong cell" issue. It is reasonable to adopt the similar UE based solution for CLTM. RAN3 can further discuss which information are needed from UE report.

- UE based solution for CHO can be taken as baseline for CLTM to identify "too early," "too late," or "wrong cell" issue. RAN3 to further discuss which information are needed from UE report.

Conclusion

In this contribution, we provide the following observations and proposals:

- Although the beam used in CLTM cell switch is not direct selected by the source gNB-DU, the configured L3 Execution condition which leads UE to select the wrong beam is determined by the gNB-CU, and the configured L1 Execution condition which leads UE to select the wrong beam is determined by the source gNB-DU.

- The source gNB-DU or gNB-CU do optimization of CLTM condition configuration in case of BFR shortly after successful C-LTM cell switch execution.

- Unlike normal LTM, the source gNB-DU does not know the beam selected by the UE when the cell switch is triggered. Even if it receives recovery beam information from the target gNB-DU, it may not know how to do optimization.

- RAN3 to discuss whether additional information is needed besides recovery beam information in order to do optimization for BFR case.

- when a target gNB-DU identifies that the UE successfully performs a RACH-based access, it cannot distinguish the following different scenarios which caused by different reasons:

  - Scenario 1: The UE perform RACH-based CLTM cell switch successful, it does not perform RACH-less CLTM cell switch since the condition of performing RACH-less is not met.

  - Scenario 2: The UE perform RACH-less CLTM cell switch failure, then perform RACH-based CLTM cell switch successful to the same cell.

  - Scenario 3: The UE perform RACH-less or RACH-based CLTM cell switch failure, then perform RACH-based CLTM cell switch successful to another cell.

- Network solution can not work without UE’s report information for CLTM. RAN3 to further discuss which information from UE are needed to do CLTM optimization for wrong beam and outdated TA issue.

- For too late C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

Case 2: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery in a C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; an RLF occurs in the cell before C-LTM execution; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell.

- For too early C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in the source cell.

Case 2: the UE receives C-LTM configuration; the C-LTM execution successes; an RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish the radio link connection in the source cell.

- For wrong cell C-LTM, the following user cases should be investigated in Rel-20:

Case 1: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 2: the UE receives C-LTM configuration; the C-LTM execution fails; the UE successfully performs C-LTM recovery in another C-LTM candidate cell.

Case 3: the UE receives C-LTM configuration; the C-LTM execution fails; the UE attempts to C-LTM recovery to a C-LTM candidate cell but fails; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 4: the UE receives C-LTM configuration; the C-LTM execution fails;  the UE attempts to C-LTM recovery to a C-LTM candidate cell and successes; a RLF occurs shortly after C-LTM recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

Case 5: the UE receives C-LTM configuration; the C-LTM execution successes; a RLF occurs shortly after the successful C-LTM; the UE attempts to re-establish or recovery the radio link connection in a cell other than the source cell and the previously selected target cells.

- UE based solution for CHO can be taken as baseline for CLTM to identify "too early," "too late," or "wrong cell" issue. RAN3 to further discuss which information are needed from UE report.

Reference

[1] RP-251869