3GPP TSG-RAN WG3 Meeting #130	R3-258690

# Dallas, USA, 17th – 21st Nov, 2025

Agenda item:	12.2.3

Source: 	Samsung

Title: 	Discussion on other handover enhancements

Document for:	Discussion and decision

Introduction

This paper provides further discussions on the study of inter-CU LTM, as per the open issue of RAN3#129bis:

Study inter-CU LTM in R20 SI?

Discussion

During the discussion of RAN3#129bis, there was no consensus on whether to study inter-CU LTM in Rel-20 SI. Some companies would prefer to study intra-CU LTM first, before starting to study inter-CU LTM.

We fully respect the fact that the discussion of inter-CU LTM is based on the completed mechanism of intra-CU LTM. However, for AI/ML-assisted LTM, we don’t think the discussion of inter-CU LTM shall be postponed until the study of intra-CU LTM is finished, especially considering the fact that we only have two meetings left for the phase of Rel-20 SI.

For intra-CU LTM, the selection of candidate cells and beams has been agreed to be studied as a point where AI/ML could be used for assistance. When considering inter-CU LTM, it is straightforward that the AI/ML could also be used at the source NG-RAN node to select candidate cells and beams. In our discussion paper for intra-CU LTM [1], we are basically proposing to use AI/ML for predicting the TA value for the UE to connect to candidate/target cell(s), which would save the TA acquisition procedures for the UE and minimize the interruption and latency for handover. We believe similar mechanism could be used for the source RAN node to achieve the TA value of the candidate cells, based on the inter-CU LTM related procedures. For example, if a predicted TA value is introduced between CU and DU, the similar enhancement shall be applied to XnAP, i.e., between the source RAN node and the target RAN node. Similar consideration applies to AI/ML-assisted cell switch command, as discussed in our discussion paper [2].

Observation 1: The AI/ML-assisted candidate cell selection, TA value prediction, cell switch command, can also be applied to inter-CU LTM.

To study inter-CU LTM, the main delta would be focused on the XnAP standard impacts. In Rel-19, after the normative work for inter-CU LTM, XnAP impacts were introduced to enable to source NG-RAN node and target NG-RAN node to coordinate about LTM configuration information, cell switch command notification, TA value information and so on. Specifically, the XnAP HO preparation procedure is enhanced for the source RAN node and target RAN node to coordinate about necessary information regarding LTM. There are also some new procedures introduced for exchanging information regarding LTM configuration, early sync, cell switch command, etc. The main procedures used for inter-CU LTM are listed below:

-	Handover Preparation

-	LTM Configuration Update

-	TA Information Transfer

-	Cell Switch Notification

-	LTM Cancel

-	CSI-RS Coordination

When looking into the XnAP aspects of inter-CU LTM, and with our observation 1 in mind, we think the basic logic of studying inter-CU LTM can be put in the way below:

1. Apply the agreed cases of intra-CU LTM to inter-CU LTM, e.g., candidate cell selection, potentially early TA prediction, etc.

2. Apply the agreements of intra-CU LTM to the study of inter-CU LTM

3. Study the XnAP impacts of AI/ML-based inter-CU LTM

Following the logic to study inter-CU LTM above, there is no confliction between studying inter-CU LTM during the study of intra-CU LTM, if we apply all the progress of intra-CU LTM to the cases of inter-CU LTM.

Observation 2: There is no confliction between the study of inter-CU LTM and intra-CU LTM, if the latest progress of intra-CU LTM is taken into account for inter-CU LTM.

Another reason we would like to study inter-CU LTM without finishing the whole study of intra-CU LTM is that there are multiple cases discussed in intra-CU LTM. We don’t have to wait the completion of all the sub-cases, instead, if some of the sub-topics have significant progress, e.g., AI/ML-assisted candidate cell selection, the corresponding enhancement for inter-CU LTM could be studied.

Based on the above discussion, we would propose to study inter-CU LTM in Rel-20 AI.ML SI.

Proposal 1: Inter-CU LTM can be studied as an item of handover enhancement in Rel-20 SI.

Last but not least, we would like to mention that in Rel-19, the discussion of inter-CU LTM not only includes the inter-CU LTM from a source NG-RAN node to a target NG-RAN node, the inter-CU LTM in dual connectivity has also been discussed and standardized, the following two cases were specified:

-	inter-CU MCG LTM

-	inter-CU SCG LTM

To simplify the study of inter-CU LTM in AI/ML SI, we tent to prioritize the normal inter-CU LTM from a source gNB to a target gNB, without touching the DC scenario. In other words, we think the inter-CU LTM in dual connectivity can be deprioritized.

Proposal 2: The basic inter-CU LTM from source RAN node to target RAN node can be prioritized, while the inter-CU LTM in dual connectivity can be deprioritized.

Conclusion

In this paper, the following proposals are provided:

Observation 1: The AI/ML-assisted candidate cell selection, TA value prediction, cell switch command, can also be applied to inter-CU LTM.

Observation 2: There is no confliction between the study of inter-CU LTM and intra-CU LTM, if the latest progress of intra-CU LTM is taken into account for inter-CU LTM.

Proposal 1: Inter-CU LTM can be studied as an item of handover enhancement in Rel-20 SI.

Proposal 2: The basic inter-CU LTM from source RAN node to target RAN node can be prioritized, while the inter-CU LTM in dual connectivity can be deprioritized.

References

[1] R3-258689 Discussion on AI-based intra-CU LTM